3GPP TSG-RAN4 Meeting #86bis 	R4-1803707

# Melbourne, Australia, 16th April – 20th April, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on MTTD and MRTD requirements for EN-DC operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-4-3 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - In RAN2#100 meeting, for inter-band EN-DC, it was agreed that asynchronous UE operation is default operation and no separate capabilities are needed. And no additional UE capability parameter is needed to distinguish whether the UE supports operation in synchronised and non synchronised cases. Thus, the sentence “provided that the UE indicates that it is capable of synchronous” shall be removed. - In RAN2#100 meeting, it was agreed that the synchronous operation is only supported in Rel-15 for intra-band TDD EN-DC. Thus, the E-UTRA TDD- NR TDD intra-band asynchronous EN-DC operation shall be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove “provided that the UE indicates that it is capable of synchronous EN-DC [16]” in section 7.5.2 and 7.6.2. Remove “and E-UTRA TDD- NR TDD” in section 75.3 and 7.6.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MTTD and MRTD requirements will be confusing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 7.5 and 7.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

7.5	Maximum Transmission Timing Difference

7.5.1	Introduction

A UE shall be capable of handling a relative transmission timing difference between subframe timing boundary of E-UTRA PCell and slot timing boundaries of PSCell to be aggregated EN-DC

7.5.2	Minimum Requirements for inter-band EN-DC

The UE shall be capable of handling a maximum uplink transmission timing difference between E-UTRA PCell and PSCell as shown in Table 7.5.2-1. The requirements for asynchronous EN-DC are applicable for E-UTRA TDD- NR TDD, E-UTRA FDD- NR FDD, E-UTRA FDD-NR TDD and E-UTRA TDD-NR FDD inter-band asynchronous EN-DC.

Table 7.5.2-1 Maximum uplink transmission timing difference requirement for asynchronous EN-DC

| Sub-carrier spacing in E-UTRA PCell (kHz) | UL Sub-carrier spacing for data in PSCell (kHz) | Maximum uplink transmission timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 500 |
| 15 | 30 | 250 |
| 15 | 60 | 125 |
| 15 | 120Note1 | 62.5 |
| Note1 : For E-UTRA FDD- NR FDD and E-UTRA TDD- NR TDD intra-band EN-DC, 120kHz is not applied. |  |  |

The UE shall be capable of handling a maximum uplink transmission timing difference between E-UTRA PCell and PSCell as shown in Table 7.5.2-2. The requirements for synchronous EN-DC are applicable for E-UTRA TDD-NR TDD, E-UTRA TDD-NR FDD and E-UTRA FDD-NR TDD inter-band EN-DC.

Table 7.5.2-2 Maximum uplink transmission timing difference requirement for inter-band synchronous EN-DC

| Sub-carrier spacing in E-UTRA PCell (kHz) | UL Sub-carrier spacing for data in PSCell (kHz) | Maximum uplink transmission timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 35.21 |
| 15 | 30 | 35.21 |
| 15 | 60 | 35.21 |
| 15 | 120 Note1 | 35.21 |
| Note 1: For E-UTRA FDD- NR FDD and E-UTRA TDD- NR TDD intra-band EN-DC, 120kHz is not applied. |  |  |

7.5.3	Minimum Requirements for intra-band EN-DC

For intra-band EN-DC, only collocated deployment is applied.

The UE shall be capable of handling a maximum uplink transmission timing difference between E-UTRA PCell and PSCell as shown in Table 7.5.2-1 provided the UE indicates that it is capable of asynchronous EN-DC  [16]. The requirements for asynchronous EN-DC are applicable for E-UTRA FDD- NR FDD intra-band asynchronous EN-DC.

No uplink transmission timing difference is applicable for synchronous EN-DC.

7.6	Maximum Receive Timing Difference

7.6.1	Introduction

A UE shall be capable of handling a relative receive timing difference between subframe timing boundary of E-UTRA PCell and slot timing boundaries of PSCell to be aggregated for EN-DC.

A UE shall be capable of handling a relative receive timing difference between slot timing boundary of different carriers to be aggregated NR carrier aggregation.

7.6.2	Minimum Requirements for inter-band EN-DC

The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from E-UTRA PCell and slot timing of signal from PSCell at the UE receiver as shown in Table 7.6.2-1. The requirements for asynchronous EN-DC are applicable for E-UTRA TDD- NR TDD, E-UTRA FDD- NR FDD, E-UTRA FDD- NR TDD and E-UTRA TDD- NR FDD inter-band EN-DC.

Table 7.6.2-1 Maximum receive timing difference requirement for asynchronous EN-DC

| Sub-carrier spacing in E-UTRA PCell (kHz) | DL Sub-carrier spacing in PSCell (kHz)Note1 | Maximum receive timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 500 |
| 15 | 30 | 250 |
| 15 | 60 | 125 |
| 15 | 120 | 62.5 |
| Note1 : DL Sub-carrier spacing is min{SCSSS, SCSDATA}. Note2 : For E-UTRA FDD- NR FDD and E-UTRA TDD- NR TDD intra-band EN-DC, 120kHz is not applied. |  |  |

The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from E-UTRA PCell and slot timing of signal from PSCell at the UE receiver as shown in Table 7.6.2-2. The requirements for synchronous EN-DC are applicable for E-UTRA TDD- NR TDD, E-UTRA TDD- NR FDD and E-UTRA FDD- NR TDD inter-band EN-DC.

Table 7.6.2-2 Maximum receive timing difference requirement for inter-band synchronous EN-DC

| Sub-carrier spacing in E-UTRA PCell (kHz) | DL Sub-carrier spacing in PSCell (kHz)Note1 | Maximum receive timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 33 |
| 15 | 30 |  |
| 15 | 60 |  |
| 15 | 120 |  |
| Note1: DL Sub-carrier spacing is min{SCSSS, SCSDATA}. Note2: For E-UTRA FDD- NR FDD and E-UTRA TDD- NR TDD intra-band EN-DC, 120kHz is not applied. |  |  |

Table 7.6.2-3	Void

7.6.3	Minimum Requirements for intra-band EN-DC

For intra-band EN-DC, only collocated deployment is applied.

The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from E-UTRA PCell and slot timing of signal from PSCell as shown in Table 7.6.2-1 provided the UE indicates that it is capable of asynchronous EN-DC [16]. The requirements for asynchronous EN-DC are applicable for E-UTRA FDD- NR FDD intra-band EN-DC.

The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from E-UTRA PCell and slot timing of signal from PSCell as shown in Table 7.6.3-1 provided the UE indicates that it is only capable of synchronous EN-DC [16]. The requirements for synchronous EN-DC are applicable for E-UTRA TDD- NR TDD and E-UTRA FDD- NR FDD intra-band EN-DC.

Table 7.6.3-1 Maximum receive timing difference requirement for intra-band synchronous EN-DC

| Sub-carrier spacing in E-UTRA PCell (kHz) | DL Sub-carrier spacing in PSCell (kHz) Note1 | Maximum receive timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 3 |
| 15 | 30 | 3 |
| 15 | 60 | 3 |
| Note1: DL Sub-carrier spacing is min{SCSSS, SCSDATA}. |  |  |

Table 7.6.3-2	Void