3GPP TSG-RAN WG4 Meeting #86	R4-1804027

Athens, Greece, 26th Feb – 2nd Mar 2018

Source: 	Nokia

Title: 					A-MPR for DC_(n)41C

Agenda Item: 			7.1.3.1.4

Document for:	Discussion

# 1	Introduction

This contribution presents A-MPR simulation results for DC_(n)41B contiguous intra-band EN-DC configuration in case of power class 2.

# 2	Discussion

## 2.1	Simulation assumptions

- PA calibration point: QPSK 100 RB DFT-s-OFDM signal with 0.5 dB MPR

- LTE LO leakage and IQ-Image = -25 dBc, NR LO leakage and IQ-Image = -28 dBc

- EN-DC ALCR from 38.101-3, Table 6.5.3.2.4-1 (below) with 31 dB threshold (NR PC2)

- n41 SEM from [1]; below, applied according to combined EN-DC bandwidth

- NR general spurious limit from 38.101, Table 6.5.3.1-2

- General spurious limit modification from -30 dBm to -25 dBm above (n)41B

- LTE with constant 23 dBm power, NR backoff is searched

Table 1: ACLR for intra-band EN-DC (contiguous sub-blocks)

| Parameter | Unit | Value |
| --- | --- | --- |
| EN-DCACLR | dBc | 30 |
| Measurement bandwidth |  | [0.95] ENBW |
| Frequency offset of adjacent channel |  | ENBW / -ENBW |
| NOTE 1: ENBW is the aggregated bandwidth in MHz of an E-UTRA sub-block and an adjacent NR sub-block; there is no frequency separation between the said sub-blocks. The sub-block bandwidths include any internal guard bands. NOTE 2: The frequency offset is that in between the centre frequencies of the measurement filters |  |  |

Table 2: NR transmission bandwidths

| SCS [kHz] | Channel bandwidths [MHz] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 40 | 50 | 60 | 80 | 100 |
| 15 | 4.5 | 9.36 | 14.22 | 19.08 | 23.94 | 38.88 | 48.6 | N.A | N.A | N.A |
| 30 | 3.96 | 8.64 | 13.68 | 18.36 | 23.40 | 38.16 | 47.88 | 58.32 | 78.12 | 98.28 |
| 60 | N.A | 7.92 | 12.96 | 17.28 | 22.32 | 36.72 | 46.8 | 56.88 | 77.04 | 97.20 |

Table 3: n41 SEM

| ΔfOOB MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ± 0 - 1 | -18 | -20 | -21 | -24 | -24 | -24 | -24 | -24 | 30 kHz |
| ± 1 - 5 | -10 |  |  |  |  |  |  |  | 1 MHz |
| ± 5 - X | -13 |  |  |  |  |  |  |  |  |
| ± X - (BWChannel + 5 MHz) | -25 |  |  |  |  |  |  |  |  |
| Note 1: X is defined in Table 2 |  |  |  |  |  |  |  |  |  |

Table 4: NS_04 additional spurious emission requirement

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | All channel bandwidths |  |
| 2490.5 ≤ f < 2495 | -13 | 1 MHz |
| 0 < f < 2490.5 | -25 | 1 MHz |

## 2.2	Results in edge case

An edge allocation scenario was simulated as a worst-case scenario where LTE and NR allocations start from outer edges of aggregated bandwidth. The results are evaluated in two different scenarios, 1) with high frequency offset to the lower edge of (n)41B and the additional spurious emission limit signalled by NS_04 and 2) with zero frequency offset to the lower edge of (n)41B and the additional spurious emission limit signalled by NS_04.

In all simulated scenarios LTE is with constant 23 dBm power level and NR backoff is reported. LTE and NR allocations are on the outer edges leaving a gap between until gap is filled as all combinations have been simulated.

In the following figures, NR backoffs higher than 19 dB are not reported, even though such a high backoff would be necessary in certain scenarios with NS_04 additional spurious emission requirement. Thus, white space inside allocated regions in the figures implies this high backoff demand. LTE+NR bandwidth combination of 10 MHz + 10 MHz is used as an example of narrow combination and 20 MHz + 60 MHz as an example of wide combination.

Figure 1: LTE 10MHz + NR 10MHz, 15kHz, QPSK results without evaluation of addition spurious emission limit (scenario with high frequency offset to lower edge of (n)41B)

Figure 2: LTE 10MHz + NR 10MHz, 15kHz, QPSK results with evaluation of addition spurious emission limit (scenario with zero frequency offset to lower edge of (n)41B)

Figure 3: LTE 20MHz + NR 60MHz, 30kHz, QPSK results without evaluation of addition spurious emission limit (scenario with high frequency offset to lower edge of (n)41B)

Figure 4: LTE 20MHz + NR 60MHz, 30kHz, QPSK results with evaluation of addition spurious emission limit (scenario with zero frequency offset to lower edge of (n)41B)

In case NR modem do not know the LTE allocation it has to assume worst case backoff which can be read from Y-axis if NR modem knows the LTE allocation then it can use the correct backoff reported in the plots.

In all cases the required NR backoff is large or very large. It seems that strategy of keeping LTE at 23 dBm power has consequences for NR power as below the LTE channel, third-order emission power within some measurement bandwidth could be roughly approximated as P0_dBm + 2*P_LTE_dBm + P_NR_dBm, thus 1 dB decrease of LTE power would reduce third-order emissions as much as a 2 dB reduction in NR power. This means that if LTE and NR power are reduced same amount NR bakcoff would be smaller but this required that LTE and NR modems know others allocations. In the lower edge of (n)41B this crucial because of the additional spurious emission limit with NS_04 signalling.

Further simulations results with different

- LTE+NR bandwidth combinations (5 MHz+10 MHz, 5 MHz +15 MHz, 10 MHz +10 MHz, 10 MHz +40 MHz, 10 MHz +50 MHz, 20 MHz +20 MHz, 20 MHz +40 MHz, 20 MHz +60 MHz, 20 MHz +80 MHz), and

- modulations (QPSK, 16QAM, 64QAM, 256QAM)

with smallest applicable NR sub-carrier spacing are included in the Appendix.

# 3	Conclusion

In this contribution we have presented backoff simulation results for DC_(n)41B.

# 4	Reference

[1] R4-1802539, n41 A-MPR simulation results,	Nokia, RAN4#86

# Appendix