3GPP TSG-RAN WG4 Meeting #86bis	R4-1804029

Melbourne, Australia, 16th – 20th Apr 2018

Source: 	Nokia, Nokia Shanghai Bell

Title: 					Notation for NR non-contiguous intraband CA

Agenda Item: 			7.1.3.7

Document for:	Approval

# 1	Introduction

This contribution discusses the CA acronym for non-contiguous intraband CA.

# 2	Discussion

So far in LTE specifications RAN4 has standardized three sub-block configurations for example CA_25A-25A-25A. The acronym is not too awkward compared for example to five DL interband CA acronyms like CA_1A-3A-7A-20A-42A. But now in NR there are proposals for eight sub-block non-contiguous intraband CA and taking the bands 257 as an example the acronym would be CA_n257A-n257A-n257A-n257A-n257A-n257A-n257A-n257A which is rather complex and un-informative.

Hence we propose following notation for NR non-contiguous intraband CA using CA_n257A-n257A-n257A-n257A-n257A-n257A-n257A-n257A as an example.

CA_n257(8xA)

In case of non-contiguous intraband CA combinations which have different CA classes in sub-blocks like CA_n257A-n257A-n257A-n257C we propose to follow LTE approach and only assign one acronym to cover all cases. As in LTE A+C and C+A are both mapped under A+C as an example check  for-example CA_40A-40C in Table 5.6A.1-3 of 36.101. So as an example C+A+A+A, A+C+A+A, A+A+C+A and A+A+A+C would belong to same CA configuration and the acronym would be

CA_nX(3xA-C)

Two examples above should be self-explanatory hence no general notation is presented here as it would be more confusing than helping.

Proposal 1: In case of non-contiguous intraband CA combinations which have different CA BW classes in sub-blocks only assign one acronym to cover all cases. As an example C+A+A+A, A+C+A+A, A+A+C+A and A+A+A+C would belong to same CA configuration and have same acronym.

Proposal 2: Use notation as presented below for NR non-contiguous intraband CA (band n257 just an example)

- CA_n257(8xA)	 (which is 8 sub-blocks, all one CC)

- CA_n257(3xA-C) (which is 4 sub-blocks, three sub-blocks having 1 CC and one sublock having 2 contiguous CC’s and order of cub-blocks is free)

# 3	Conclusion

In this contribution we have made proposal how to make notation for NR non-contiguous intraband CA.

Proposal 1: In case of non-contiguous intraband CA combinations which have different CA BW classes in sub-blocks only assign one acronym to cover all cases. As an example C+A+A+A, A+C+A+A, A+A+C+A and A+A+A+C would belong to same CA configuration and have same acronym.

Proposal 2: Use notation as presented below for NR non-contiguous intraband CA (band n257 just an example)

- CA_n257(8xA)	 (which is 8 sub-blocks, all one CC)

- CA_n257(3xA-C) (which is 4 sub-blocks, three sub-blocks having 1 CC and one sublock having 2 contiguous CC’s and order of cub-blocks is free)