3GPP TSG-RAN WG4#86Bis Meeting	R4-1804130

Melbourne, Australia, 16 – 20 April 2018

Agenda item:	6.29.2

Title: 	Discussion on 8Rx UE demodulation performance requirements

Source: 	Intel Corporation

Document for:	Discussion

# Introduction

In RAN Plenary Meeting #78, the Rel.15 WI “UE requirements for LTE DL 8Rx antenna ports” has been approved and its WID is included in [1]. In the scope of the WID [1], there is part related to RAN4 UE demodulation/CSI performance tests as shown below:

- For UE demodulation/CSI requirements

  - Define channel model for downlink 8Rx antennas

    - Specify the antenna configuration and MIMO channel correlation matrices for 8Rx antennas;

    - Specify the static channel model;

  - Define test cases for the rank lower than or equal to 4.

    - Adopt the test cases studied in SI to define requirements.

  - Define test cases for the rank higher than 4 in fading channel.

    - Base on combinations of rank and MCS that can achieve the maximum configured throughput.

  - Define SDR test for 8Rx in WI.

  - Define CSI test for 8Rx.

  - No PDCCH/PCFICH demodulation requirement is expected for 8Rx.

- Considering the test coverage of 8Rx, test applicability rule is needed to define

  - Define applicability rule of existing performance requirements for 8Rx capable UEs.

In RAN4#86 Meeting, the timeline for demodulation performance part was agreed [2]:

RAN4#86bis (April 2018)

- Initialize discussions on 8Rx channel model

- Initialize discussion on test cases for demodulation performance requirements

- Initialize discussion on SDR tests

- Initialize discussion on CSI tests

RAN4#87 (May 2018)

- Specify the antenna configuration and MIMO channel correlation matrices for 8Rx

- Specify static and fading channel models

- Agree on test cases for single-carrier requirements

RAN4#88 (August 2018)

- Provide alignment results for single-carrier test cases

- Initialize discussion on test applicability rule for 8Rx

- Initialize discussion on CA scenarios

RAN4#88bis (October 2018)

- Endorse draft CRs

- Define requirements for CA scenarios

RAN4#89 (November 2018)

- Approve final CRs

According to the above timeline, in this contribution, we provide our views to initialize discussion on 8Rx UE demodulation/CSI performance requirements.

# Discussion

In general, the demodulation/CSI work in this 8Rx WI is divided into two phases [1]:

  - Phase I: focus on the single carrier scenario

  - Phase II: After finalizing the single carrier requirements, the following CA scenarios will be studied and specified, if feasible:

    - Intra-band contiguous CA with 2CC and 8Rx with up to 8-layers supported per CC

    - Inter-band CA with 8Rx/8-layers supported on one or two contiguous CCs on the identified operating bands in this work item, and with 2Rx or 4Rx supported on the other CC(s).

    - NOTE: before specifying the requirements for CA, RAN4 shall conduct the feasibility study.

Therefore, we first focus on the single carrier scenarios for 8Rx demodulation/CSI requirements.

## Channel Model and Demodulation Performance Tests

It is stated in the 8Rx WID [1] that:

  - Define test cases for the rank lower than or equal to 4.

    - Adopt the test cases studied in SI to define requirements.

  - Define test cases for the rank higher than 4 in fading channel.

    - Base on combinations of rank and MCS that can achieve the maximum configured throughput.

In the 8Rx study item report [3], there were two test cases studied for rank lower than or equal to 4 as listed in Table 1 below:

Table 1 Simulation parameters for TM2 and TM3 tests

| Test number | Transmission mode | Bandwidth and MCS | Reference channel | Propagation condition | Correlation matrix and antenna configuration |
| --- | --- | --- | --- | --- | --- |
| 1 | TM2 | 10 MHz 16QAM,1/2 | R.11 FDD | EVA5 | 2x8 Low Medium correlation B, ULA |
| 2 | TM3 | 10 MHz 16QAM,1/2 | R.11 FDD | EVA70 | 2x8 Low |

For rank higher than 4 in fading channel scenarios, there were also test cases studied in the 8Rx SI [3]. However, there were no consensus on which combination of rank and MCS to be a suitable test case.

As shown in Figure B-3 in Annex B of the 8Rx study item report [3], it is observed that the throughput performance of rank=8 degrades dramatically when there is a very slight increase from zero to 10-6 in antenna correlation level (e.g. β value). Therefore, in order to achieve satisfactory performance by using 8Rx for ranks higher than 4, good low antenna correlation has to be assumed, which in some cases would be a rather artificial antenna correlation assumption.

Therefore, considering the goal of achieving high throughput and the practical impact of antenna correlation with large number of antennas, in our view, a test with suitable combination of rank and MCS can be given as in Table 2 below.

Table 2 Simulation parameters for TM9 test

| Test number | Transmission mode | Bandwidth and MCS | Rank | Propagation condition | Correlation matrix and antenna configuration |
| --- | --- | --- | --- | --- | --- |
| 3 | TM9 | 10 MHz 256QAM MCS table MCS=20 | Rank=4 | EPA5 | 8x8 Low ULA |

Proposal 1: Define 8Rx UE demodulation performance tests for TM2, TM3 and TM9 modes as specified in Table 1 and Table 2.

## SDR Tests

Based on the demodulation tests in proposal 1, for SDR tests, we can reuse the 2x8 and 8×8 static channels derived in [4], which are given below:

Fig.1 2x8 Static Channel Model

Fig.2 8x8 Static Channel Model

Considering different UE capabilities, we propose that:

Proposal 2: Define separate 8Rx UE SDR tests covering

        - 2 and 4 MIMO layers

        - Bandwidth = 5, 10, 15 and 20MHz

        - 64QAM and 256QAM

## CSI Tests

Since there was none of CSI discussion in the 8Rx SI [3], it is necessary to initiate the discussion on the coverage of the 8Rx CSI tests. In our view, based on the demodulation tests in proposal 1, the following CSI tests should be considered for 8Rx:

Proposal 3: Define 8Rx CSI tests covering

        - Wideband CQI tests in AWGN channels for 2 and 4 MIMO layers

        - Wideband CQI tests in fading channels for 2 or 4 MIMO layers

        - Subband PMI tests for 2 or 4 MIMO layers

        - RI tests for up to 4 MIMO layers

# Conclusion

In this contribution, we provide our views to initialize discussion on 8Rx UE demodulation/CSI performance requirements.

Proposal 1: Define 8Rx UE demodulation performance tests for TM2, TM3 and TM9 modes as specified in Table 1 and Table 2.

Proposal 2: Define separate 8Rx UE SDR tests covering

        - 2 and 4 MIMO layers

        - Bandwidth = 5, 10, 15 and 20MHz

        - 64QAM and 256QAM

Proposal 3: Define 8Rx CSI tests covering

        - Wideband CQI tests in AWGN channels for 2 and 4 MIMO layers

        - Wideband CQI tests in fading channels for 2 or 4 MIMO layers

        - Subband PMI tests for 2 or 4 MIMO layers

        - RI tests for up to 4 MIMO layers

# Reference

[1] R4-172842, “New WI proposal: UE requirements for LTE DL 8Rx antenna ports”, Huawei, HiSilicon, Dec. 2017

[2] R4-1801858, “Work plan for WI on UE requirements for LTE DL 8Rx antenna ports”, Huawei, HiSilicon, Feb. 2018

[3] TR36.757, “Study on LTE DL 8Rx antenna ports”, Dec. 2017

[4] R4-1708538, “Way forward on 8x8 channel model”, Huawei, HiSilicon, Aug. 2017