3GPP TSG-RAN WG4 Meeting #86bis	R4-1804252

Melbourne, Australia, 16 – 20 April 2018

Agenda Item:	7.7.4.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Manufacturer declarations for BS Type 1-O OTA sensitivity conformance test

Document for:	Approval

# 1.	Introduction

This contribution provides our proposals on the manufacturer declarations for NR BS Type 1-O OTA sensitivity conformance test.

# 2.	Discussion

The manufacturer declarations for AAS BS OTA sensitivity conformance test currently specified in TS 37.145-2 [1] is provided below in Table 1.

Table 1: Manufacturer declarations for AAS BS OTA sensitivity conformance test

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| D10.1 | OSDD identifier | A unique identifier for the OSDD. |
| D10.2 | OSDD operating band support | Operating band supported by the OSDD, declared for every OSDD identified in D10.1. NOTE 2: As each identified OSDD has a declared minimum EIS value (D10.6), multiple operating band can be only be declared if they have the same minimum EIS declaration. |
| D10.3 | OSDD RAT support | RAT(s) supported by the OSDD for each supported operating band, declared for every OSDD identified in D10.1. NOTE 3: If the OSDD supports multiple RAT's with different minimum EIS value (D10.6) if all other parameters are the same then different EIS values for different RATS and signal BW's may be declared for an OSDD. |
| D10.4 | OTA sensitivity E-UTRA supported channel BW | The E-UTRA channel BW's supported by each OSDD. |
| D10.5 | Redirection of receiver target support | Ability to redirect the receiver target related to the OSDD |
| D10.6 | Minimum EIS | The minimum EIS requirement (i.e. maximum allowable EIS value) applicable to all sensitivity RoAoA per OSDD. Declared for every supported RAT and E-UTRA supported channel BW for the OSDD. Declared per OSDD identified in D10.1. NOTE 4: If the AAS BS is not capable of redirecting the receiver target related to the OSDD then there is only one RoAoA applicable to the OSDD. |
| D10.7 | Receiver target reference direction Sensitivity Range of Angle of Arrival | The sensitivity RoAoA associated with the receiver target reference direction (D10.9) for each OSDD. |
| D10.8 | Receiver target redirection range | For each OSDD the associated union of all the sensitivity RoAoA achievable through redirecting the receiver target related to the OSDD |
| D10.9 | Receiver target reference direction | For each OSDD an associated direction inside the receiver target redirection range (D10.8). NOTE 5: For an OSDD without receiver target redirection range, this is a direction inside the sensitivity RoAoA. |
| D10.10 | Conformance test directions sensitivity RoAoA | For each OSDD that includes a receiver target redirection range, four sensitivity RoAoA comprising the conformance test directions (D10.11). |
| D10.11 | Conformance test directions | For each OSDD four conformance test directions. If the OSDD includes a receiver target redirection range the following four directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the receiver target redirection range, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the receiver target redirection range, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the receiver target redirection range, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the receiver target redirection range, while φ value being the closest possible to the receiver target reference direction. If an OSDD does not include a receiver target redirection range the following 4 directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the sensitivity RoAoA, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the sensitivity RoAoA, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the sensitivity RoAoA, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the sensitivity RoAoA, while φ value being the closest possible to the receiver target reference direction. |

Since NR BS Type 1-O OTA sensitivity requirement is similar to AAS BS OTA sensitivity requirement, except it is only specified for NR but not are RAT. Therefore, the current manufacturer declarations for AAS BS OTA sensitivity conformance test can be reused for NR BS Type 1-O sensitivity conformance test, except D.10.3. Therefore, we propose to adopt the manufacturer declarations provided below in Table 2 for NR BS Type 1-O OTA sensitivity conformance test.

Table 2: Manufacturer declarations for NR BS Type 1-O OTA sensitivity conformance test

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| D10.1 | OSDD identifier | A unique identifier for the OSDD. |
| D10.2 | OSDD operating band support | Operating band supported by the OSDD, declared for every OSDD identified in D10.1. NOTE 1: As each identified OSDD has a declared minimum EIS value (D10.5), multiple operating band can be only be declared if they have the same minimum EIS declaration. |
| D10.3 | OTA sensitivity NR supported channel BW | The NR BS channel BW's supported by each OSDD. |
| D10.4 | Redirection of receiver target support | Ability to redirect the receiver target related to the OSDD |
| D10.5 | Minimum EIS | The minimum EIS requirement (i.e. maximum allowable EIS value) applicable to all sensitivity RoAoA per OSDD. Declared for every NR supported channel BW for the OSDD. Declared per OSDD identified in D10.1. NOTE 2: If the AAS BS is not capable of redirecting the receiver target related to the OSDD then there is only one RoAoA applicable to the OSDD. |
| D10.6 | Receiver target reference direction Sensitivity Range of Angle of Arrival | The sensitivity RoAoA associated with the receiver target reference direction (D10.8) for each OSDD. |
| D10.7 | Receiver target redirection range | For each OSDD the associated union of all the sensitivity RoAoA achievable through redirecting the receiver target related to the OSDD |
| D10.8 | Receiver target reference direction | For each OSDD an associated direction inside the receiver target redirection range (D10.7). NOTE 3: For an OSDD without receiver target redirection range, this is a direction inside the sensitivity RoAoA. |
| D10.9 | Conformance test directions sensitivity RoAoA | For each OSDD that includes a receiver target redirection range, four sensitivity RoAoA comprising the conformance test directions (D10.10). |
| D10.10 | Conformance test directions | For each OSDD four conformance test directions. If the OSDD includes a receiver target redirection range the following four directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the receiver target redirection range, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the receiver target redirection range, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the receiver target redirection range, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the receiver target redirection range, while φ value being the closest possible to the receiver target reference direction. If an OSDD does not include a receiver target redirection range the following 4 directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the sensitivity RoAoA, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the sensitivity RoAoA, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the sensitivity RoAoA, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the sensitivity RoAoA, while φ value being the closest possible to the receiver target reference direction. |

# 3.	Conclusion

This contribution has provided our proposals on the manufacturer declarations for NR BS Type 1-O OTA sensitivity conformance test.

Proposal: To adopt the manufacturer declarations provided above in Table 2 for NR BS Type 1-O OTA sensitivity conformance test.

# References

[1]	3GPP TS 37.145-2 v14.4.0, “Active Antenna System (AAS) Base Station (BS) conformance testing; Part 2: radiated conformance testing”.