3GPP TSG-RAN WG4#86bis                          R4-1804449

Melbourne, Australia, 16-20, Apr, 2018

Agenda item:	7.6.3.2

Source: 	ZTE

Title: 	Further discussion on FR1 dynamic range requirement

Document for:	Approval

# Introduction

In the RAN4#86 meeting, there are two draft CRs approved for 1-C/1-H and 1-O dynamic range requirement, however power level for wanted signal is not calculated correctly. In addition, the power level for 1-C/1-H is also not aligned with that of 1-O. Meanwhile, dynamic requirement for CBW 100MHz with SCS 60KHz is missing in existing spec. Therefore in this contribution, we want to share some further clarification how the power levels are calculated and update the power levels for wanted signal.

# Discussion

For FR1 NR BS, the power level of wanted signal and interfering signal could be calculated as following:

PIntf = -174+10*log10(BWchannel) + NF + 20

Pwanted= = -174+10*log10(NRB*SCS*12) + NF + 20+SNR+IM

Where

- BWchannel is NR channel bandwidth;

- NF is noise figure of NR BS receiver in dB, where 5dB is assumed for WA, 10dB for MR ,13dB for LA;

- IM is implementation margin reserved for BS manufacturer in dB; For high MCS level, IM is assumed to be 2.5dB;

- SNR is the value to satisfy the 95% throughput of measurement channel;

Table1. wanted signal power level for FR1 WA NR BS dynamic range requirement [1]

|  | CBW (MHz) | SCS (KHz) | RB num | SNR [dB] | Wanted signal power[dBm] | Wanted signal power[dBm] |
| --- | --- | --- | --- | --- | --- | --- |
| G-FR1-A2-1 | 5 | 15 | 25 | 9.3 | -70.6679 | -70.7 |
| G-FR1-A2-2 | 5 | 30 | 11 | 9.1 | -71.4230 | -71.4 |
| G-FR1-A2-3 | 10 | 60 | 11 | 9.1 | -68.4127 | -68.4 |
| G-FR1-A2-4 | 20 | 15 | 106 | 9.2 | -64.4942 | -64.5 |
| G-FR1-A2-5 | 20 | 30 | 51 | 9.4 | -64.4613 | -64.5 |
| G-FR1-A2-6 | 20 | 60 | 24 | 9.3 | -64.8246 | -64.8 |

Table2. wanted signal power level for FR1 MR NR BS dynamic range requirement [1]

|  | CBW (MHz) | SCS (KHz) | RB num | SNR [dB] | Wanted signal power[dBm] | Wanted signal power[dBm] |
| --- | --- | --- | --- | --- | --- | --- |
| G-FR1-A2-1 | 5 | 15 | 25 | 9.3 | -70.6679 | -65.7 |
| G-FR1-A2-2 | 5 | 30 | 11 | 9.1 | -71.4230 | -66.4 |
| G-FR1-A2-3 | 10 | 60 | 11 | 9.1 | -68.4127 | -63.4 |
| G-FR1-A2-4 | 20 | 15 | 106 | 9.2 | -64.4942 | -59.5 |
| G-FR1-A2-5 | 20 | 30 | 51 | 9.4 | -64.4613 | -59.5 |
| G-FR1-A2-6 | 20 | 60 | 24 | 9.3 | -64.8246 | -59.8 |

Table3. wanted signal power level for FR1 LA NR BS dynamic range requirement [1]

|  | CBW (MHz) | SCS (KHz) | RB num | SNR [dB] | Wanted signal power[dBm] | Wanted signal power[dBm] |
| --- | --- | --- | --- | --- | --- | --- |
| G-FR1-A2-1 | 5 | 15 | 25 | 9.3 | -70.6679 | -62.7 |
| G-FR1-A2-2 | 5 | 30 | 11 | 9.1 | -71.4230 | -63.4 |
| G-FR1-A2-3 | 10 | 60 | 11 | 9.1 | -68.4127 | -60.4 |
| G-FR1-A2-4 | 20 | 15 | 106 | 9.2 | -64.4942 | -56.5 |
| G-FR1-A2-5 | 20 | 30 | 51 | 9.4 | -64.4613 | -56.5 |
| G-FR1-A2-6 | 20 | 60 | 24 | 9.3 | -64.8246 | -56.8 |

Table 7.3.2-1: Wide Area BS dynamic range

| BS channel bandwidth [MHz] | Subcarrier spacing [kHz] | Reference measurement channel | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A2-1 | -70.7 | -82.5 | AWGN |
|  | 30 | G- FR1-A2-2 | -71.4 |  |  |
| 10 | 15 | G-FR1-A2-1 | -70.7 | -79.3 | AWGN |
|  | 30 | G- FR1-A2-2 | -71.4 |  |  |
|  | 60 | G- FR1-A2-3 | -68.4 |  |  |
| 15 | 15 | G-FR1-A2-1 | -70.7 | -77.5 | AWGN |
|  | 30 | G- FR1-A2-2 | -71.4 |  |  |
|  | 60 | G- FR1-A2-3 | -68.4 |  |  |
| 20 | 15 | G- FR1-A2-4 | -64.5 | -76.2 | AWGN |
|  | 30 | G- FR1-A2-5 | -64.5 |  |  |
|  | 60 | G- FR1-A2-6 | -64.8 |  |  |
| 25 | 15 | G- FR1-A2-4 | -64.5 | -75.2 | AWGN |
|  | 30 | G- FR1-A2-5 | -64.5 |  |  |
|  | 60 | G- FR1-A2-6 | -64.8 |  |  |
| 30 | 15 | G- FR1-A2-4 | -64.5 | -74.4 | AWGN |
|  | 30 | G- FR1-A2-5 | -64.5 |  |  |
|  | 60 | G- FR1-A2-6 | -64.8 |  |  |
| 40 | 15 | G- FR1-A2-4 | -64.5 | -73.1 | AWGN |
|  | 30 | G- FR1-A2-5 | -64.5 |  |  |
|  | 60 | G- FR1-A2-6 | -64.8 |  |  |
| 50 | 15 | G- FR1-A2-4 | -64.5 | -72.2 | AWGN |
|  | 30 | G- FR1-A2-5 | -64.5 |  |  |
|  | 60 | G- FR1-A2-6 | -64.8 |  |  |
| 60 | 30 | G- FR1-A2-5 | -64.5 | [-71.4] | AWGN |
|  | 60 | G- FR1-A2-6 | -64.8 |  |  |
| 70 | 30 | G- FR1-A2-5 | -64.5 | [-70.8] | AWGN |
|  | 60 | G- FR1-A2-6 | -64.8 |  |  |
| 80 | 30 | G- FR1-A2-5 | -64.5 | -70.1 | AWGN |
|  | 60 | G- FR1-A2-6 | -64.8 |  |  |
| 90 | 30 | G- FR1-A2-5 | -64.5 | [-69.6] | AWGN |
|  | 60 | G- FR1-A2-6 | -64.8 |  |  |
| 100 | 30 | G- FR1-A2-5 | -64.5 | -69.1 | AWGN |
|  | 60 | G- FR1-A2-6 | -64.8 |  |  |

Table 7.3.2-2: Medium Range BS dynamic range

| BS channel bandwidth [MHz] | Subcarrier spacing [kHz] | Reference measurement channel | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A2-1 | -65.7 | -77.5 | AWGN |
|  | 30 | G- FR1-A2-2 | -66.4 |  |  |
| 10 | 15 | G-FR1-A2-1 | -65.7 | -74.3 | AWGN |
|  | 30 | G- FR1-A2-2 | -66.4 |  |  |
|  | 60 | G- FR1-A2-3 | -63.4 |  |  |
| 15 | 15 | G-FR1-A2-1 | -65.7 | -72.5 | AWGN |
|  | 30 | G- FR1-A2-2 | -66.4 |  |  |
|  | 60 | G- FR1-A2-3 | -63.4 |  |  |
| 20 | 15 | G- FR1-A2-4 | -59.5 | -71.2 | AWGN |
|  | 30 | G- FR1-A2-5 | -59.5 |  |  |
|  | 60 | G- FR1-A2-6 | -59.8 |  |  |
| 25 | 15 | G- FR1-A2-4 | -59.5 | -70.2 | AWGN |
|  | 30 | G- FR1-A2-5 | -59.5 |  |  |
|  | 60 | G- FR1-A2-6 | -59.8 |  |  |
| 30 | 15 | G- FR1-A2-4 | -59.5 | -69.4 | AWGN |
|  | 30 | G- FR1-A2-5 | -59.5 |  |  |
|  | 60 | G- FR1-A2-6 | -59.8 |  |  |
| 40 | 15 | G- FR1-A2-4 | -59.5 | -68.1 | AWGN |
|  | 30 | G- FR1-A2-5 | -59.5 |  |  |
|  | 60 | G- FR1-A2-6 | -59.8 |  |  |
| 50 | 15 | G- FR1-A2-4 | -59.5 | -67.2 | AWGN |
|  | 30 | G- FR1-A2-5 | -59.5 |  |  |
|  | 60 | G- FR1-A2-6 | -59.8 |  |  |
| 60 | 30 | G- FR1-A2-5 | -59.5 | [-66.4] | AWGN |
|  | 60 | G- FR1-A2-6 | -59.8 |  |  |
| 70 | 30 | G- FR1-A2-5 | -59.5 | [-65.8] | AWGN |
|  | 60 | G- FR1-A2-6 | -59.8 |  |  |
| 80 | 30 | G- FR1-A2-5 | -59.5 | -65.1 | AWGN |
|  | 60 | G- FR1-A2-6 | -59.8 |  |  |
| 90 | 30 | G- FR1-A2-5 | -59.5 | [-64.6] | AWGN |
|  | 60 | G- FR1-A2-6 | -59.8 |  |  |
| 100 | 30 | G- FR1-A2-5 | -59.5 | -64.1 | AWGN |
|  | 60 | G- FR1-A2-6 | -59.8 |  |  |

Table 7.3.2-3: Local Area BS dynamic range

| BS channel bandwidth [MHz] | Subcarrier spacing [kHz] | Reference measurement channel | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A2-1 | -62.7 | -74.5 | AWGN |
|  | 30 | G- FR1-A2-2 | -63.4 |  |  |
| 10 | 15 | G-FR1-A2-1 | -62.7 | -71.3 | AWGN |
|  | 30 | G- FR1-A2-2 | -63.4 |  |  |
|  | 60 | G- FR1-A2-3 | -60.4 |  |  |
| 15 | 15 | G-FR1-A2-1 | -62.7 | -69.5 | AWGN |
|  | 30 | G- FR1-A2-2 | -63.4 |  |  |
|  | 60 | G- FR1-A2-3 | -60.4 |  |  |
| 20 | 15 | G- FR1-A2-4 | -56.5 | -68.2 | AWGN |
|  | 30 | G- FR1-A2-5 | -56.5 |  |  |
|  | 60 | G- FR1-A2-6 | -56.8 |  |  |
| 25 | 15 | G- FR1-A2-4 | -56.5 | -67.2 | AWGN |
|  | 30 | G- FR1-A2-5 | -56.5 |  |  |
|  | 60 | G- FR1-A2-6 | -56.8 |  |  |
| 30 | 15 | G- FR1-A2-4 | -56.5 | -66.4 | AWGN |
|  | 30 | G- FR1-A2-5 | -56.5 |  |  |
|  | 60 | G- FR1-A2-6 | -56.8 |  |  |
| 40 | 15 | G- FR1-A2-4 | -56.5 | -65.1 | AWGN |
|  | 30 | G- FR1-A2-5 | -56.5 |  |  |
|  | 60 | G- FR1-A2-6 | -56.8 |  |  |
| 50 | 15 | G- FR1-A2-4 | -56.5 | -64.2 | AWGN |
|  | 30 | G- FR1-A2-5 | -56.5 |  |  |
|  | 60 | G- FR1-A2-6 | -56.8 |  |  |
| 60 | 30 | G- FR1-A2-5 | -56.5 | [-63.4] | AWGN |
|  | 60 | G- FR1-A2-6 | -56.8 |  |  |
| 70 | 30 | G- FR1-A2-5 | -56.5 | [-62.8] | AWGN |
|  | 60 | G- FR1-A2-6 | -56.8 |  |  |
| 80 | 30 | G- FR1-A2-5 | -56.5 | -62.1 | AWGN |
|  | 60 | G- FR1-A2-6 | -56.8 |  |  |
| 90 | 30 | G- FR1-A2-5 | -56.5 | [-61.6] | AWGN |
|  | 60 | G- FR1-A2-6 | -56.8 |  |  |
| 100 | 30 | G- FR1-A2-5 | -56.5 | -61.1 | AWGN |
|  | 60 | G- FR1-A2-5 | -56.8 |  |  |

# Conclusions

In this proposal, we share some further clarification how the power levels are calculated and update the power levels for wanted signal.

Proposal: to use the power levels for wanted signal in table 7.3.2-1/2/3 for FR1 dynamic range requirement.

# References

[1] R4-1802315, Summary of simulation results for NR BS Rx requirements, Ericsson, approved.

[2] R4-1803304, Draft CR for TS 38.104: Conducted dynamic Range for BS type 1-C and BS type 1-H, CATT, Nokia, Endorsed

[3] R4-1803305, Draft CR for 38.104: Dynamic range update, Huawei, HiSilicon, Endorsed