3GPP TSG-RAN WG4#86bis                           R4-1804452

Melbourne, Australia, 16-20, Apr, 2018

Agenda item:	7.6.3.7.2

Source: 	ZTE

Title: 	Further discussion on FR2 ICS requirement

Document for:	Approval

# Introduction

In the existing TS38.104, FR2 ICS requirement is not defined yet due to ongoing FR2 OTA REFSENS which is expected to be defined in this meeting. Therefore in this contribution, we share some further considerations on this requirement based on the some agreement or assumption for OTA REFSENS of FR2 WA NR BS.

# Discussion

As indicated in the approved TP [2], FR2 ICS requirement should be defined in the following method.

Table 1. PRB allocation of in-channel selectivity for FR2 NR

| NR channel bandwidth [MHz] | subcarrier spacing[KHz] | Reference measurement channel | Wanted signal PRB | Interfering signal PRB |
| --- | --- | --- | --- | --- |
| 50 | 60 | G-FR2-A1-4 | 33PRB | 33PRB |
| 100,200 | 60 | G-FR2-A1-1 | 66PRB | 66PRB |
| 50 | 120 | G-FR2-A1-5 | 16PRB | 16PRB |
| 100,200,400 | 120 | G-FR2-A1-2 | 32PRB | 32PRB |

In the existing TS38.104, the interfering signal type for ICS requirement has been changed to DFT-S-OFDM, however the RB number of interfering signal is not aligned with basic DFT processing principle (e.g. RB number of interfering signal is 33PRB for 50MHz with SCS 60KHz). Therefore we propose to update the interfering signal PRB number to align with basic DFT processing principle.

Table 2. PRB allocation of in-channel selectivity for FR2 NR

| NR channel bandwidth [MHz] | subcarrier spacing[KHz] | Reference measurement channel | Wanted signal PRB | Interfering signal PRB |
| --- | --- | --- | --- | --- |
| 50 | 60 | G-FR2-A1-4 | 33PRB | 32PRB |
| 100,200 | 60 | G-FR2-A1-1 | 66PRB | 64PRB |
| 50 | 120 | G-FR2-A1-5 | 16PRB | 16PRB |
| 100,200,400 | 120 | G-FR2-A1-2 | 32PRB | 32PRB |

Proposal1: to update the interfering PRB number as shown in Table2..

Wanted signal power level for ICS requirement for BS type 2-O could be calculated as following:

Wanted signal power level=-174dBm/Hz+10*log10(BWwanted)+NF+SNR+IM-G+3dB

=EISREFSENS+3dB;

Interfering signal power level=-174dBm/Hz+10*log10(BWInterfering)+NF+ICS-G

=EISREFSENS-SNR-IM+ICS-10*log10(BWwanted)+ 10*log10(BWInterfering)

~= EISREFSENS+13dB;

Where:

- SNR is assumed as -1dB which has been used for FR2 OTA REFSENS in [3];

- IM is implementation margin which is assumed as 2dB;

- ICS is assumed as 14dB for all BS type.

Based on the above formula, we could obtain the following FR2 ICS requirement.

Table 3. FR2 OTA ICS requirement

| BS channel bandwidth [MHz] | Subcarrier spacing[KHz] | Reference measurement channel | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 50 | 60 | G-FR2-A1-4 | EISREFSENS+3dB | EISREFSENS+13dB | DFT-S-OFDM NR signal, SCS 60 kHz, 32 PRB |
| 100,200 | 60 | G-FR2-A1-1 |  |  | DFT-S-OFDM NR signal, SCS 60 kHz, 64PRB |
| 50 | 120 | G-FR2-A1-5 |  |  | DFT-S-OFDM NR signal, SCS 120 kHz, 16PRB |
| 100,200,400 | 120 | G-FR2-A1-2 |  |  | DFT-S-OFDM NR signal, SCS 120 kHz, 32PRB |
| NOTE: Wanted and interfering signal are placed adjacently around Fc |  |  |  |  |  |

Proposal2: propose to specify the FR2 ICS requirement as shown in Table3.

In addition, channel bandwidth assumed for FR2 OTA REFSENS is 50MHz which could only cover G-FR2-A1-1 and G-FR2-A1-2. For the G-FR2-A1-4 and G-FR2-A1-5, the channel bandwidth is ~25MHz, therefore we propose the low boundary of EISREFSENS should be lowered by 3dB further.

Proposal2: propose to decrease low boundary of EISREFSENS for G-FR2-A1-4 and G-FR2-A1-5 by 3dB further.

# Conclusions

In this proposal, we share some further considerations on FR2 ICS requirement and proposals are made as following:

Proposal1: to update the interfering PRB number as shown in Table2.

Table 2. PRB allocation of in-channel selectivity for FR2 NR

| NR channel bandwidth [MHz] | subcarrier spacing[KHz] | Reference measurement channel | Wanted signal PRB | Interfering signal PRB |
| --- | --- | --- | --- | --- |
| 50 | 60 | G-FR2-A1-4 | 33PRB | 32PRB |
| 100,200 | 60 | G-FR2-A1-1 | 66PRB | 64PRB |
| 50 | 120 | G-FR2-A1-5 | 16PRB | 16PRB |
| 100,200,400 | 120 | G-FR2-A1-2 | 32PRB | 32PRB |

Proposal2: propose to specify the FR2 ICS requirement as shown in Table3.

Table 3. FR2 OTA ICS requirement

| BS channel bandwidth [MHz] | Subcarrier spacing[KHz] | Reference measurement channel | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 50 | 60 | G-FR2-A1-4 | EISREFSENS+3dB | EISREFSENS+13dB | DFT-S-OFDM NR signal, SCS 60 kHz, 32 PRB |
| 100,200 | 60 | G-FR2-A1-1 |  |  | DFT-S-OFDM NR signal, SCS 60 kHz, 64PRB |
| 50 | 120 | G-FR2-A1-5 |  |  | DFT-S-OFDM NR signal, SCS 120 kHz, 16PRB |
| 100,200,400 | 120 | G-FR2-A1-2 |  |  | DFT-S-OFDM NR signal, SCS 120 kHz, 32PRB |
| NOTE: Wanted and interfering signal are placed adjacently around Fc |  |  |  |  |  |

Proposal3: propose to decrease low boundary of EISREFSENS for G-FR2-A1-4 and G-FR2-A1-5 by 3dB further.

# References

[1] R4-1802315, Summary of simulation results for NR BS Rx requirements, Ericsson, approved.

[2] R4-1803323, TP to TR38.817 ICS requirement (Section 10.9),ZTE, approved.

[3] R4-1803302,WF on FR2 receiver sensitivity range, Huawei, approved.