3GPP TSG-RAN WG4 Meeting #86bis                 	R4-1804554

Melbourne, Australia, April 16th – 20th 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation results and remaining parameters for SPUSCH

Agenda item:	6.20.6.1

Document for:	Discussion

# 1	Introduction

In the previous RAN4 meeting in Athens, first way forward for simulation test setup for sTTI BS demodulation was agreed in [1]. Simulation parameters were agreed for SPUSCH and SPUCCH demodulation tests. While most of the parameters were agreed, some parameters still remained open. In this contribution, we first discuss the remaining parameters, and then provide initial simulation results based on the agreed and proposed test setup for SPUSCH.

2	Discussion

## 2.1 Agreed simulation parameters

Simulation parameters as defined for SPUSCH in [1] are repeated below.

Table 1: General SPUSCH parameters.

| Parameter | Value |
| --- | --- |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 2, 4 |
| Propagation condition | EPA5 |
| Cyclic prefix | Normal |
| Reference receiver | MRC |
| Channel bandwidth | [5 MHz], 10 MHz, 15 MHz, 20 MHz |
| TTI length | Subslot TTI |
| Number of OFDM-symbols | Subslot TTI: 2 and 3 |
| Fraction of maximum throughput | 70 % |
| Maximum number of HARQ transmissions | 4 |
| DMRS configuration | IFDMA is not configured |
| Number of HARQ processes | 16 |
| RV sequence | 0, 2, 3, 1, 0, 2, 3, 1 |
| Modulation | 16QAM |

- DMRS pattern for subslot-based PUSCH:

  - Option 1:

    - With DMRS sharing RDD DD DD RD DD RDD

  - Option 2:

    - With DMRS sharing: RDD DD|R RD DD|R RD RDD

- UCI on PUSCH:

  - Not define performance requirements for UCI on sPUSCH

## 2.2 Discussion on remaining parameters

The most significant parameter that was left open was DMRS pattern for SPUSCH. In [1], two options were listed:

- DMRS pattern for subslot-based PUSCH:

  - Option 1:

    - With DMRS sharing RDD DD DD RD DD RDD

  - Option 2:

    - With DMRS sharing: RDD DD|R RD DD|R RD RDD

While both options are realistic from implementation point of view, we think both of them can be used. In our simulation setup we have used Option 2, which we also propose to be used as the default DMRS pattern in sTTI demodulation tests.

Use DMRS pattern RDD DD|R RD DD|R RD RDD with DRMS sharing in SPUSCH demodulation tests.

In the last meeting there was one offline proposal about FRCs for SPUSCH. After checking, we think the proposed FRCs as shown in Table 2 are reasonable and we have also used these in our simulations.

Table 2: FRC parameters for performance requirements (16QAM 3/4).

| Reference channel | Ax-1 |  | Ax-2 |  | Ax-3 |  | Ax-4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Allocated resource blocks | 24 |  | 48 |  | 72 |  | 100 |  |
| DFT-OFDM Symbols per subframe | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 |
| Modulation | 16QAM |  | 16QAM |  | 16QAM |  | 16QAM |  |
| Code rate | 3/4 |  | 3/4 |  | 3/4 |  | 3/4 |  |
| Payload size (bits) | 872 | 1736 | 1736 | 3496 | 2536 | 5160 | 3624 | 7224 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 |
| Total number of bits per sub-frame | 1152 | 2304 | 2304 | 4608 | 3456 | 6912 | 4800 | 9600 |
| Total symbols per sub-frame | 288 | 576 | 576 | 1152 | 864 | 1728 | 1200 | 2400 |

Use FRCs as defined in Table 2 for SPUSCH demodulation tests.

3	Simulation results for SPUSCH

In Table 3, we introduce simulation results for SPUSCH without DMRS sharing. We have used RDD-only pattern in the simulations for the first results. Updated results with DMRS sharing are to be provided later.

Table 3: Initial simulation results for SPUSCH without DMRS sharing.

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix | FRC | Fraction of maximum throughput | SNR, dB |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | EPA 5Hz Low | Ax-1 | 70% | 10.91 |
|  |  |  |  | Ax-2 | 70% | 11 |
|  |  |  |  | Ax-3 | 70% | 11.21 |
|  |  |  |  | Ax-4 | 70% | 11.47 |
|  | 4 | Normal | EPA 5Hz Low | Ax-1 | 70% | 7.26 |
|  |  |  |  | Ax-2 | 70% | 7.27 |
|  |  |  |  | Ax-3 | 70% | 7.22 |
|  |  |  |  | Ax-4 | 70% | 7.63 |

4	Conclusion

In this contribution, we have discussed sTTI SPUSCH demodulation test setup and given the first simulation results. We have made the following proposals and introduced first set of simulation results.

Use DMRS pattern RDD DD|R RD DD|R RD RDD with DRMS sharing in SPUSCH demodulation tests.

- Use FRCs as defined in Table 2 for SPUSCH demodulation tests.

References

- R4-1803102, WF on BS demodulation for sTTI and PT, Ericsson, Huawei, Nokia, 3GPP TSG-RAN WG4 Meeting #86, Athens,Greece, 26 Feb – 2 Mar 2018.