3GPP TSG-RAN WG4 Meeting #86bis	R4-1804642

Melbourne, Australia, 16 – 20 April 2018

Agenda Item:	6.20.7.2.1

Source:	Ericsson

Title:	Simulation results of CRS based CQI reporting for sTTI

Document for:	Discussion

# 1	Introduction

RAN4#86 agreed with the simulation assumption for CQI reporting test for sTTI [1]. In this contribution we provide the initial simulation result for discussion.

# 2	Simulation results

## 2.1	Wideband CQI reporting

According the existing wideband CQI reporting requirements, test metrics consist of three factors:

a)	a CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least  % of the time;

b)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ ;

c)	when transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater or equal to [0.02]

Figure 1 shows the median wideband CQI over SNR and throughput ratio specified in b) above. Table 1 and Table 2 are the simulation results analysis with regard to the metrics. We choose the test point for median CQI indexes for 4, 8, and 12, corresponding to 1/3 QPSK, 0.5 16QAM and 0.65 64QAM. It is observed the ratio of throughputs is more than 1.3 for most test points. Since the SNR test points corresponding to 1/3 QPSK is around -5dB and it is low, we can choose two test points corresponding to 16QAM and 64QAM.

Figure 1	(Left) Median CQI and (Right) throughput ratio between followed CQI and fixed median CQI.

Table 1	Analysis of wideband CQI reporting (Slot-based PDSCH).

| SNR [dB] | Median CQI | 1-Prob(medCQI, medCQI+1, MedCQI-1) | Gamma | BLER |
| --- | --- | --- | --- | --- |
| -6 | 4 | 0.34 | 1.45 | 0.20 |
| -5 | 4 | 0.33 | 1.39 | 0.17 |
| -4 | 4 | 0.38 | 1.41 | 0.13 |
| 3 | 8 | 0.35 | 1.41 | 0.08 |
| 4 | 8 | 0.39 | 1.32 | 0.08 |
| 11 | 12 | 0.37 | 1.48 | 0.10 |
| 12 | 12 | 0.37 | 1.34 | 0.10 |

Table 2	Analysis of wideband CQI reporting (Subslot-based PDSCH).

| SNR [dB] | Median CQI | 1-Prob(medCQI, medCQI+1, MedCQI-1) | Gamma | BLER |
| --- | --- | --- | --- | --- |
| -5 | 4 | 0.33 | 1.36 | 0.22 |
| -4 | 4 | 0.37 | 1.38 | 0.19 |
| 3 | 8 | 0.36 | 1.30 | 0.18 |
| 4 | 8 | 0.40 | 1.21 | 0.19 |
| 11 | 12 | 0.35 | 1.43 | 0.11 |
| 12 | 12 | 0.35 | 1.30 | 0.09 |

## 2.2	Subband CQI reporting

According the existing subband CQI reporting requirements, test metrics consist of three factors:

a)	a sub-band differential CQI offset level of 0 shall be reported at least  % of the time but less than % for each sub-band;

b)	the ratio of the throughput obtained when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected sub-band in set S shall be ≥ ;

c)	when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS, the average BLER for the indicated transport formats shall be greater or equal to [0.02].

For subbaband CQI test, RAN4 uses two tap model given as follows:

ht,τ=δτ+aexp-i2πfdtδτ-τd

And usually RAN4 sets a=1, τd=0.45μs, and fd=5 Hz.

### 2.2.1	Delay=0.45us

Figure 2 shows the median CQI and ratio of throughputs in the test metric b) above with tau_d=0.45us, and Table 3/Table 4 show the analysis of the simulation results with regard to the test metrics. It is observed especially for slot-based PDSCH that gamma values are less than 1.0 at several test points. According to our analysis, it is because the subband CQI reporting for sTTI is based on 12 PRBs for 10MHz. With the existing tau_d=0.45us, there no difference in CQI index among subbands.

Figure 2	(Left) Median CQI and (Right) throughput ratio between followed CQI and fixed median CQI with random SB. Delay=0.45us.

Table 3	Analysis of subband CQI reporting with delay of 0.45us (Slot-based PDSCH).

| SNR [dB] | Median CQI | Prob(CQI offset 0) | Gamma | BLER |
| --- | --- | --- | --- | --- |
| -6 | 4 | 0.78 | 0.94 | 0.14 |
| -5 | 4 | 0.90 | 0.89 | 0.20 |
| 2 | 8 | 0.58 | 1.24 | 0.05 |
| 3 | 8 | 0.81 | 1.00 | 0.06 |
| 4 | 8 | 0.73 | 0.91 | 0.19 |
| 11 | 12 | 0.58 | 0.99 | 0.08 |
| 12 | 12 | 0.76 | 0.93 | 0.13 |
| 13 | 12 | 0.80 | 0.96 | 0.12 |
| 14 | 12 | 0.54 | 0.97 | 0.13 |

Table 4	Analysis of subband CQI reporting with delay of 0.45us (Subslot-based PDSCH).

| SNR [dB] | Median CQI | Prob(CQI offset 0) | Gamma | BLER |
| --- | --- | --- | --- | --- |
| -6 | 4 | 0.74 | 1.19 | 0.34 |
| -5 | 4 | 0.75 | 1.21 | 0.17 |
| 2 | 8 | 0.59 | 0.98 | 0.45 |
| 3 | 8 | 0.78 | 1.09 | 0.21 |
| 4 | 8 | 0.78 | 1.13 | 0.11 |
| 5 | 8 | 0.56 | 1.25 | 0.11 |
| 12 | 12 | 0.69 | 1.26 | 0.05 |
| 13 | 12 | 0.80 | 1.19 | 0.01 |
| 14 | 12 | 0.64 | 1.18 | 0.02 |

### 2.2.2	Delay=0.20us

According to the observation we set tau_d=0.20us so to realize frequency selective condition even for subband CQI based on 12PRB.

Figure 3 shows median CQI and ratio of throughputs with tau_d=0.20us and Table 5/Table 6 show the analysis of simulation results with regard to the test metrics. It is observed that Gamma factors are more than 1.8 for both slot-/subslot-PDSCH scenarios.

Figure 3	(Left) Median CQI and (Right) throughput ratio between followed CQI and fixed median CQI with random SB. Delay=0.20us.

Table 5	Analysis of subband CQI reporting with delay of 0.20us (Slot-based PDSCH).

| SNR [dB] | Median CQI | Prob(CQI offset 0) | Gamma | BLER |
| --- | --- | --- | --- | --- |
| -7 | 4 | 0.18 | 1.98 | 0.09 |
| -6 | 4 | 0.17 | 1.97 | 0.12 |
| -5 | 4 | 0.16 | 2.20 | 0.08 |
| 2 | 8 | 0.12 | 1.92 | 0.59 |
| 3 | 8 | 0.12 | 1.97 | 0.06 |
| 4 | 8 | 0.12 | 1.95 | 0.08 |
| 12 | 12 | 0.08 | 1.91 | 0.06 |
| 13 | 12 | 0.09 | 1.81 | 0.1 |
| 14 | 12 | 0.08 | 1.80 | 0.1 |

Table 6	Analysis of subband CQI reporting with delay of 0.20us (Subslot-based PDSCH).

| SNR [dB] | Median CQI | Prob(CQI offset 0) | Gamma | BLER |
| --- | --- | --- | --- | --- |
| -6 | 4 | 0.17 | 2.19 | 0.32 |
| -5 | 4 | 0.16 | 2.54 | 0.28 |
| 2 | 8 | 0.12 | 2.03 | 0.27 |
| 3 | 8 | 0.12 | 2.26 | 0.16 |
| 4 | 8 | 0.12 | 2.28 | 0.13 |
| 5 | 8 | 0.10 | 2.31 | 0.14 |
| 12 | 12 | 0.08 | 1.94 | 0.17 |
| 13 | 12 | 0.10 | 1.91 | 0.14 |
| 14 | 12 | 0.09 | 1.92 | 0.10 |

# 3	Conclusion

Observation: For subband CQI reporting, the parameter tau_d=0.45us in the propagation model is not suitable to verify subband CQI reporting because subband CQI is based on 12PRBs.

Proposal 1: If RAN4 will specify the subband CQI reporting requirements for sTTI, set smaller tau_d for two-path propagation model such as 0.20us. If RAN4 does not change the tau_d, RAN4 will specify the wideband CQI reporting requirements for sTTI.

Proposal 2: For CQI reporting test, RAN4 will set 2 test points; one corresponding 16QAM and another for 64QAM.

# References

- R4-1803013 “Way forward on sTTI UE demodulation and CSI requirements”, Ericsson.

- R4-1804639 “Update of simulation assumption for sTTI UE demodulation and CSI requirements”, Ericsson.