3GPP TSG-RAN WG4 Meeting #86 bis	R4-1804894

Melbourne, Australia, 16 April – 20 April 2018

Source:	Ericsson, Telstra

Title:	TP to TR 37.863-03-01: TP for DC_3A-7A-28A_n78

Agenda item:	7.1.3.3.3

Document for:	Approval

# 1. Introduction

This is a text proposal for 37.863-03-01 for DC_3A-7A-28A_n78.

# 2. Text Proposal for TR 37.863-03-01

##### ---Start of changes---

## 6.x DC_3A-7A-28A_n78A _BCS0

### 6.x.1	Operating bands for DC

# Table 6.x.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-7A-28A_n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz |  |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.x.2	Channel bandwidths per operating band for DC

# Table 6.x.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| DC_3A-7A-28A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 160 | 0 |
|  | 7 | 15 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |

### 6.x.3	Co-existence studies

Tables 6.x.3-1 and 6.x.3-2 list Band 7A-28A + Band n78A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

# Table 6.x.3-1: Band 7 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 |  | 3300 | 3800 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 5000 – 5140 |  |  | 6600 – 7600 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 7500 – 7710 |  |  | 9900 – 11400 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 730 – 1300 |  |  | 5800 – 6370 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 1200 – 1840 |  |  | 4030 – 5100 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 8300 – 8940 |  |  | 9100 – 10170 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 2400 – 2670 |  |  | 3280 – 3820 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3700 – 4410 |  |  | 7330 – 8900 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 1460 – 2600 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10800 – 11510 |  |  | 12400 – 13970 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 11600 – 12740 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10630 – 12700 |  |  | 6200 – 6980 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4760 – 6400 |  |  | 100 – 1110 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700 – 17770 |  |  | 13300 – 14080 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900 – 16540 |  |  | 14100 – 15310 |  |  |

Based on Table 6.x.3-1,

- 2nd order IMD may fall into own Rx of band 28 and Rx frequencies of Band 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 44, 46, 47, 67, and 68

- 3rd order IMD may fall into Rx frequencies of Band 3, 11, 21, 22, 24, 32, 38, 41, 42, 43, 45, 48 n77 and n78

- 4th order IMD may fall into Rx frequencies of Band 1, 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 30, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43, 45, 65, 66, 69, 70, n77 and n78

- 5th order IMD may fall into own Rx of band 28 and Rx frequencies of Band 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 29, 31, 44, 46, 47, 67, 68

- It should be noted that IMD will not be an issue for Band n78 (no self-interference for the TDD band) even through the IMD products may fall into the concerning band

# Table 6.x.3-2: Band 28 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 3300 | 3800 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 6600 |  | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 9900 |  | 11400 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 3097 | 2552 | 4003 |  | 4548 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 2394 | 1804 | 5852 |  | 6897 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 4706 | 5296 | 7303 |  | 8348 |
| Two-tone 3rd order IMD products | ¦fx_low – max BW fy¦ | ¦fx_high + max BW fy¦ | ¦fy_low – max BW fx¦ |  | ¦fy_high + max BW fx¦ |
| IMD frequency limits (MHz) | 603 | 848 | 3280 |  | 3820 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 6194 | 5104 | 8006 |  | 9096 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ |  | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1691 | 1056 | 9152 |  | 10697 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ |  | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5409 | 6044 | 10603 |  | 12148 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ |  | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14497 | 12452 | 308 |  | 988 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ |  | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13903 | 15948 | 6112 |  | 6792 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ |  | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9994 | 8404 | 4356 |  | 5491 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ |  | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11306 | 12896 | 8709 |  | 9844 |

Based on Table 6.x.3-2,

- 2nd order harmonics may fall into Rx frequencies of Band 11, 21, 32, 45, 50, 51, 74, 75 and 76

- 3rd order harmonics may fall into Rx frequencies of Band 1, 4, 10, 23, 65 and 66

- 2nd order IMD may fall into own Rx of band 7 and Rx frequencies of Band 38, 41, 69, 77 and 79

- 3rd order IMD may fall into Rx frequencies of Band 1, 2, 3, 4, 9, 10, 23, 25, 30, 33, 34, 35, 36, 37, 39, 40, 46, 47, 65, 66, 70 and 79

- 4th order IMD may fall into Rx frequencies of Band 11, 21, 24, 32, 45, 46, 47, 50, 51, 74, 75, 76

- 5th order IMD may fall into own Rx of band 28 and Rx frequencies of Band 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 29, 31, 44, 46, 67, 68, 71, 72 and 79

- It should be noted that IMD will not be an issue for Band n78 (no self-interference for the TDD band) even though the IMD products may fall into the concerning band

# Table 6.x.3-3: Band 3 and Band 78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 |  | 3300 | 3800 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 3420 – 3570 |  |  | 6600 – 7600 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 5130 – 5355 |  |  | 9900 – 11400 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1515 – 2090 |  |  | 5010 – 5585 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 270 – 380 |  |  | 4815 – 5890 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 6720 – 7370 |  |  | 8310 – 9385 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 1610 – 1885 |  |  | 3280 – 3820 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1330 – 2055 |  |  | 8115 – 9690 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 3030 – 4180 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8430 – 9155 |  |  | 11610 – 13185 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 10020 – 11170 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11415 – 13490 |  |  | 3040 – 3840 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6330 – 7980 |  |  | 1245 – 2470 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 – 16985 |  |  | 10140 – 10940 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 – 14970 |  |  | 11730 – 12955 |  |  |

Based on Table 6.x.3-3,

- 2nd order harmonics may fall into own Rx of band n78 and Rx frequencies of Band 22, 42, 48 and n77

- 3rd order harmonics may fall into Rx frequencies of Band 46

- 2nd order IMD may fall into own Rx of band 3 and Rx frequencies of Band 2, 9, 24, 25, 33, 34, 35, 36, 37, 39, 46, and 70

- 3rd order IMD may fall into own Rx of band n78, and Rx frequencies of Band 9, 22, 35, 39, 42, 43, 46, 47, 48 and n77

- 4th order IMD may fall into own Rx of band 3 and n78, and also Rx frequencies of Band 2, 9, 11, 21, 22, 24, 25, 32, 33, 34, 35, 36, 37, 39, 42, 43, 45, 48, 70 and n77

- and 5th order IMD may fall into own Rx of band 3 and n78, and also Rx frequencies of Band 1, 2, 4, 9, 10, 11, 21, 22, 23, 24, 25, 30, 32, 33, 34, 35, 36, 37, 39, 40, 42, 43, 45, 48, 70 and n77

- It should be noted that IMD will not be an issue for Band n78 (no self-interference for the TDD band) even through the IMD products may fall into the concerning band

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS; the harmonics and intermodulation products can have impact on these systems. Table 6.x.3-3 and 6.x.3-4 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.x.3-4: 2UL Band 7 + Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3, 4 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD3. 4 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD3, 4 |
| GPS | 1563 | - | 1587 | Yes |  | IMD3, 4 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD3, 4 |
|  | 2400 | - | 2494 | Yes | Asia | IMD3, 4 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD2, 5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD5 |
|  | 5470 | - | 5725 | Yes |  | IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | IMD2, 5 |

Table 6.x.3-5: 2UL Band 28 + Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD4 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD4 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD4 |
| GPS | 1563 | - | 1587 | Yes |  | IMD4 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3, 4, 5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD4, 5 |
|  | 5470 | - | 5725 | Yes |  | IMD4, 5 |
|  | 5150 | - | 5825 | Yes | Asia | IMD3, 4, 5 |

Table 6.x.3-6: 2UL Band 3 + Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD2, 4, 5 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD2, 4, 5 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD2, 4, 5 |
| GPS | 1563 | - | 1587 | Yes |  | IMD2, 4, 5 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD5 |
|  | 2400 | - | 2494 | Yes | Asia | IMD5 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | Harmonics3, IMD2, 3 |
|  | 5150 | - | 5350 | Yes | Europe | Harmonics3, IMD2, 3 |
|  | 5470 | - | 5725 | Yes |  | IMD2, 3 |
|  | 5150 | - | 5825 | Yes | Asia | Harmonics3, IMD2, 3 |

The protected bands for DC_3A-7A-28A_n78A are covered in the constituent fall-back modes.

### 6.x.4	∆TIB and ∆RIB values

For DC_3A-7A-28A_n78A, the TIB,c and RIB values are given in the tables below. Values are derived from DC_3A-5A-7A_n78A who also have band 3, 7 and n78 combined with a low band.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-7A-28A_n78A | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 28 | 0.6 |
|  | n78 | 0.8 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-7A-28A_n78A | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 28 | 0.2 |
|  | n78 | 0.5 |

### 6.x.5	MSD

For the 4DL/2UL DC cases, the related IMD problems will be solved at lower DC band combinations such as 3DL/2UL DC combos and 2DL/2UL DC combos. Thus, there is no need to specify the MSD requirement for this DC combination.

##### ---End of changes---