3GPP TSG-RAN WG4 Meeting #86bis                 	R4-1805147

Melbourne, Australia, April 16th – 20th 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	NR Inactive Mode and Mobility for SA

Agenda item:	7.9.7

Document for:	Discussion

# 1	Introduction

As in RAN4#86 meeting, most NR idle mode requirements were agreed and left some issues for further discussion. Also, a way forward on NR inactive mode and mobility for SA[1] was agreed.

In this contribution, we will provide our view on the requirements RAN4 would need to work on NR inactive mode and mobility due to support of SA.

2	Discussion

Inactive state is a new state compared to e.g. E-UTRAN where only Idle mode and Connected mode are defined. This means that there are no existing requirements for such state defined in RAN4 earlier.

Support of RRC_INACTIVE state means that RAN4 need to work on a number of new requirements. RAN2 is having the Idle and Inactive requirements in one specification (38.304). Discussion in RAN2 on IDLE/INACTIVE are ongoing and basic principles so far are that it is similar in both states.  Therefore, we propose that in RAN4 we can likely re-use most idle mode requirements. For each agreed idle mode requirements we should then list any difference.

Inactive mode section structure could be aligned with Idle mode section

Anyway, as RAN2 discussion is ongoing, RAN4 would need to wait and align with RAN2 decisions.

## 2.1	Inactive Mode

## 2.1.1	Cell Selection

Main thinking in RAN2 was that if UE cannot reselect to cell then it is same to fallback to IDLE if UE cannot find suitable cell to camp on. But in fact, when UE is sent to INACTIVE state then UE needs to do some sort of cell selection.

It is under discussion in RAN2, RAN4 would need to wait RAN2 decision.

## 2.1.2	Cell Re-selection

Although not much has been agreed in RAN2 officially, the basic principles so far are that reselection is similar in idle mode and inactive mode.

2.1.2.1 UE measurement capability

In a similar manner as for idle mode, RAN4 will need to discuss and decide on the UE measurement capability in inactive mode. RAN4 will need to define the number of NR carriers and E-UTRAN inter-RAT carriers and the total number of carriers the UE as minimum shall be capable of monitoring.

As way forward [1], we have considered the UE measurement capability in inactive mode, the following numbers based on the earlier agreements for inactive mode we propose:

In Inactive mode, the UE operating in NR SA mode shall be capable of supporting at least

-	Intra-frequency carrier, and

-	7 NR Inter-frequency carriers, and

-	Depending on UE capability, 7 FDD E-UTRA inter-frequency carriers, and

-	Depending on UE capability, 7 TDD E-UTRA inter-frequency carriers.

And the Total number of carriers the UE should be capable of monitoring should be 14.

A UE in Inactive mode shall be capable of monitoring a total of at least 14 carrier frequency layers

In [2] we provide a draft CR with text proposal.

2.1.2.2 Evaluation of cell reselection criteria

The potentially necessary requirements of this section would depend on the RAN2 procedures and decisions.

Example:

The UE shall be able to identify new intra-frequency cells and perform SS-RSRP and SS-RSRQ measurements of identified intra-frequency cells without an explicit intra-frequency neighbour list containing physical layer cell identities.

The UE shall evaluate the intra-frequency, inter-frequency and inter-RAT cell reselection criteria defined in [38.331] at least every [FFS]. When a non zero value of [Treselection] is used, the UE shall only perform reselection on an evaluation which occurs simultaneously to, or later than the expiry of the [Treselection] timer.

The measurements the UE shall perform in inactive mode could be: SS-RSRP and SS-RSRQ

2.1.2.3 Maximum interruption in paging reception

This requirement could be covered by the requirement in idle mode unless RAN4 sees a need for introducing additional requirements related to paging interruption in inactive mode.

The requirement of Maximum interruption in paging reception for a UE in Idle mode shall apply also for a UE in inactive mode.

In [3] we provide a draft CR with text proposal.

Above could be applied at least for FR1. Whether presence of beams in FR2 will need to be accounted should be discussed further.

3	Conclusion

In this contribution we discussed the requirements in inactive mode.  We have made the following proposals.

- Inactive mode section structure could be aligned with Idle mode section

- In Inactive mode, the UE operating in NR SA mode shall be capable of supporting at least

- -	Intra-frequency carrier, and

- -	7 NR Inter-frequency carriers, and

- -	Depending on UE capability, 7 FDD E-UTRA inter-frequency carriers, and

- -	Depending on UE capability, 7 TDD E-UTRA inter-frequency carriers.

- A UE in Inactive mode shall be capable of monitoring a total of at least 14 carrier frequency layers

- The requirement of Maximum interruption in paging reception for a UE in Idle mode shall apply also for a UE in inactive mode

References

- R4-1803500, Way Forward of NR Inactive Mode and Mobility for SA, Nokia, Nokia Shanghai Bell

- R4-1805149, CR for 38.133 introducing UE measurement capability for NR Inactive mode, Nokia, Nokia Shanghai Bell

- R4-1805148, CR for 38.133 maximum interruption in paging reception for NR Inactive mode, Nokia, Nokia Shanghai Bell