3GPP TSG-RAN WG4 Meeting #86bis	R4-1805343

# Melbourne, AU, 16th – 20th April, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TR 37.843: On MU of the indoor anechoic chamber test method for ACLR OTA requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-04-06 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For adding MU for an indoor anechoic chamber test method on ACLR OTA requirement |  |  |  |  |  |  |  |  |
| Summary of change: |  | MU of the anechoic chamber test method for ACLR OTA requirement is described |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Appropriate MU for ACLR OTA requirement is not determined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 10.3.1 	Measurement uncertainty for TRP requirements

Based on the definition of the TRP measurement, the measurement uncertainty of the TRP requirement relies on the measurement uncertainty derivation for the directional requirements, with consideration of the additional uncertainty contributors, depending on the OTA test range and sampling grid.

### 10.3.1.[1]	ACLR OTA requirement

### 10.3.1.[1.1]	Indoor anechoic chamber test method

### 10.3.1.[1.1.1] Description

10.3.1.[1.1.2] Test Method limitations and scope

10.3.1.[1.1.3]	Procedure

10.3.1.[1.1.4]	Measurement Budget Format

10.3.1.[1.1.5]	Uncertainty Assessment

# Table 1: Indoor Anechoic Chamber uncertainty assessment for EIRP measurement of ACLR OTA

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz < f ≦ 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | 0.03 | 0.03 | Rectangular | √3 | 1 | 0.02 | 0.02 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | 0.3 | 0.3 | Rectangular | √3 | 1 | 0.17 | 0.17 |
| 3 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 6 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 |  | 0.05 | 0.05 |
| 7 | Uncertainty of the RF Power Measurement Equipment | 0.14 | 0.26 | Gaussian | 1 | 1 | 0.14 | 0.26 |
| 8 | Impedance mismatch in the receiving chain | 0.14 | 0.33 | U-shaped | √2 | 1 | 0.10 | 0.23 |
| 9 | Random uncertainty | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 13 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 14 | Polarization mismatch for reference antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 16 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 | 1 | 0.05 | 0.05 |
| 17 | Uncertainty of the network analyzer | 0.13 | 0.20 | Gaussian | 1 | 1 | 0.13 | 0.20 |
| 18 | Influence of the reference antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 19 | Reference antenna feed cable loss measurement uncertainty | 0.06 | 0.06 | Gaussian | 1 | 1 | 0.06 | 0.06 |
| 20 | Influence of the receiving antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 21 | Uncertainty of the absolute gain of the reference antenna | 0.5 | 0.43 | Rectangular | √3 | 1 | 0.29 | 0.25 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.44 | 0.54 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 0.87 | 1.06 |