3GPP TSG-RAN WG4 Meeting #86bis	R4-1805359

# Melbourne, AU, 16th – 20th April, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TR 37.843: On MU of the indoor anechoic chamber test method for frequency error OTA requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-04-06 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For adding MU for an indoor anechoic chamber test method on frequency error OTA requirement |  |  |  |  |  |  |  |  |
| Summary of change: |  | MU of the anechoic chamber test method for frequency error OTA requirement is described |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Appropriate MU for frequency error OTA requirement is not determined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 10.2.2 	Measurement uncertainty for directional requirements

For the derivation of the measurement uncertainty for direction requirements, the approach already defined in the TR 37.842 [4] shall be reused as much as possible.

### 10.2.2.[1]	Frequency error OTA requirement

### 10.2.2.[1.1] Indoor anechoic chamber method

### 10.2.2.[1.1.1] Description

### 10.2.2.[1.1.2] Test method limitations and scope

### 10.2.2.[1.1.3] Procedure

### 10.2.2.[1.1.4] Measurement budget format

### 10.2.2.[1.1.5] Uncertainty assessment

The measurement uncertainty is the same as the conducted frequency error requirement that was already specified in Rel-13 AAS WI in TR 37.842 [4].