3GPP TSG-RAN WG4 Meeting #86bis	R4-1805533

Melbourne, Australia, 16 – 20 April 2018

Agenda Item:	7.11.2

Source:	Ericsson

Title:	Summary of view on NR UE demodulation and CSI requirements

Document for:	Discussion

# Introduction

Attendees: Intel, Qualcomm, CMCC, Samsung, CATT, Huawei, HiSilicon, MediaTek, Rohde & Schwarz, NTT DOCOMO, LGE, Spirent, Nokia, Ericsson, Keysight.

This contribution summarizes the companies view on the NR UE demodulation and CSI reporting requirements.

# 1	Common setting

# 1.1	Duplex mode

Agreements:

- FR2

  - TDD

- FR1

  - TDD/FDD

# 2	UE demodulation requirements

## 2.1	Test scope

| Company | Proposals |
| --- | --- |
| Samsung | PDSCH |
| NTT DOCOMO | PDSCH, SDR, PDCCH, PBCH |
| Qualcomm | PDSCH |
| Intel | PDSCH, SDR, PDCCH, PBCH |
| Ericsson | PDSCH, PDCCH |
| Huawei, HiSilicon | PDSCH, PDCCH, PBCH |
| CATT | PDCCH |

Agreements:

- RAN4 will specify UE demodulation requirements for

  - PDSCH (normal demodulation and SDR)

  - PDCCH

    - At least DL scheduling grant (DCI 1_0 and/or 1_1)

- FFS for PBCH

## 2.2		UE channel bandwidth and sub-carrier spacing

| Company | FR1 | FR2 |
| --- | --- | --- |
| Samsung | 50MHz and 30kHz | 100MHZ and 60kHz |
| NTT DOCOMO | 10/20MHz and 15kHz for 2Rx 50/100MHz and 30kHz for 4Rx | 100/200MHz and 120kHz for 2Rx |
| Qualcomm | 40MHz and 30kHz for TDD 20MHz and 15kHz for FDD | 100MHz and 120kHz for TDD |
| Intel | 10/15/20MHz and 15/30kHz | 60/120kHz |
| Ericsson | 20/40/60/80/100MHz and 30kHz | 100/150/200MHz and 120kHz |
| Huawei, HiSilicon |  |  |
| CATT |  |  |

Agreements:

- For test setup, set of SCS and UE CBW combinations is FFS. The applicable bands are FFS.

## 2.3	MCS/TBS for PDSCH demodulation

| Company | FR1 | FR2 |
| --- | --- | --- |
| Samsung | 1/3 QPSK, 1/2 16QAM, 3/4 64QAM, 4/5 256QAM |  |
| NTT DOCOMO | QPSK/16QAM/64QAM/256QAM | QPSK/16QAM/64QAM |
| Qualcomm | QPSK/16QAM/64QAM for 1 layer 16QAM/64QAM for 2 layers 16QAM/64QAM for 4 layers | QPSK/16QAM/64QAM for 1 layer 16QAM/64QAM for 2 layers |
| Intel | QPSK/16QAM/64QAM/256QAM | QPSK/16QAM/64QAM |
| Ericsson | QPSK/16QAM/64QAM/256QAM | QPSK/16QAM/64QAM |
| Huawei, HiSilicon |  |  |
| CATT |  |  |

(Note: choose MCS/TBS so that both LPDC base graphs are used)

Agreements:

- FR1

  - QPSK/16QAM/64QAM/256QAM

- FR2

  - QPSK/16QAM/64QAM

## 2.5	Antenna configuration and MIMO layers

| Company | FR1 | FR2 |
| --- | --- | --- |
| Samsung | 2Rx (up to 2 layers) 4Rx (up to 4 layers) |  |
| NTT DOCOMO | UE: both 2Rx and 4Rx BS: up to 8Tx | UE: 2Rx (Applicability TBD) BS: up to 2Tx (FFS: higher Tx) |
| Qualcomm | 1x2 1 layer 2x2 2 layers 4x4 4 layers | 1x2 1 layer 2x2 2 layers |
| Intel | 2Rx (up to 2 layers) 4Rx (up to 4 layers) | 2Rx (up to 2 layers) (single RX panel) |
| Ericsson | 8x2 (up to 2 layers) 8x4 (up to 4 layers) | 2x2 (up to 2 layers) |
| Huawei, HiSilicon |  |  |
| CATT |  |  |

Agreements:

- FR1

  - 2Rx/4Rx

  - Up to 4 layers

- FR2

  - 2Rx

  - Up to 2 layers

## 2.6	Propagation channels

| Company | FR1 | FR2 |
| --- | --- | --- |
| Samsung | TDL | TDL |
| NTT DOCOMO | TDL | TDL |
| Qualcomm | TDL-A (30ns) TDL-B (100ns) TDL-C (300ns) ETU70 (FDD only) (3km/h, 30km/h, and 120km/h) |  |
| Intel | TDL | TDL or modified TDL? |
| Ericsson | EPA/EVA up to CBW=30MHz TDL for CBW>30MHz | TDL |
| Huawei, HiSilicon | TDL with DS=30/100/300ns |  |
| CATT |  |  |

Agreements:

- FR1

  - Simplified TDL channel in TR 38.901

    - FFS for which power delay profile is used.

    - FFS for delay spared

    - FFS how to simplify the models

  - FFS for LTE propagation channels, e.g., EPA/EVA (<= 30MHz UE CBW)

- FR2

  - FFS

## 2.7	TDD slot configuration

| Company | Proposals |
| --- | --- |
| Samsung |  |
| NTT DOCOMO | Same as LTE conf2 for FR1 |
| Qualcomm | DSDU. S(D, Gp, U) = [9, 3, 2] |
| Intel | Semi-static configuration |
| Ericsson | Semi-static configuration DDSU. S(D, Gp, U) = [13, 1, 0] for FR1 DDSU. S(D, Gp, U) = [12, 2, 0] for FR2 |
| Huawei, HiSilicon | Semi-static configuration {DSUUDDSUUD} for FR1 15kHz. S(D, Gp, U)=[12, 1, 1] {DDDSU} for FR1 30kHz. S(D, Gp, U) = [10, 2, 2] {DDDSU} for FR2 60/120kHz. S(D, Gp, U) = [10, 2, 2] |
| CATT |  |

Agreements:

- Only semi-static configuration in Rel-15

## 2.8	Others

### 2.8.1	Common

- Assumed Tx EVM

  - FR1: 6% for QPSK/16QAM/64QAM and 3% for 256QAM

  - FR2: TBD for QPSK/16QAM/64QAM. [FFS for phase noise modeling?]

- Single numerology

- …

### 2.8.1	PDSCH

- Metric for demodulation: [70]% of maximum throughput

- HARQ processes and RV sequence:

- FD RB Interleaving

- PRB bundle size

- DRMS configuration

- CSI-RS for TRS

- PTRS

- QCL

- …

### 2.8.2	PDCCH

- Aggregation level

- CORESET configuration

- DCI format (number of bits)

- …

### 2.8.3	PBCH

- PBCH burst

- …

### 2.8.4	DC/CA

# 3	CSI reporting requirements

## 3.0		Test scope

Agreements:

- CQI reporting

- PMI reporting

- RI reporting

- FFS for:

  - CRI reporting

  - L1-RSRP reporting

  - LI reporting

## 3.1		CQI reporting

| Company | Proposals |
| --- | --- |
| Samsung | Reporting test with Table 1/2/3 |
| NTT DOCOMO |  |
| Qualcomm |  |
| Intel | Reporting test with Table 1/2/3 WB-CQI/SB-CQI |
| Ericsson | Reporting test with Table 1 and 2 WB-CQI |
| Huawei, HiSilicon |  |
| CATT |  |

Common view:

- Introduce wideband CQI reporting test for tables 1 (up to 64QAM) and table 2 (up to 256QAM)

## 3.2	PMI reporting

| Company | Proposals |
| --- | --- |
| Samsung | Type-I single, Type-I multi, Type-II, Type-II port |
| NTT DOCOMO |  |
| Qualcomm |  |
| Intel | WB-PM/SB-PMI, Type-I/Type-II, Up to 32 ports |
| Ericsson |  |
| Huawei, HiSilicon | WB-PMI, Type-I single, 16/32 ports |
| CATT |  |

Common view:

- Introduce wideband PMI reporting test with Type-I codebook up to 32 CSI-RS ports.

## 3.3	Other CSI reporting

| Company | Proposals |
| --- | --- |
| Samsung | L1-RSRP. Consider measurement restriction. Configure CSI-IM. Periodic, Aperiodic, and Semi-persistent |
| NTT DOCOMO |  |
| Qualcomm |  |
| Intel | RI, L1-RSRP Consider measurement restriction. Configure CSI-IM. Periodic, Aperiodic, Semi-persistent |
| Ericsson |  |
| Huawei, HiSilicon |  |
| CATT |  |

# References

- R4-1803713	“Discussion on NR PDCCH demodulation test”, CATT.

- R4-1803843	“Overview of UE performance requirements”, Samsung.

- R4-1804006	“Proposals on NR UE performance test scenarios”,	Ericsson.

- R4-1804085	“Framework for Single-Carrier Demodulation Tests”, Qualcomm.

- R4-1804129	“Discussion on NR UE CSI feedback performance requirements”, Intel.

- R4-1804156	“NR UE demodulation requirements”, Intel.

- R4-1804172	“Views on NR UE performance requirements”, Intel.

- R4-1804174	“Views on NR UE soft buffer requirements”, Intel.

- R4-1804980	“UE demodulation requirements for forward compatibility”, NTT DOCOMO.

- R4-1804981	“Discussion on general assumption for UE demodulation requirements”, NTT DOCOMO.

- R4-1805308	“Discussion on NR FR1 DL demodulation requirements”, Huawei, HiSilicon.

- R4-1805309	“Discussion on NR FR1 CSI requirements”, Huawei, HiSilicon.

- R4-1804167 “Propagation channel models for NR UE performance requirements”, Intel.

- R4-1804648, “Propagation channel model for NR UE demodulation requirements”, Ericsson.

- R4-1805315, “Discussion on NR channel models for performance requirements”, Huawei, HiSilicon.

- R4-1804216, “UL/DL Configuration for Demodulation Tests”, Qualcomm.

- R4-1805307, “DL-UL configuration for NR demodulation performance requirements”, Huawei, HiSilicon.