3GPP TSG-RAN WG4 #86-bis	R4-1805847

Melbourne, Australia, 16th – 20th April 2018

Source:	MVG Industries

Title:	TP to Draft CR 37.843 – MU budget for Near Field Test Range

Agenda item:	6.26.3.1.2

Document for:	Approval

# Introduction

During 3GPP RAN4 #86, a contribution highlighting the test procedures for EVM type of measurements in Near Field Test Range was provided [1].

This contribution is providing the MU format and assessment for TX directional requirements such as EVM, frequency error, and occupied BW measurements in Near Field test Range. The intention of this contribution is to move the current text in TR 37.843 to adopt to this structure agreed in [3].

# Discussion

Near Field is a testing methodology which can be used for radiated transmit power. Basically, the test would require to measure EVM in some manufacturers declared directions (OTA coverage range). The requirement needs to be defined so that all transmitter units are active and the system is operating at the declared maximum rated total radiated power. For conformance testing purposes, the EVM is tested at the maximum and minimum power settings, in order to comply with the total power dynamic range requirement (refer to the Total power dynamic range requirement for E-UTRA in section 5.3).

EVM is a directional requirement, the calibration of the system setup might not be taken into account.

For all the other TX directional requirements the same MU as the EVM MU could be used. The way the standard uncertainties will be combined is FFS.

# Proposal

<< Unchanged sections omitted >>

----------------------------------------------------------------------------------------------------------------------------------------------------

<< start of proposal >>

### 10.2 TX directional requirements

## 10.2.1 General

### 10.2.2 OTA E-UTRA DL RS power

10.2.2.3 Near Field

10.2.2.3.1 General

## 10.2.2.3.2 Calibration

## 10.2.2.3.3 Procedure

10.2.2.3.4 MU assessment

# Table 10.2.2.3.4-1: Near field test range uncertainty contributors in AAS EIRP measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
|  | Stage 2:, EIRP near field radiation pattern measurement and EIRP near field DUT power measurement |  |
| 1 | Axes Intersection | B4-1 |
| 2 | Axes Orthogonality | B4-2 |
| 3 | Horizontal Pointing | B4-3 |
| 4 | Probe Vertical Position | B4-4 |
| 5 | Probe H/V pointing | B4-5 |
| 6 | Measurement Distance | B4-6 |
| 7 | Amplitude and Phase Drift | B4-7 |
| 8 | Amplitude and Phase Noise | B4-8 |
| 9 | Leakage and Crosstalk | B4-9 |
| 10 | Amplitude Non-Linearity | B4-10 |
| 11 | Amplitude and Phase Shift in rotary joints | B4-11 |
| 12 | Channel Balance Amplitude and Phase | B4-12 |
| 13 | Probe Polarization Amplitude and Phase | B4-13 |
| 14 | Probe Pattern Knowledge | B4-14 |
| 15 | Multiple Reflections | B4-15 |
| 16 | Room Scattering | B4-16 |
| 17 | DUT support Scattering | B4-17 |
| 18 | Scan Area Truncation | B4-18 |
| 19 | Sampling Point Offset | B4-19 |
| 20 | Spherical Mode Truncation | B4-20 |
| 21 | Positioning | B4-21 |
| 22 | Probe Array Uniformity | B4-22 |
| 23 | Mismatch of receiver chain (i.e. between receiving antenna and measurement receiver) | B4-23 |
| 24 | Insertion loss of receiver chain | B4-24 |
| 25 | Uncertainty of the absolute gain of the probe antenna | B4-25 |
| 26 | Measurement Receiver | E |
| 27 | Measurement repeatability - Positioning Repeatability | B4-27 |
|  | Stage 1: Calibration measurement |  |
| 28 | Uncertainty of network analyser | E |
| 29 | Mismatch of receiver chain | B4-29 |
| 30 | Insertion loss of receiver chain | B4-30 |
| 31 | Mismatch in the connection of the calibration antenna | B4-31 |
| 32 | Influence of the calibration antenna feed cable | B4-32 |
| 33 | Influence of the probe antenna cable | B4-33 |
| 34 | Uncertainty of the absolute gain of the calibration antenna | E |
| 35 | Short term repeatability | B4-35 |

Note: Refer to TR37.842 Annex B for description of each uncertainty term

10.2.2.3.4.1 MU budget

# Table 10.2.2.3.4.1-1: Near field test range uncertainty assessment for EIRP measurement

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Axes Intersection | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 2 | Axes Orthogonality | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 3 | Horizontal Pointing | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 4 | Probe Vertical Position | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 5 | Probe H/V pointing | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 6 | Measurement Distance | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 7 | Amplitude and Phase Drift | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 8 | Amplitude and Phase Noise | 0.02 | 0.02 | Gaussian | 1.00 | 1 | 0.02 | 0.02 |
| 9 | Leakage and Crosstalk | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 10 | Amplitude Non-Linearity | 0.04 | 0.04 | Gaussian | 1.00 | 1 | 0.04 | 0.04 |
| 11 | Amplitude and Phase Shift in rotary joints | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 12 | Channel Balance Amplitude and Phase | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 13 | Probe Polarization Amplitude and Phase | 0.0001 | 0.0001 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 14 | Probe Pattern Knowledge | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 15 | Multiple Reflections | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 16 | Room Scattering | 0.09 | 0.09 | Gaussian | 1.00 | 1 | 0.09 | 0.09 |
| 17 | DUT support Scattering | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 18 | Scan Area Truncation | 0.003 | 0.003 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 19 | Sampling Point Offset | 0.0058 | 0.0058 | Gaussian | 1.00 | 1 | 0.01 | 0.01 |
| 20 | Spherical Mode Truncation | 0.015 | 0.015 | Gaussian | 1.00 | 1 | 0.02 | 0.02 |
| 21 | Positioning | 0.03 | 0.03 | Rectangular | 1.73 | 1 | 0.02 | 0.02 |
| 22 | Probe Array Uniformity | 0.055 | 0.055 | Gaussian | 1.00 | 1 | 0.06 | 0.06 |
| 23 | Mismatch of receiver chain | 0.284 | 0.284 | U-Shaped | 1.41 | 1 | 0.20 | 0.20 |
| 24 | Insertion loss of receiver chain | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 25 | Uncertainty of the absolute gain of the probe antenna | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 26 | Measurement Receiver | 0.14 | 0.26 | Gaussian | 1.00 | 1 | 0.14 | 0.26 |
| 27 | Measurement repeatability - positioning repeatability | 0.15 | 0.15 | Gaussian | 1.00 | 1 | 0.15 | 0.15 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 28 | Network analyzer | 0.13 | 0.20 | Gaussian | 1.00 | 1 | 0.13 | 0.20 |
| 29 | Mismatch of receiver chain | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 30 | Insertion loss of receiver chain | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 31 | Mismatch in the connection of the calibration antenna | 0.02 | 0.02 | U-Shaped | 1.41 | 1 | 0.01 | 0.01 |
| 32 | Influence of the calibration antenna feed cable | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 33 | Influence of the probe antenna cable | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 34 | Reference antenna | 0.50 | 0.25 | Rectangular | 1.73 | 1 | 0.29 | 0.25 |
| 35 | Short term repeatability | 0.088 | 0.088 | Gaussian | 1.00 | 1 | 0.09 | 0.09 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.45 | 0.50 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 0.89 | 0.99 |

NOTE:	Standard uncertainty values for the signal generator, network analyzer and reference antenna are according to the test equipment vendors proposal, as captured in annex E.

10.2.2.3.4.2 MU value

{editors note: this section may contain additional subclauses}

### 10.2.4 OTA Transmitted Signal Quality – Frequency Error

10.2.4.3 Near Field

10.2.4.3.1 General

10.2.4.3.2 Calibration

10.2.4.3.3 Procedure

10.2.4.3.4 MU assessment

Same uncertainty budget format as for EVM type of measurements.

10.2.4.3.4.1 MU budget

Table 10.2.4.3.4.1-1: Near Field Test Range Uncertainty assessment for eAAS OTA Frequency Error measurement

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz < f ≦ 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Axes Intersection | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 2 | Axes Orthogonality | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 3 | Horizontal Pointing | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 4 | Probe Vertical Position | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 5 | Probe H/V pointing | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 6 | Measurement Distance | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 7 | Amplitude and Phase Drift | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 8 | Amplitude and Phase Noise | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 9 | Leakage and Crosstalk | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 10 | Amplitude Non-Linearity | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 11 | Amplitude and Phase Shift in rotary joints | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 12 | Channel Balance Amplitude and Phase | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 13 | Probe Polarization Amplitude and Phase | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 14 | Probe Pattern Knowledge | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 15 | Multiple Reflections | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 16 | Room Scattering | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 17 | DUT support Scattering | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 18 | Scan Area Truncation | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 19 | Sampling Point Offset | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 20 | Spherical Mode Truncation | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 21 | Positioning | FFS | FFS | Rectangular | 1.73 | 1 | FFS | FFS |
| 22 | Probe Array Uniformity | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 23 | Mismatch of receiver chain | FFS | FFS | U-Shaped | 1.41 | 1 | FFS | FFS |
| 24 | Insertion loss of receiver chain | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 25 | Uncertainty of the absolute gain of the probe antenna | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 26 | RF measurement equipment | FFS | FFS | Rectangular | 1.73 | 1 | FFS | FFS |
| 27 | Measurement repeatability – positioning repeatability | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |

Note: The way the different standard uncertainty terms will be combined is FFS.

10.2.4.3.4.2 MU value

{editors note: this section may contain additional subclauses}

### 10.2.5 OTA Transmitted Signal Quality – EVM

10.2.5.3 Near Field

10.2.5.3.1 General

### 10.2.5.3.2 Test method limitations and scope

10.2.5.3.3 Calibration

10.2.5.3.4 Procedure

10.2.5.3.5 MU assessment

Table 10.2.5.3.5-1: Near Field Test Range Uncertainty contributors for eAAS OTA EVM measurement

| UID | Description of uncertainty contribution | Details in paragraph |
| --- | --- | --- |
| 1 | Axes Intersection | B4-1 |
| 2 | Axes Orthogonality | B4-2 |
| 3 | Horizontal Pointing | B4-3 |
| 4 | Probe Vertical Position | B4-4 |
| 5 | Probe H/V pointing | B4-5 |
| 6 | Measurement Distance | B4-6 |
| 7 | Amplitude and Phase Drift | B4-7 |
| 8 | Amplitude and Phase Noise | B4-8 |
| 9 | Leakage and Crosstalk | B4-9 |
| 10 | Amplitude Non-Linearity | B4-10 |
| 11 | Amplitude and Phase Shift in rotary joints | B4-11 |
| 12 | Channel Balance Amplitude and Phase | B4-12 |
| 13 | Probe Polarization Amplitude and Phase | B4-13 |
| 14 | Probe Pattern Knowledge | B4-14 |
| 15 | Multiple Reflections | B4-15 |
| 16 | Room Scattering | B4-16 |
| 17 | DUT support Scattering | B4-17 |
| 18 | Scan Area Truncation | B4-18 |
| 19 | Sampling Point Offset | B4-19 |
| 20 | Spherical Mode Truncation | B4-20 |
| 21 | Positioning | B4-21 |
| 22 | Probe Array Uniformity | B4-22 |
| 23 | Mismatch of receiver chain (i.e. between receiving antenna and measurement receiver) | B4-23 |
| 24 | Insertion loss of receiver chain | B4-24 |
| 25 | Uncertainty of the absolute gain of the probe antenna | B4-25 |
| 26 | RF Measurement Equipment | E |
| 27 | Measurement repeatability - Positioning Repeatability | B4-27 |

Note: Refer to TR37.842 Annex B for description of each uncertainty term

10.2.5.3.5.1 MU budget

Table 10.2.5.3.5.1-1: Near Field Test Range Uncertainty assessment for eAAS OTA EVM measurement

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz < f ≦ 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Axes Intersection | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 2 | Axes Orthogonality | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 3 | Horizontal Pointing | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 4 | Probe Vertical Position | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 5 | Probe H/V pointing | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 6 | Measurement Distance | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 7 | Amplitude and Phase Drift | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 8 | Amplitude and Phase Noise | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 9 | Leakage and Crosstalk | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 10 | Amplitude Non-Linearity | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 11 | Amplitude and Phase Shift in rotary joints | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 12 | Channel Balance Amplitude and Phase | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 13 | Probe Polarization Amplitude and Phase | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 14 | Probe Pattern Knowledge | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 15 | Multiple Reflections | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 16 | Room Scattering | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 17 | DUT support Scattering | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 18 | Scan Area Truncation | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 19 | Sampling Point Offset | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 20 | Spherical Mode Truncation | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 21 | Positioning | FFS | FFS | Rectangular | 1.73 | 1 | FFS | FFS |
| 22 | Probe Array Uniformity | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 23 | Mismatch of receiver chain | FFS | FFS | U-Shaped | 1.41 | 1 | FFS | FFS |
| 24 | Insertion loss of receiver chain | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 25 | Uncertainty of the absolute gain of the probe antenna | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 26 | RF measurement equipment | FFS | FFS | Rectangular | 1.73 | 1 | FFS | FFS |
| 27 | Measurement repeatability – positioning repeatability | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |

Note: The way the different standard uncertainty terms will be combined is FFS.

10.2.5.3.5.2 MU value

{editors note: this section may contain additional subclauses}

### 10.2.6 OTA Transmitted Signal Quality – Occupied BW

10.2.6.3 Near Field

10.2.6.3.1 General

10.2.6.3.2 Calibration

10.2.6.3.3 Procedure

10.2.6.3.4 MU assessment

Same uncertainty budget format as for EVM type of measurements.

10.2.6.3.4.1 MU budget

Table 10.2.6.3.4.1-1: Near Field Test Range Uncertainty assessment for eAAS OTA Occupied BW measurement

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz < f ≦ 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Axes Intersection | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 2 | Axes Orthogonality | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 3 | Horizontal Pointing | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 4 | Probe Vertical Position | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 5 | Probe H/V pointing | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 6 | Measurement Distance | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 7 | Amplitude and Phase Drift | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 8 | Amplitude and Phase Noise | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 9 | Leakage and Crosstalk | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 10 | Amplitude Non-Linearity | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 11 | Amplitude and Phase Shift in rotary joints | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 12 | Channel Balance Amplitude and Phase | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 13 | Probe Polarization Amplitude and Phase | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 14 | Probe Pattern Knowledge | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 15 | Multiple Reflections | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 16 | Room Scattering | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 17 | DUT support Scattering | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 18 | Scan Area Truncation | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 19 | Sampling Point Offset | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 20 | Spherical Mode Truncation | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 21 | Positioning | FFS | FFS | Rectangular | 1.73 | 1 | FFS | FFS |
| 22 | Probe Array Uniformity | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 23 | Mismatch of receiver chain | FFS | FFS | U-Shaped | 1.41 | 1 | FFS | FFS |
| 24 | Insertion loss of receiver chain | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 25 | Uncertainty of the absolute gain of the probe antenna | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 26 | RF measurement equipment | FFS | FFS | Rectangular | 1.73 | 1 | FFS | FFS |
| 27 | Measurement repeatability – positioning repeatability | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |

Note: The way the different standard uncertainty terms will be combined is FFS.

10.2.6.3.4.2 MU value

{editors note: this section may contain additional subclauses}

<< end of proposal >>

# Conclusion

Text proposal to TR 37.843 is presented in order to add the MU for TX directional requirements measurements in a Near Field test range.

# Reference

[1] R4-1803039, “Test Procedure for OTA EVM measurement in a Near Field Test Range”, Feb 2018

[2] 3GPP TR 37.842 v 13.2.0, March 2017

[3] R4-18xxxxx – “Structure”, Docomo