3GPP TSG RAN WG4 Meeting #86bis	R4-1803602

Melbourne, AU, 16th – 20th April, 2018

3GPP TSG RAN WG1 Meeting 92						         R1-1803150

Athens, Greece, February 26 – March 2, 2018

Title:	LS on wake-up signal

Response to:	-

Release:	Rel-15

Work Item:	NB_IOTenh2-Core, LTE_eMTC4-Core

Source:	TSG RAN WG1

To:	TSG RAN WG4

Cc:	TSG RAN WG2

Contact Person:

#### Name:	Xiaolei Tie

E-mail Address:	tiexiaolei@hisilicon.com

1. Overall Description:

RAN1 has made the single working assumption for NB-IoT and eMTC included in the Annex, and would like to request RAN4’s feedback on the feasibility of RRM measurement relaxation in the part on NB-IoT WUS.

2. Actions:

To RAN4

RAN1 respectfully asks RAN4 to take the above information into account, and to provide input on the feasibility of RRM measurement relaxation in the part on NB-IoT WUS.

3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #92bis	16-20 April 2018 	             	Sanya, CN

TSG RAN WG1 Meeting #93	21-25 May 2018			Busan, KR

4. Annex

Working assumption

on NB-IoT WUS: - For UE operating WUS, UE is allowed to relax RRM measurements, at least for low mobility UEs, to once every N DRX cycles, where N is FFS - The RRM measurement relaxation is enabled/disabled by the network. - WUS can still be enabled by the network when the RRM measurement relaxation is disabled. - Note: This does not imply any change in the random access procedure / power control / CE level selection, nor relaxations in the requirements related to the random access procedure. - WUS provides sync of up to the timing/frequency offset resulting from not synchronizing for N DRX cycles - FFS for eDRX case - FFS whether N depends on the length of PTW. - For 164dB MCL, there is a WUS configuration that enables sync for at least a value of N>1 for at least the smallest DRX cycle. - FFS for other DRX cycles and values of N - FFS whether N is fixed, configurable, or depends on the DRX cycle on eMTC WUS/sync: - For eMTC, a new periodic synchronization signal is introduced. - The new periodic sync signal is configurable (including OFF/ON configuration) - FFS on the functionality/information provided by the synchronization signal, including - whether the additional synchronization signal can provide WUS-related information for a subset or a group of POs - In case the additional synchronization signal provides WUS-related information, FFS whether there is an additional WUS/DTX signal, which may be separately configured. - In case the additional synchronization signal does not provides WUS-related information, there is an additional WUS/DTX signal - System information change notification - FFS on location of the sync signal - The new synchronization signal can be used also for non-WUS purposes (e.g. by UEs that need to synchronize after exiting PSM state) - FFS whether the “new sync signal” can reuse the NB-IoT WUS sequence or a different sequence.