3GPP TSG-RAN WG4 #87	R4-1806161

Busan, Korea, 21 May-26 May 2018

Agenda Item:	6.22.6.2

Source:	Ericsson

Title:	Simulation results for sPUCCH

Document for:	Discussion

# Introduction

In this contribution, initial simulation results for sPUCCH is proposed. The simulation assumption is aligned with [1].

# Simulation assumption for sPUCCH

In Table 1, simulation assumption is given.

Table 1: Simulation assumption for sPUCCH

| Parameter | Value |
| --- | --- |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 2, 4 |
| Propagation condition | EPA5 |
| Cyclic prefix | Normal |
| Reference receiver | MRC |
| Channel bandwidth | [5 MHz], 10 MHz, 15 MHz, 20 MHz |
| TTI length | Subslot TTI |
| Number of OFDM-symbols | Subslot TTI: 2 and 3 |

# Simulation results for Format 1a

In this section, simulation results for Format 1a are given. In the simulation, we reuse the metric defined in 36.104 for PUCCH format 1a. In PUCCH format 1a, the ACK missed detection probability is given as the metric for the final performance requirements. The ACK missed detection probability is the probability of not detecting an ACK when an ACK was sent. The results for 10MHz are shown in Figure 1 and Figure 2.

Figure 1: ACK missed detection for Format 1a with 2RX (10MHz)

Figure 2: ACK missed detection for Format 1a with 4RX (10MHz)

From Figure 1 and Figure 2, we can see that the target SNR of 10-2 ACK missed detection is about 0 and -4 dB for 2 Rx and 4 Rx, respectively. Further, we can see that if we only run sPUCCH with 2 OS, the performance is 1.2 dB worse than that with 3 OS, and the error from sPUCCH with 2 OS is dominated the error of the final results.

# Simulation results for Format 4

In this section, the ACK missed detection performances are given for Format 4.

Figure 3: ACK missed detection for Format 4 with 2 RX (10MHz)

Figure 4: ACK missed detection for Format 4 with 4 RX (10MHz)

From Figure 3 and Figure 4, we can see that the target SNR of 10-2 ACK missed detection is about 0.7 and -5.2 dB for 2 Rx and 4 Rx, respectively. Further, we can see that if we only run sPUCCH with 2 OS, the performance is 1.5 dB worse than that with 3 OS, and the error from sPUCCH with 2 OS is dominated the error of the final results. The results for 10MHz are shown in Figure 3 and Figure 4.

# Conclusion

In this contribution, initial simulation assumption is proposed for sPUCCH. We hope the group can consider these simulation results in the sPUCCH performance requirements discussion.

# Reference

R4-1803102, WF on BS demodulation for sTTI and PT, Ericsson, Huawei, Nokia