3GPP TSG-RAN WG4 Meeting #87                                                                R4-1806320

Busan, KR, 21th – 25th May, 2018

Agenda item:	6.21.4.2

Source:	Intel Corporation

Title:	SDR requirements with 1024QAM

Document for:	Discussion

- Introduction

In RAN4#86bis demodulation and CSI requirements for high capacity stationary wireless link and 1024QAM were discussed and a way forward [1] was approved. In this paper we provide simulation results and discuss SDR requirements with 1024QAM.

- Discussion

In RAN4#86bis the following common parameters for SDR tests were agreed:

- Transmission Mode: TM3

- Antenna Configuration: 2 x 2 for 2-layer transmission per CC, and 4 x 4 for 4-layer transmission per CC

- CFI: 1

Companies were encouraged to provide simulation results to decide target SNR range and MCS for SDR tests.

## SDR Requirements for 2-layers

The following parameters were used to evaluate maximum achievable throughput in static channel conditions with 2 layers:

- Antenna configuration: 2x2

- Tx EVM: 1.5%, 2%

- Rx EVM: 0

- # Subframes of PDSCH transmission: 8

- MCS: 26

- Transmission Mode: TM3

- System BW: 20MHz, 10MHz, 15MHz, 5MHz

- CFI: 1

Figure 1: SDR performance with 2 layers

With 2 layers in 20MHz, with TxEVM=2%, max throughput is not achieved with MCS of 26. Also the degradation with 2% Tx EVM compared to 1.5% is > 3dB. When practical Rx EVM is taken into account the performance could be further degraded and we recommend using MCS of 25 for 2x2 2 layers in 20MHz.

Observation #1: Tx EVM of 2% has > 3dB performance degradation compared to 1.5% with 2 layers in 20MHz

Observation #2: For 20MHz with 2 layers, max throughput is not achieved with MCS of 26

Proposal #1: For 20MHz 2 layers SDR requirement define test with MCS of 25

With 15MHz, 10MHz and 5MHz MCS 26 could be used for SDR requirements as max throughput is achieved with MCS of 26 and Tx EVM of 2%.

Proposal #2: For 15MHz, 10MHz, 5MHz 2 layers SDR requirement define test with MCS of 26

Based on the proposals we recommend the following reference channels for 1024QAM with 2 layers per CC in Table 1.

Table 1: Fixed reference channels for 1024QAM SDR tests with 2 layers

| BW | 20MHz | 15MHz | 10MHz | 5MHz |
| --- | --- | --- | --- | --- |
| Allocated subframes per Radio Frame | 10 | 10 | 10 | 10 |
| Allocated resource blocks | SF#5: 4,…99 SF#0,1,2,3,4,6,7,8,9: 0,…99 | SF#5:4,…74 SF#0,1,2,3,4,6,7,8,9: 0,…74 | SF#5:4,…49 SF#0,1,2,3,4,6,7,8,9: 0,…49 | SF#5:4,…24 SF#0,1,2,3,4,6,7,8,9: 0,…24 |
| Information Bit Payload SF#0 | 110136 | 84760 | 57336 | 28336 |
| Information Bit Payload SF#5 | 105528 | 81176 | 52752 | 24496 |
| Information Bit Payload SF#1,2,3,4,6,7,8,9 | 115040 | 93800 | 61664 | 30576 |

## SDR Requirements for 4-layers

The following parameters were used to evaluate maximum achievable throughput in static channel conditions with 4 layers:

- Antenna configuration: 4x4

- Tx EVM: 1.5%, 2%

- Rx EVM: 0

- # Subframes of PDSCH transmission: 8

- MCS: 26, 25

- Transmission Mode: TM3

- System BW: 20MHz, 10MHz, 15MHz, 5MHz

- CFI: 1

Figure 1: SDR performance with 4 layers

For all cases with 4 layers max throughput is achieved with MCS of 26. The performance degradation with 2% Tx EVM compared to 1.5% is ~3dB across all cases.

Observation #3: Max throughput is achieved across all cases for 4x4, 4 layers with MCS of 26

Observation #4: Performance degradation is ~3dB with 2% Tx EVM compared to 1.5% with 4 layers

Proposal #3: For 20, 15MHz, 10MHz, 5MHz 4 layers SDR requirement define test with MCS of 26

Based on the proposals we recommend the following reference channels for 1024QAM with 4 layers per CC in Table 2.

Table 2: Fixed reference channels for 1024QAM SDR tests with 4 layers

| BW | 20MHz | 15MHz | 10MHz | 5MHz |
| --- | --- | --- | --- | --- |
| Allocated subframes per Radio Frame | 10 | 10 | 10 | 10 |
| Allocated resource blocks | SF#5: 4,…99 SF#0,1,2,3,4,6,7,8,9: 0,…99 | SF#5:4,…74 SF#0,1,2,3,4,6,7,8,9: 0,…74 | SF#5:4,…49 SF#0,1,2,3,4,6,7,8,9: 0,…49 | SF#5:4,…24 SF#0,1,2,3,4,6,7,8,9: 0,…24 |
| Information Bit Payload SF#0 | 230105 | 169544 | 110136 | 57336 |
| Information Bit Payload SF#5 | 220296 | 161760 | 115040 | 52752 |
| Information Bit Payload SF#1,2,3,4,6,7,8,9 | 251640 | 187712 | 125808 | 61664 |

- Conclusion

In this paper we have presented simulation results to derive SDR requirements for 1024QAM. Our observations and proposals are summarized as follows:

Observation #1: Tx EVM of 2% has > 3dB performance degradation compared to 1.5% with 2 layers in 20MHz

Observation #2: For 20MHz with 2 layers, max throughput is not achieved with MCS of 26

Observation #3: Max throughput is achieved across all cases for 4x4, 4 layers with MCS of 26

Observation #4: Performance degradation is ~3dB with 2% Tx EVM compared to 1.5% with 4 layers

Proposal #1: For 20MHz 2 layers SDR requirement define test with MCS of 25

Proposal #2: For 15MHz, 10MHz, 5MHz 2 layers SDR requirement define test with MCS of 26

Proposed reference channel for 2 layer SDR test:

Table 1: Fixed reference channels for 1024QAM SDR tests with 2 layers

| BW | 20MHz | 15MHz | 10MHz | 5MHz |
| --- | --- | --- | --- | --- |
| Allocated subframes per Radio Frame | 10 | 10 | 10 | 10 |
| Allocated resource blocks | SF#5: 4,…99 SF#0,1,2,3,4,6,7,8,9: 0,…99 | SF#5:4,…74 SF#0,1,2,3,4,6,7,8,9: 0,…74 | SF#5:4,…49 SF#0,1,2,3,4,6,7,8,9: 0,…49 | SF#5:4,…24 SF#0,1,2,3,4,6,7,8,9: 0,…24 |
| Information Bit Payload SF#0 | 110136 | 84760 | 57336 | 28336 |
| Information Bit Payload SF#5 | 105528 | 81176 | 52752 | 24496 |
| Information Bit Payload SF#1,2,3,4,6,7,8,9 | 115040 | 93800 | 61664 | 30576 |

Proposal #3: For 20, 15MHz, 10MHz, 5MHz 4 layers SDR requirement define test with MCS of 26

Proposed reference channel for 4 layer SDR test:

Table 2: Fixed reference channels for 1024QAM SDR tests with 4 layers

| BW | 20MHz | 15MHz | 10MHz | 5MHz |
| --- | --- | --- | --- | --- |
| Allocated subframes per Radio Frame | 10 | 10 | 10 | 10 |
| Allocated resource blocks | SF#5: 4,…99 SF#0,1,2,3,4,6,7,8,9: 0,…99 | SF#5:4,…74 SF#0,1,2,3,4,6,7,8,9: 0,…74 | SF#5:4,…49 SF#0,1,2,3,4,6,7,8,9: 0,…49 | SF#5:4,…24 SF#0,1,2,3,4,6,7,8,9: 0,…24 |
| Information Bit Payload SF#0 | 230105 | 169544 | 110136 | 57336 |
| Information Bit Payload SF#5 | 220296 | 161760 | 115040 | 52752 |
| Information Bit Payload SF#1,2,3,4,6,7,8,9 | 251640 | 187712 | 125808 | 61664 |

- References

- R4-1805959, “Way forward on demodulation and CSI requirements for high capacity stationary wireless link and 1024QAM”, Huawei, HiSilicon, Anritsu, Rohde & Schwarz