3GPP TSG-RAN4 Meeting #87 	R4-1806333

# Busan, Korea, 21 – 25 May, 2018

Agenda item:	7.10.3.2

Source:	Intel Corporation

Title:	Further discussion on UE measurement mode with gap for SA

Document for:	Discussion

# Introduction

In last RAN4 meeting, LS (R4-1804202)[1] was approved, in which we agreed the assumptions for UE measurement mode discussion for SA mode

In order to determine the UE measurement mode with measurement gap for SA mode, RAN4 would assume that, In NR standalone mode, (1) NR serving cell can configure both an FR1 gap pattern and an FR2 gap pattern to be active at the same time. (2) If NR serving cell configures measurement gap for a UE, NR serving cell can also indicate if configured measurement gap applies to FR1/FR2 serving cell(s) (i.e. per UE gap) or applies to FR1 serving cell(s) (i.e. per FR gap to FR1) or applies to FR2 serving cell(s) (i.e. per FR gap to FR2). RAN4 will continue work with the above assumptions and suggest RAN2 to design the corresponding signalings according to the above assumptions, unless RAN2 has any issue from the signaling perspective.

And also the open issues were summarized in the approved WF[2].  In this contribution, the UE measurement mode with gap for SA are further analysed.

# Analysis on UE measurement mode with gap for SA

In this section we analyse all the possible UE measurement behaviour with gap for SA. Since RAN2 has no conclusion on the gap configuration flow for NR+NR DC case, we cannot determine whether MgNB and SgNB will configure gap separately or only MgNB can configure the measurement gap. In that sense, the cases we analysed in this contribution concentrate on the following scenarios, agreed in [2]:

Scenario 1: Serving cell is FR1 cell only, or Serving cells are NR CA in FR1

Scenario 2: Serving cell is FR2 cell only, or Serving cells are NR CA in FR2

Scenario 3: Serving cells are FR1+ FR2 CA

In SA, UE can also indicate to support per-UE gap only or support both per-UE and per-FR gap; and we think it is still useful to let network explicitly indicate the measurement gap type to UE associated with the gap configuration, as what we assumed in the LS[1].

## If UE support per-UE gap only

Based on the last meeting agreement [2], for UE who supports per-UE gap only, only per-UE gap can be configured to this UE.

## If UE support per-FR gap

Based on the above listed scenarios, we summarize all the UE measurement mode in the following tables.

#### Scenario 1: Serving cell is FR1 cell only, or Serving cells are NR CA in FR1

Table 1. UE measurement mode with gap in scenario 1

|  | Measurement object involves LTE, FR1 only | Measurement object involves LTE, FR1 and FR2 | Measurement object involves FR2 only |
| --- | --- | --- | --- |
| Serving cell configure per-UE gap | From scheduling opportunity and gap applicability perspective, - it’s per-UE gap From measurement requirement perspective, - UE applies per-UE gap for measurement | From scheduling opportunity and gap applicability perspective, - it’s per-UE gap From measurement requirement perspective, - UE applies per-UE gap for measurement | From scheduling opportunity and gap applicability perspective, - it’s per-UE gap From measurement requirement perspective, - UE applies per-UE gap for measurement |
| Serving cell configure per-FR gap for LTE/FR1 only | From scheduling opportunity and gap applicability perspective, - it’s per-FR gap to FR1 serving cell(s) From measurement requirement perspective, - UE applies per-FR gap for LTE/FR1 measurement | From scheduling opportunity and gap applicability perspective, - it’s per-FR gap to FR1 serving cell(s) From measurement requirement perspective, - UE applies per-FR gap for LTE/FR1 measurement - UE fulfils the per-FR requirements for FR2 measurement objects based on effective MGRP=20ms | From scheduling opportunity and gap applicability perspective, - Invalid gap configuration (because no FR1 MO was configured) From measurement requirement perspective, - UE fulfils the per-FR requirements for FR2 measurement objects based on effective MGRP=20ms |
| Serving cell configure per-FR gap for FR2 only | Invalid gap configuration (no FR2 serving cell) | Invalid gap configuration (no FR2 serving cell) | Invalid gap configuration (no FR2 serving cell) |
| Serving cell configure per-FR gaps for LTE/FR1 and FR2 | From scheduling opportunity and gap applicability perspective, - per-FR gap for FR1 applies to FR1 serving cell(s) - Invalid gap configuration for per-FR gap for FR2 (no FR2 serving cell and FR2 MO) From measurement requirement perspective, - UE applies per-FR gap for LTE/FR1 measurement | From scheduling opportunity and gap applicability perspective, - per-FR gap for FR1 applies to FR1 serving cell(s) - Invalid gap configuration for per-FR gap for FR2 (no FR2 serving cell) From measurement requirement perspective, - UE applies per-FR gap for LTE/FR1 measurement - Invalid gap configuration for per-FR gap for FR2, and UE behavior is unpredictable on FR2 | Invalid configuration (no FR2 serving cell and no FR1 MO) |

From the above table 1, if serving cell configure per-FR gap for LTE/FR1 only but MO involve LTE/FR1 and FR2, then UE may not need gap to conduct FR2 measurement; and the measurement requirement can be designed based on a fixed MGRP=20ms, which is similar as the methodology we used for EN-DC case.

Since serving cell(s) is in FR1 only for scenario 1, configuration of FR2 gap can be deemed as invalid configuration on FR2, and for the invalid configuration cases, no RRM requirements will be defined for the UE behaviour.

#### Scenario 2: Serving cell is FR2 cell only, or Serving cells are NR CA in FR2

Table 2. UE measurement mode with gap in scenario 2

|  | Measurement object involves LTE, FR1 only | Measurement object involves LTE, FR1 and FR2 | Measurement object involves FR2 only |
| --- | --- | --- | --- |
| Serving cell configure per-UE gap | From scheduling opportunity and gap applicability perspective, - it’s per-UE gap From measurement requirement perspective, - UE applies per-UE gap for measurement | From scheduling opportunity and gap applicability perspective, - it’s per-UE gap From measurement requirement perspective, - UE applies per-UE gap for measurement | From scheduling opportunity and gap applicability perspective, - it’s per-UE gap From measurement requirement perspective, - UE applies per-UE gap for measurement |
| Serving cell configure per-FR gap for LTE/FR1 only | Invalid gap configuration (no FR1 serving cell) | Invalid gap configuration (no FR1 serving cell) | Invalid gap configuration (no FR1 serving cell) |
| Serving cell configure per-FR gap for FR2 only | From scheduling opportunity and gap applicability perspective, - Invalid gap configuration (because no FR2 MO was configured) From measurement requirement perspective, - UE fulfils the per-FR requirements for FR1 measurement objects based on effective MGRP=40ms | From scheduling opportunity and gap applicability perspective, - it’s per-FR gap to FR2 serving cell(s) From measurement requirement perspective, - UE applies per-FR gap for LTE/FR2 measurement - UE fulfils the per-FR requirements for FR1 measurement objects based on effective MGRP=40ms | From scheduling opportunity and gap applicability perspective, - it’s per-FR gap to FR2 serving cell(s) From measurement requirement perspective, - UE applies per-FR gap for FR2 measurement |
| Serving cell configure per-FR gaps for LTE/FR1 and FR2 | Invalid configuration (no FR1 serving cell and no FR2 MO) | From scheduling opportunity and gap applicability perspective, - per-FR gap for FR2 applies to FR2 serving cell(s) - Invalid gap configuration for per-FR gap for FR1 (no FR1 serving cell) From measurement requirement perspective, - UE applies per-FR gap for FR2 measurement - Invalid gap configuration for per-FR gap for FR1, and UE behavior is unpredictable on FR1 | From scheduling opportunity and gap applicability perspective, - per-FR gap for FR2 applies to FR2 serving cell(s) - Invalid gap configuration for per-FR gap for FR1 (no FR1 serving cell and FR1 MO) From measurement requirement perspective, - UE applies per-FR gap for FR2 measurement |

From the above table 2, if serving cell configure per-FR gap for FR2 only but MO involve LTE/FR1 and FR2, then UE may not need gap to conduct LTE/FR1 measurement; and the measurement requirement can be designed based on a fixed MGRP. In this case, since the MO is LTE/FR1, we propose to follow the LTE legacy MGRP as a baseline which is 40ms.

Since serving cell(s) is in FR2 only for scenario 2, configuration of FR1 gap can be deemed as invalid configuration on FR1, and for the invalid configuration cases, no RRM requirements will be defined for the UE behaviour.

#### Scenario 3: Serving cells are FR1+ FR2 CA

Table 3. UE measurement mode with gap in scenario 3

|  | Measurement object involves LTE, FR1 only | Measurement object involves LTE, FR1 and FR2 | Measurement object involves FR2 only |
| --- | --- | --- | --- |
| Serving cell configure per-UE gap | From scheduling opportunity and gap applicability perspective, - it’s per-UE gap From measurement requirement perspective, - UE applies per-UE gap for measurement | From scheduling opportunity and gap applicability perspective, - it’s per-UE gap From measurement requirement perspective, - UE applies per-UE gap for measurement | From scheduling opportunity and gap applicability perspective, - it’s per-UE gap From measurement requirement perspective, - UE applies per-UE gap for measurement |
| Serving cell configure per-FR gap for LTE/FR1 only | From scheduling opportunity and gap applicability perspective, - it’s per-FR gap to FR1 serving cell(s) From measurement requirement perspective, - UE applies per-FR gap for LTE/FR1 measurement | From scheduling opportunity and gap applicability perspective, - it’s per-FR gap to FR1 serving cell(s) From measurement requirement perspective, - UE applies per-FR gap for LTE/FR1 measurement - UE cannot perform measurement toward measurement objects in FR2 | Invalid gap configuration (FR1 gap cannot be used for FR2 MO) |
| Serving cell configure per-FR gap for FR2 only | Invalid gap configuration (FR2 gap cannot be used for LTE/FR1 MO) | From scheduling opportunity and gap applicability perspective, - it’s per-FR gap to FR2 serving cell(s) From measurement requirement perspective, - UE applies per-FR gap for FR2 measurement - UE cannot perform measurement toward measurement objects in LTE/FR1 | From scheduling opportunity and gap applicability perspective, - it’s per-FR gap to FR2 serving cell(s) From measurement requirement perspective, - UE applies per-FR gap for FR2 measurement |
| Serving cell configure per-FR gaps for LTE/FR1 and FR2 | From scheduling opportunity and gap applicability perspective, - per-FR gap for FR1 applies to FR1 serving cell(s) - Invalid gap configuration for per-FR gap for FR2 (because no FR2 MO was configured) From measurement requirement perspective, - UE applies per-FR gap for LTE/FR1 measurement - Invalid gap configuration for per-FR gap for FR2, and UE behavior is unpredictable on FR2 | From scheduling opportunity and gap applicability perspective, - per-FR gap for FR1 applies to FR1 serving cell(s) - per-FR gap for FR2 applies to FR2 serving cell(s) From measurement requirement perspective, - UE applies per-FR gap for LTE/FR1 measurement - UE applies FR2 gap for FR2 measurement; | From scheduling opportunity and gap applicability perspective, - per-FR gap for FR2 applies to FR2 serving cell(s) - Invalid gap configuration for per-FR gap for FR1 (because no FR1 MO was configured) From measurement requirement perspective, - UE applies per-FR gap for FR2 measurement - Invalid gap configuration for per-FR gap for FR1, and UE behavior is unpredictable on FR1 |

In the above table 3, for the invalid configuration cases, no RRM requirements will be defined for the UE behaviour.

# Conclusion

In this contribution, the UE measurement mode with gap for SA are analysed. The proposal is,

Proposal: the UE measurement modes with gap shall follow the definitions in table 1, 2, and 3.

# References

[1] R4-1804202, “LS to RAN2 on measurement gap type indication for SA”, Intel, RAN4#86bis

[2] R4-1804201, “Wayforward on UE measurement gap for NR”, Intel, RAN4#86bis