3GPP TSG-RAN WG4 Meeting #87                	R4-1806422

Busan, Korea, 21 May – 25 May 2018

Title:	Simulation results for SPUCCH

Source:	Samsung

Agenda item:	6.22.6.2

Document for:	Discussion

# 1	Introduction

During RAN4#86b Melbourne meeting, WF about BS demodulation performance requirements was approved [1]. As for sPUCCH part, the simulation assumption as follows inTable1 [2]:

Table 1: Simulation assumption for sPUCCH

| Parameter | Value |
| --- | --- |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 2, 4 |
| Propagation condition | EPA5 |
| Cyclic prefix | Normal |
| Reference receiver | MRC |
| Channel bandwidth | [5 MHz], 10 MHz, 15 MHz, 20 MHz |
| TTI length | Subslot TTI |
| Number of OFDM-symbols | Subslot TTI: 2 and 3 |
| SPUCCH formats | Format 1a, format 4 |
| Number of bits per PRB for format4 | 3 bits per 2PRB |

For the test metric of sPUCCH, the follows are agreed in [1]:

- DTX to ACK probability that shall not exceed 1%

- ACK missed detection probability that shall not exceed 1% at the given SNR requirements for sPUCCH performance requirements

- How to formulate the above metric in the specification, follows the legacy way

In this contribution, based on above agreements, we provide our simulation results for sPUCCH format1a for alignment.

# 2	Simulation Results

In this section, simulation results for format1a are provided. For the test metric, we follow the legacy way defined in the 36.104. Both DTX to ACK probability and ACK missed detection probability are given as the performance requirement for format1a. The DTX to ACK probability is the probability that ACK is detected when nothing was sent, while ACK missed detection probability is the probability of not detecting an ACK when an ACK was sent. The results of ACK missed detection probability for all the channel bandwidth with 2OS and 3OS are shown in Table 2.

Table 2: Simulation results for SPUCCH format 1a

| Rx | BW(MHz) | ACK missed detection (<=1%) |  |
| --- | --- | --- | --- |
|  |  | 2OS | 3OS |
| 2 | 5 | 1.9 | 0.0 |
|  | 10 | 1.2 | -0.4 |
|  | 15 | 1.5 | -0.2 |
|  | 20 | 1.4 | -0.1 |
| 4 | 5 | -2.5 | -4.3 |
|  | 10 | -2.7 | -4.5 |
|  | 15 | -2.8 | -4.5 |
|  | 20 | -2.7 | -4.4 |

From this table, we can observe that with 2Rx, the ACK missed detection probability  has about  4dB worse than that of with 4Rx, at the target SNR of 1% ACK.. Compared with 3OS, there is about 1.5~1.9dB loss with 2OS for the ACK missed detection probability with evaluation channel bandwidth for both 2Rx and 4Rx antenna configuration.

# 3	Conclusion

In this contrition, the performance of sPUCCH with format1a was proposed for alignment.

# References

R4-1805494, WF on sTTI BS demodulation requirements, Ericsson, Huawei, Nokia

R4-1805495, Simulation assumption for BS demodulation requirements, Ericsson