3GPP TSG-RAN WG4 Meeting #87 	R4-1806475

Busan, Korea, 21st- 25th Mayl 2018

Agenda item:		7.10.4.1

Source:	MediaTek Inc.

Title:	Discussion on intra-frequency measurement requirement

Document for:		Discussion

# 1	Introduction

In [1], RAN 4 had agreed the basic framework of intra-frequency measurement requirements in scenarios 2a and 2b in FR1. The scope of agreed requirements is shown as follows:

Discuss the requirement considering

- Multiple NR serving cells

- Collision between SMTC occasions and MG

  - Scenarios to be first focused on

    - 2a and 2b when measurement is conducted only outside the MG.

      - The requirement for “2a and 2b when measurement can also be conducted within the MG” can be later extended, if RAN4 reaches consensus on whether to specify the corresponding requirements

    - The requirements of 3a can be simplified from that of 2a

  - Other scenarios to be discussed after RAN4 reaches consensus on whether to specify the corresponding requirements

    - 1a and 1b

    - 3b

  - Different DRX cycles

  - Misalignment between DRX on duration and SMTC window

The corresponding requirement tables of PSS/SSS detection, SBI acquisition, and measurement are also updated in [2]. In this paper, we will provide the complete requirement tables of intra measurement requirements without gaps.

# 2	Requirement of Intra-frequency measurement without gap

In this paper, based on the basic framework in our other paper [3], we provide the complete requirements tables of intra-frequency measurement without gap. In these tables, we use i to denote the target carrier index #i and define some parameters as follows:

- TSMTCi is the SMTC periodicity configured for target carrier #i

- TDRX is the DRX cycle length

- Kpi denotes the scaling factor for target carrier #i, considering the SMTC occasions are punctured by the measurement gap and/or shared with RLM

- KCarrieri denotes the per-carrier defined scaling factor for target carrier #i, considering that the SMTC occasions of multiple carriers are collided or not

As discussed in [3], the requirement should be addressed in the following 5 scenarios:

- Scenario A (1a/1b. Fully overlapped between MG and SMTC in Type A/B):

  - MGs are shared by Type A/B, Type C, and Type D.

- Scenario B (2a/2b. Partial overlapped between MG and SMTC in Type A/B):

  - Type A/B are conducted outside MG. But whether RLM can be performed together with Type A and B needs to be further considered for FR2. As a result, Scenario B can be further divided into

    - Scenario B1: The SMTC in Type A/B and RLM-RS are partially overlapped.

    - Scenario B2: The SMTC in Type A/B and RLM-RS are fully overlapped.

  - MGs are shared by Type C, and Type D.

- Scenario C (3a/3b. Fully non-overlapped between MG and SMTC in Type A/B):

  - Type A/B are conducted outside MG. But whether RLM can be performed together with Type A and B needs to be further considered for FR2. As a result, Scenario C can be further divided into

    - Scenario C1: The SMTC in Type A/B and RLM-RS are partially overlapped.

    - Scenario C2: The SMTC in Type A/B and RLM-RS are fully overlapped.

  - MGs are shared by Type C, and Type D.

Therefore, we specify the values of defined parameters in different scenarios as follows:

- Scenario A (1a/1b. Fully overlapped between MG and SMTC in Type A/B): Table 1-6 are used. Where

    - KCarrieri= ceil(∆ij=0GURP/MGRPfshare#j(Kintra)×ftarget#j(εi,jintra))

    - Kpi=1

- Scenario B (2a/2b. Partial overlapped between MG and SMTC in Type A/B):

  - Scenario B1: The SMTC in Type A/B and RLM-RS are partially overlapped. Table 7-12 are used. Where

    - KCarrieri= 1,    PCell/PSCell  NCollision i,  SCells

    - Kpi=1/(1-TSMTC#iMGRP)

  - Scenario B2: The SMTC in Type A/B and RLM-RS are fully overlapped. Table 7-12 are used. Where

    - KCarrieri= 1,    PCell/PSCell  NCollision i,  SCells

    - Kpi=1/(1-TSMTCiMGRP),   in FR11/(1-TSMTCiMGRP)×KRLM,   in FR2

- Scenario C (3a/3b. Fully non-overlapped between MG and SMTC in Type A/B):

  - Scenario C1: The SMTC in Type A/B and RLM-RS are partially overlapped. Table 7-12 are used. Where

    - KCarrieri= 1,    PCell/PSCell  NCollision i,  SCells

    - Kpi=1

  - Scenario C2: The SMTC in Type A/B and RLM-RS are fully overlapped. Table 7-12 are used. Where

    - KCarrieri= 1,    PCell/PSCell  NCollision i,  SCells

    - Kpi=1,   in FR1KRLM,   in FR2

The detail values of KCarrieri have been discussed in [3] and [4]. The value of Kpi in scenarios B1 was agreed in last meeting [1]. However, in other scenarios, the value of Kpi will depend on the ratio of SMTC occasions outside MG are shared with the RLM KRLM. Considering that this ratio has not yet be defined, we propose that

Proposal 1: The ratio of SMTC occasions outside MG are shared with the RLM is a fixed value and equal to 2.

# Table 1: Time period for PSS/SSS detection, (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync |
| --- | --- |
| No DRX | max[ 600ms, ceil( [5] x Kpi) x TSMTCix KCarrieri ]Note 1 |
| DRX cycle≤ 320ms | max[ 600ms, ceil(1.5x [5] x Kpi) x max(TSMTCi, TDRX) x KCarrieri] |
| DRX cycle>320ms | Ceil([5] x Kpi) x TDRXx KCarrieri |
| Note 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

# Table 2: Time period for PSS/SSS detection, (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync |
| --- | --- |
| No DRX | max[ 600ms, ceil( [5] x Kpi) x N1 x TSMTCi x KCarrieri]Note 1 |
| DRX cycle≤ 320ms | max[ 600ms, ceil(1.5 x [5] x Kpi) x N1 x max(TSMTCi, TDRX) x KCarrieri] |
| DRX cycle>320ms | Ceil( [5] x Kpi) x N1 x TDRX x KCarrieri |
| … | … |
| Note 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

# Table 3: Time period for time index detection (Frequency range FR1)

| DRX cycle | TSSB_time_index |
| --- | --- |
| No DRX | max[120ms, ceil( 3 x Kpi ) x TSMTCi x KCarrieri]Note 1 |
| DRX cycle≤ 320ms | max[120ms, ceil (1.5 x 3 x Kpi) x max(TSMTCi, TDRX) x KCarrieri ] |
| DRX cycle>320ms | Ceil(3 x Kpi) x TDRX x KCarrieri |
| Note 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

# Table 4: Time period for time index detection (Frequency range FR2)

| DRX cycle | TSSB_time_index |
| --- | --- |
| No DRX | max[ 200ms, ceil( [5] x Kpi) x N2 x TSMTCi x KCarrieri]Note 1 |
| DRX cycle≤ 320ms | max[ 200ms, ceil (1.5 x[5] x Kpi) x N2 x max(TSMTCi, TDRX) x KCarrieri] |
| DRX cycle>320ms | Ceil( [5] x Kpi) x N2 x TDRX x KCarrieri |
| … | … |
| Note 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

# Table 5: Measurement period for intra-frequency measurements without gaps(Frequency FR1)

| DRX cycle | T SSB_measurement_period |
| --- | --- |
| No DRX | max[ 200ms, ceil( 5 x Kpi) x TSMTCi x KCarrieri]Note 1 |
| DRX cycle≤ 320ms | max[ 200ms, ceil(1.5x 5 x Kpi) x max(TSMTCi, TDRX) x KCarrieri ] |
| DRX cycle>320ms | ceil( 5 x Kpi ) x TDRX x KCarrieri |
| Note 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

# Table 6: Measurement period for intra-frequency measurements without gaps(Frequency FR2)

| DRX cycle | T SSB_measurement_period |
| --- | --- |
| No DRX | max[ 400ms, ceil( 5 x Kpi) x N3 x TSMTCi x KCarrieri]Note 1 |
| DRX cycle≤ 320ms | max[ 400ms, ceil(1.5x 5 x Kpi) x N3 x max(TSMTCi, TDRX) x KCarrieri ] |
| DRX cycle>320ms | ceil(5 x Kpi ) x N3 x TDRX x KCarrieri |
| Note 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

# Table 7: Time period for PSS/SSS detection, (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync |
| --- | --- |
| No DRX | max[ 600ms, ceil( [5] x Kpi) x max(MGRP,TSMTCi)x KCarrieri ]Note 1 |
| DRX cycle≤ 320ms | max[ 600ms, ceil(1.5x [5] x Kpi) x max(MGRP, TSMTCi, TDRX) x KCarrieri] |
| DRX cycle>320ms | Ceil([5] x Kpi) x max(MGRP, TDRX) x KCarrieri |
| Note 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

# Table 8: Time period for PSS/SSS detection, (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync |
| --- | --- |
| No DRX | max[ 600ms, ceil( [5] x Kpi) x N1 x max(MGRP,TSMTCi)x KCarrieri]Note 1 |
| DRX cycle≤ 320ms | max[ 600ms, ceil(1.5 x [5] x Kpi) x N1 x max(MGRP,TSMTCi, TDRX) x KCarrieri] |
| DRX cycle>320ms | Ceil( [5] x Kpi) x N1 x max(MGRP, TDRX) x KCarrieri |
| … | … |
| Note 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

# Table 9: Time period for time index detection (Frequency range FR1)

| DRX cycle | TSSB_time_index |
| --- | --- |
| No DRX | max[120ms, ceil( 3 x Kpi ) x max(MGRP,TSMTCi) x KCarrieri]Note 1 |
| DRX cycle≤ 320ms | max[120ms, ceil (1.5 x 3 x Kpi) x max(MGRP, TSMTCi, TDRX) x KCarrieri ] |
| DRX cycle>320ms | Ceil(3 x Kpi) x max(MGRP, TDRX) x KCarrieri |
| Note 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

# Table 10: Time period for time index detection (Frequency range FR2)

| DRX cycle | TSSB_time_index |
| --- | --- |
| No DRX | max[ 200ms, ceil( [5] x Kpi) x N2 x max(MGRP,TSMTCi) x KCarrieri]Note 1 |
| DRX cycle≤ 320ms | max[ 200ms, ceil (1.5 x[5] x Kpi) x N2 x max(MGRP, TSMTCi, TDRX) x KCarrieri] |
| DRX cycle>320ms | Ceil( [5] x Kpi) x N2 x max(MGRP, TDRX) x KCarrieri |
| … | … |
| Note 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

# Table 11: Measurement period for intra-frequency measurements without gaps(Frequency FR1)

| DRX cycle | T SSB_measurement_period |
| --- | --- |
| No DRX | max[ 200ms, ceil( 5 x Kpi) x max(MGRP,TSMTCi) x KCarrieri]Note 1 |
| DRX cycle≤ 320ms | max[ 200ms, ceil(1.5x 5 x Kpi) x max(MGRP, TSMTCi, TDRX) x KCarrieri ] |
| DRX cycle>320ms | ceil( 5 x Kpi ) x max(MGRP, TDRX) x KCarrieri |
| Note 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

# Table 12: Measurement period for intra-frequency measurements without gaps(Frequency FR2)

| DRX cycle | T SSB_measurement_period |
| --- | --- |
| No DRX | max[ 400ms, ceil( 5 x Kpi) x N3 x max(MGRP,TSMTCi) x KCarrieri]Note 1 |
| DRX cycle≤ 320ms | max[ 400ms, ceil(1.5x 5 x Kpi) x N3 x max(MGRP, TSMTCi, TDRX) x KCarrieri ] |
| DRX cycle>320ms | ceil(5 x Kpi ) x N3 x max(MGRP, TDRX) x KCarrieri |
| Note 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

# 3	Summary

In this contribution, we propose

Proposal 1: The ratio of SMTC occasions outside MG are shared with the RLM is a fixed value and equal to 2.

# 4	Reference

[1] R4-1805521, Wayforward on requirement of intra-frequency measurement without gap, Mediatek, Apr. 2018.

[2] R4-1805978, Intra frequency measurement requirements without gaps corrections, Ericsson, Apr. 2018.

[3] R4-1806472, Discussion on collision between measurement gap and SMTC, Mediatek, May 2018.

[4] R4-1806474, Discussion on gap sharing between intra-frequency and inter-frequency measurements, Mediatek, May 2018.