3GPP TSG- RAN WG4 Meeting #87 	R4-1806590

# Busan, Korea, 21 – 25 May 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR 37.843: AAS OTA sensitivity conformance testing with One Dimensional Compact Range Chamber (10.3.2.4) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-05-14 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The details of Rel-13 AAS BS OTA sensitivity conformance testing with One Dimensional Compact Range Chamber have been recorded in TR 37.842, and they can be reused in TR 37.843 for Rel-15 eAAS BS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Record the details of Rel-15 eAAS BS OTA sensitivity conformance testing with One Dimensional Compact Range Chamber in TR 37.843, according to those in TR 37.842. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The details of Rel-15 eAAS BS OTA sensitivity conformance testing with One Dimensional Compact Range Chamber in TR 37.843 remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

#### 10.3.2.4	One Dimensional Compact Range Chamber

##### 10.3.2.4.1	General

The principle of the measurement set up for OTA sensitivity is shown in figure 10.3.2.4.1-1. It closely resembles the standard compact range chamber. But for the generation of the plane wave in the quiet zone the proposed method uses a special one dimensional probe instead of the standard set up comprising a concave mirror and a feed horn. The main advantage is a reduced chamber size and a simplified probe system replacing the standard mirror feed horn set up.

# Figure 10.3.2.1.3.1-1: One Dimensional Compact Range Chamber measurement system setup for EIS

The probe antenna is set up of a one dimensional array of antenna modules, connected by a special feed system. Optimized amplitude and phase settings in the feed system result in a plane wave in vertical direction. As no near field to far field transformation has to be carried out, it is not required to measure a whole set of azimuth and elevation angles.

##### 10.3.2.4.2	Calibration and Procedure

1)	Connect the feed system of the compact probe to the signal generator.

2)	Calibrate the test range, using a reference antenna with standard gain installed in the quiet zone of the probe and measure the path loss between signal generator and reference antenna.

3)	Position the DUT in the quiet zone of the probe with its manufacturer declared coordinate system reference point in the same place as the phase centre of the reference antenna. The manufacturer declared coordinate system orientation of the DUT is set to be aligned with testing system.

4)	Connect the DUT to the measurement equipment.

5)	Align with the conformance test direction (subclause 9.2 Dx.x).

6)	Set the signal generator to the manufacturer declared sensitivity (EIS) level plus the measured path loss.

7)	Measure the throughput and BER of the DUT.

8)	Repeat test steps 3 to 7 for all conformance test directions.

##### 10.3.2.4.3 	MU assessment

###### 10.3.2.4.3.1 	MU Budget

# Table 10.3.2.4.3.1-1: One Dimensional Compact Range Chamber uncertainty contributions

for EIS measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Misalignment DUT and pointing error | C3-1 |
| 2 | Standing wave between DUT and test range antenna | C3-2 |
| 3 | Quiet zone ripple DUT | C3-3 |
| 4 | Signal generator | C3-4, E |
| 5 | Phase curvature | C3-5 |
| 6 | Polarization mismatch between DUT and transmitting antenna | C3-6 |
| 7 | Mutual coupling between DUT and transmitting antenna | C3-7 |
| 8 | Measurement equipment | C3-8, E |
| 9 | Impedance mismatch in transmitting chain | C3-9 |
| 10 | RF leakage and dynamic range a) Feeder package terminated b) DUT terminated | C3-10 |
| Stage 1: Calibration measurement |  |  |
| 11 | Misalignment positioning system | C3-11 |
| 12 | Pointing error between reference antenna and test range antenna | C3-12 |
| 13 | Impedance mismatch in path to reference antenna | C3-13 |
| 14 | Impedance mismatch in path to compact probe | C3-14 |
| 15 | Standing wave between reference antenna and test range antenna | C3-2 |
| 16 | Quiet zone ripple reference antenna | C3-4 |
| 17 | Phase curvature | C3-5 |
| 18 | Polarization mismatch between reference antenna and transmitting antenna | C3-6 |
| 19 | Mutual coupling between reference antenna and transmitting antenna | C3-7 |
| 20 | Measurement equipment | C3-8, E |
| 21 | Influence of reference antenna feed cable (flexing cables, adapters, attenuators, connector repeatability) | C3-15 |
| 22 | Mismatch of transmitter chain | C3-16 |
| 23 | Insertion loss of transmitter chain | C3-17 |
| 24 | Uncertainty of absolute gain of reference antenna | C3-18, E |
| 25 | RF leakage (SGH connector terminated and test range antenna connector cable terminated) | C3-10 |

###### 10.3.2.4.3.2 	MU Value

Table 10.3.2.4.3.2-1: One Dimensional Compact Range Chamber uncertainty assessment for EIS measurement

| EIS uncertainty budget |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty Source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz < f ≦ 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz |  | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT and pointing error | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 |  |
| 2 | Standing wave between DUT and test range antenna | 0.18 | 0.18 | U-shaped | √2 | 1 | 0.13 | 0.13 |  |
| 3 | Quiet zone ripple DUT | 0.0325 | 0.0325 | Gaussian | 1 |  | 0.03 | 0.03 |  |
| 4 | Signal generator | 0.46 | 0.46 | Gaussian | 1 | 1 | 0.46 | 0.46 |  |
| 5 | Phase curvature | 0.01 | 0.01 | Gaussian | 1 | 1 | 0.01 | 0.01 |  |
| 6 | Polarization mismatch between DUT and transmitting antenna | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |  |
| 7 | Mutual coupling between DUT and transmitting antenna | 0 | 0 | Rectangular | √3 | 1 | 0 | 0 |  |
| 8 | Measurement equipment | 0.14 | 0.26 | Gaussian | 1 | 1 | 0.14 | 0.26 |  |
| 9 | Impedance mismatch in transmitter chain | 0.0056 | 0.01 | U-shaped | √2 | 1 | 0 | 0.01 |  |
| 10 | RF leakage and dynamic range a) feeder package terminated b) DUT terminated | 0 | 0 | Gaussian | 1 | 1 | 0 | 0 |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |
| 11 | Misalignment positioning system | 0 | 0 | Exp. Normal | 2 | 1 | 0 | 0 |  |
| 12 | Pointing error between reference antenna and test range antenna | 0 | 0 | Rectangular | √3 | 1 | 0 | 0 |  |
| 13 | Impedance mismatch in path to reference antenna | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |  |
| 14 | Impedance mismatch in path to compact probe | 0.03 | 0.03 | U-shaped | √2 | 1 | 0.02 | 0.02 |  |
| 15 | Standing wave between reference antenna and test range antenna | 0.15 | 0.15 | U-shaped | √2 | 1 | 0.11 | 0.11 |  |
| 16 | Quiet zone ripple reference antenna | 0.178 | 0.178 | Gaussian | 1 | 1 | 0.18 | 0.18 |  |
| 17 | Phase curvature | 0 | 0 | Gaussian | 1 | 1 | 0 | 0 |  |
| 18 | Polarization mismatch between reference antenna and transmitting antenna | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |  |
| 19 | Mutual coupling between reference antenna and transmitting antenna | 0 | 0 | Rectangular | √3 | 1 | 0 | 0 |  |
| 20 | Measurement equipment | 0.14 | 0.26 | Gaussian | 1 | 1 | 0.14 | 0.26 |  |
| 21 | Influence of reference antenna feed cable (flexing cables, adapters, attenuators, connector repeatability) | 0.082 | 0.082 | Rectangular | √3 | 1 | 0.05 | 0.05 |  |
| 22 | Mismatch of transmitter chain | 0.2 | 0.3 | U-shaped | √2 | 1 | 0.14 | 0.21 |  |
| 23 | Insertion loss of transmitter chain | 0.18 | 0.18 | Rectangular | √3 | 1 | 0.1 | 0.1 |  |
| 24 | Uncertainty of absolute gain of reference antenna | 0.5 | 0.43 | Rectangular | √3 | 1 | 0.29 | 0.25 |  |
| 25 | RF leakage (SGH connector terminated and test range antenna connector cable terminated) | 0 | 0 | Gaussian | 1 | 1 | 0 | 0 |  |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.66 | 0.73 |  |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1.29 | 1.43 |  |

<End of change>