3GPP TSG-RAN WG4 Meeting #87	R4-1806660

Busan, Korea, 21st – 25th May 2018

Source: 	Qualcomm

Title: 					A-MPR definition for band n41

Agenda Item: 			7.2.2.1.2

Document for:	Approval

# 1	Introduction

This contribution presents a definition for the A-MPR for band n41 as well as the frequency offset to avoid A-MPR. The proposal is based on simulation results and verified PA model and is a slight modification of [1].

# 2	Discussion

## 2.1	Definition of A-MPR regions

We analyse n41 to find the A-MPR regions by simulating the total back-off. Figure 1 depicts two examples of band n41 when the channel is at the lower band edge. In these triangles in Figure 1, we can identify 2 regions:

- IMD3 region at the lower left corner, caused by IMD3 of the allocation and its I/Q image as defined in [1].

- Outer/Inner regrowth region (Left edge and Inner) caused by spectral regrowth as defined in [1].

Regrowth characteristics can vary over PA model and operation mode. Taking this aspect into account, the parameters in Table 2 need to be modified. So, we apply and tweak the algorithm in [1] to ensure that the back-off requirement can be met. This means tweaking the frequency offsets, RB_START, and LCRB boundary conditions.

Figure 1. Examples of simulated Total Back-off (20 MHz and 100 MHz channels)

## 2.2	Frequency Offsets

Frequency Offsets in Table 1 are calculated for 2 regions, so when the frequency offset is exceeded, we simply apply the regular MPR, If the frequency offset is not exceeded, then we apply the back-off within the A-MPR region.

- IMD3 region. The frequency offset is simply calculated so to avoid IMD due to RSB falling into additional spurious region. We simplify from [1] and make independent of RB_START. and SCS. This simplifies to Foffset_IMD=[BW-6 MHz]. The reason for simplification is that more time is needed to see if SW can handle the complexity of reducing the frequency offset as a function of RB_START. For example, the impact is ~20RB’s in a 20MHz BW, and the impacted RB’s would scale with SCS and BW.

- Regrowth regions. Our regrowth regions have Foffset_Regrowth=[max(10MHz, 025*BW]  for CP-OFDM. Foffset_Regrowth=[max(10MHz, 0.60*BW] for DFT-s-OFDM. The reason for modifying these parameters is that the regrowth pattern can vary with PA model and PA operation mode. Further, the MPR is sufficient in CP-OFDM mode to handle a lower offset frequency than the DFT-s-OFDM counterpart.

We test the required back-off coverage for FC - BWChannel/2 < FUL_low + offset_regrowth to see if the required total back-off reduces to the required MPR. If the required total back off simply reduces to the required MPR to meet general requirements except NS_04 SEM, then we know that offset_regrowth is enough. Figure 3 shows the total back-off using a simple shift or offset_regrowth of 10MHz as proposed by the Nokia rule [1]. Clearly at 100MHz, the back-off for the additional spurious (NS_spur) is still shown. However, when we increase the offset to 25MHz for CP-OFDM and 60MHz for DFT-s-OFDM, the back-off from NS_spur is below MPR. Figure 3 shows total back-off for the 10MHz shift Vs the shifts required for the 2 waveform types at 100MHz. Table 4 summarizes the shifts required for all BW’s.

Figure 3. Using the modified frequency offset_regrowth parameter for CP-OFDM and DFT-s-OFDM. The original 10MHz shift is not enough

## 2.3	Modifying algorithm parameters

Not only the AMPR’ values need to be modified. Other parameters need to be modified to successfully reduce the amount of back-off required after applying the algorithm. We tested this by reducing the frequency offset to zero or channel edge is parked at 2496MHz. We had to make the following changes to account for different PA model behaviour and operation mode:

AWmax,IMD3		Modify from 4MHz/(12*SCS) to 8MHz/(2*SCS).

offsetmax,IMD3		Modify from [BWChannel – 6 MHz-3*12SCS*RBstart] to [BWChannel – 6 MHz] to reduce possible UE software complexities

start				Modify from [0.08 BWChannel] to [0.00 BWChannel]

AWmax,regrowth		Modify from 61MHz  to 100MHz because regrowth spreads wider for our PA model.

offsetregrowth		See section 2.2

RBstart ≤ LCRB/Slope + start / (12SCS)		Modify the slope from 2 to 1.25

Figure 4 shows example of simulated results showing the difference of using unmodified parameters to the modified parameters. The unmodified parameters still require back-off.

Figure 4. QPSK CP-OFDM. Top is unmodified parameters [1]. Bottom is modified parameters and applying a modified variant of the proposed rule as in Table 2.

## 2.4	Proposed modified rule for A-MPR and frequency offset

Use the same rule proposed in [1], but with some modifications. AMPR’ = Total Back off and AMPR = max (MPR, AMPR’).

Proposal: A-MPR = max (MPR, AMPR’)

Note that AMPR’ = 0dB means only MPR is applied
Given the parameters defined in Table 1 and symbol definitions in Table 2,

if RBstart ≤ fstart,max,IMD3 / (12SCS)
and LCRB ≤ AWmax,IMD3 / (12SCS)
and FC - BWChannel/2 < FUL_low + offsetIMD3,
then

the AMPR’ is defined according to Table 3,

else, if RBstart ≤ LCRB/1.25 + start / (12SCS)
and LCRB ≤  AWmax,regrowth / (12SCS)
and FC - BWChannel/2 < FUL_low + offsetregrowth,
then

the AMPR’ is defined according to Table 3,

else,

only AMPR’ = 0dB and apply MPR.

Table 2. Parameters for region edges and frequency offsets [1] with some exceptions

| Parameter | Symbol | Value |  |  | Related condition |
| --- | --- | --- | --- | --- | --- |
|  |  | OFDM | DFT-S-OFDM |  |  |
| Max allocation start in IMD3 region | fstart,max,IMD3 | 0.33 BWChannel |  |  | RBstart ≤ fstart,max,IMD3 / (12SCS) |
| Max allocation BW in IMD3 region | AWmax,IMD3 | 8 MHz |  |  | LCRB ≤ AWmax,IMD3 / (12SCS) |
| Max freq. offset for IMD3 region | offsetmax,IMD3 | [BWChannel – 6 MHz] |  |  |  |
| Freq. offset required to avoid A-MPR in IMD3 region | offsetIMD3 | offsetmax,IMD3 |  |  | FC - BWChannel/2 ≥ FUL_low + offsetIMD3 |
| Right edge of regrowth region | start | [0.00 BWChannel] | [0.00 BWChannel] |  | RBstart ≤ LCRB/1.25 + start / (12SCS) |
| Max allocation BW in regrowth region | AWmax,regrowth | 100 MHz |  |  | LCRB ≤ AWmax,regrowth / (12SCS) |
| Freq. offset required to avoid A-MPR in regrowth region | offsetregrowth | Max (10MHz, 0.25* BWChannel MHz] |  | Max (10MHz, 0.6* BWChannel MHz] | FC - BWChannel/2 ≥ FUL_low + offsetregrowth |

Table 2. Symbols adopted from 38.101 and 36.101

| Symbol | Definition |
| --- | --- |
| BWChannel | Channel bandwidth |
| FC | Centre frequency of channel |
| FUL_low | The lower edge frequency of the uplink operating band |
| SCS | Subcarrier spacing |
| RBstart | Index of lowest allocated resource block; RBstart=0 is the lowest RB in the channel. |
| LCRB | Number of contiguously allocated resource blocks |

Table 3. A-MPR’ values

| Access | Modulation | A-MPR' [dB] |
| --- | --- | --- |
| DFT-S-OFDM | pi/2-BPSK | 3.5 |
|  | QPSK | 4 |
|  | 16-QAM | 4 |
|  |  | 64-QAM |  | 4 |  |
|  |  | 256-QAM |  | 4.5 |  |
| CP-OFDM | QPSK | 5.5 |
|  | 16-QAM | 5.5 |
|  | 64-QAM | 5.5 |
|  | 256-QAM | 6.5 |

Table 4. Example Frequency Offset Values in Table Format

| BW, MHz | 10 | 15 | 20 | 25 | 40 | 50 | 60 | 80 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offsetIMD3, MHz | [4.5] | [9.5] | [14.5] | [19.5] | [34.5] | [44.5] | [54.5] | [74.5] | [94.5] |
| CP-OFDM offsetregrowth, MHz | [10] | [10] | [10] | [10] | [10] | [12.5] | [15] | [20] | [25] |
| DFT-s--OFDM offsetregrowth, MHz | [10] | [10] | [12] | [15] | [24] | [30] | [36] | [48] | [60] |

# 3. Conclusions

In this contribution, we proposed a formula-based A-MPR rule for band n41 that effectively avoids excess back-off while it still is relatively simple despite the overwhelmingly large number of possible parameter combinations.

Proposal: A-MPR = max (MPR, AMPR’)

Note that AMPR’ = 0dB means only MPR is applied
Given the parameters defined in Table 1 and symbol definitions in Table 2,

if RBstart ≤ fstart,max,IMD3 / (12SCS)
and LCRB ≤ AWmax,IMD3 / (12SCS)
and FC - BWChannel/2 < FUL_low + offsetIMD3,
then

the AMPR’ is defined according to Table 3,

else, if RBstart ≤ LCRB/1.25 + start / (12SCS)
and LCRB ≤ AWmax,regrowth / (12SCS)
and FC - BWChannel/2 < FUL_low + offsetregrowth,
then

the AMPR’ is defined according to Table 3,

else,

only AMPR’ = 0dB and apply MPR.

# References

- R4-1804025, A-MPR definition for band n41, Nokia, RAN4#86, Athens, March 2018