3GP P TSG-RAN4 Meeting #87	R4-1806693

# Busan, South Korea, 21 May - 25 May 2018

Source:		Skyworks Solutions Inc., Ericsson

Title:			MPR Measurements of Sub-PRB Feature for Cat-M2 Devices

Agenda item:		6.20.2 [LTE_eMTC4-Core]

Document for:	Approval

# Introduction

In RAN4 #86bis, a way forward [1] encourages companies to submit MPR and A-MPR results for CAT-M2 for sub-PRB allocation according to agreements in RAN 1.

In this paper, measurements performed on a power amplifier (PA) are used to estimate MPR using 2, 3 and 6 sub-carrier (SC) waveforms.

# Discussion

## Measurement Setup

### Measurement Conditions

For MPR measurements, the PA input waveforms are aligned with [1]  to meet the following RF transceiver impairments:

- IQ image rejection: -25 dBc,

- LO leakage: -25 dBc,

- Counter-IM3: not used.

Measurements are performed at a 782.5 MHz carrier frequency and focus on meeting SEM mask and ACLR.

The measured device is an LTE Cat0 IoT module which includes the PA, low pass filter and transmit/receive antenna switch. The output of the module is thus representative of the performance at the user equipment (UE) antenna connector. The PA has a fully programmable bias current which facilitates calibration of PA linearity. The PA is calibrated for PC3 using a fully allocated QPSK 5 MHz transmission bandwidth (BW) carrier so that ACLR compliance is met with 1 dB back-off.

## Tested Waveform

Measurements are performed with the following three sub-PRB waveforms according to RAN1#92bis agreements:

  - Sub-carrier length (Lcsc) 6 with SC-FDMA QPSK modulation,

  - Lcsc=3, with SC-FDMA QPSK modulation,

  - 3 subcarriers with SC-FDMA Pi/2 BPSK modulation:

    - The Pi/2 rotation is performed across SC-FDMA symbols,

    - Use only 2 adjacent subcarriers out of the three allocated subcarriers with DFT-spread of length 2,

    - The active sub-carriers are positioned at the lowest possible allocation within the block of three sub-carriers.

For each waveform, the active sub-carriers are positioned at the lowest possible offset within the lowest PRB of a 5 MHz cell bandwidth (BW). Due to lack of time, Lcsc=3 and 6 waveforms are generated by upconverting a single side-band modulated NB-IoT carrier and by adjusting sub-carrier position within the targeted PRB.

### MPR Results

Figure 1 shows the measured power spectral density (PSD) for CAT-M2 5MHz transmission bandwidth versus LTE general spectrum emission mask (SEM) for PC3. Left: Lcsc=2 at 0.5 dB MPR, centre: Lcsc=3 at 2 dB MPR, right: Lcsc=6 at 1 dB MPR. Black: measured PA output spectrum, Green: LTE general SEM.

Measurements show that for PC3 CAT-M2 devices at 5 MHz channel bandwidth:

- 0.5 dB MPR is needed for Lcsc=2,

- 2 dB MPR is needed for Lcsc=3,

- 1 dB MPR is needed for Lcsc=6.

Figure 1: LTE general SEM measurements

Proposal 1: 0.5, 2 and 1 dB MPR is needed for Cat-M2, 5 MHz devices for sub-PRB Lcsc=2, 3 and 6 allocations respectively.

# Conclusions

MPR is required for CAT-M2 devices using sub-PRB feature in 5 MHz bandwidth.

Proposal 1: 0.5, 2 and 1 dB MPR is needed for Cat-M2, 5 MHz devices for sub-PRB Lcsc=2, 3 and 6 allocations respectively.

# References

- R4-1805140, WF on UE RF for sub-PRB R15 feature, Ericsson, Skyworks Solutions, Inc., Qualcomm