3GPP TSG-RAN WG4 Meeting #87	R4-1806762

Busan, Korea (Republic of), 21 – 25 May 2018

Agenda Item:	6.22.7.1.1

Source:	Ericsson

Title:	TBS tables used for sTTI UE demodulation requirements

Document for:	Discussion

# 1	Introduction

This contribution clarifies the TBS selection for sTTI demodulation requirements according to TS36.213 7.1.7 [1] described as follows:

- for DCI format 7-1A/7-1B/7-1C/7-1D/7-1E/7-1F/7-1G, the derived transport block size (after TBS translation as described in sections 7.1.7.2.2, 7.1.7.2.4, 7.1.7.2.5 when the transport block is mapped to more than one spatial layer) is scaled by α (for slot-PDSCH, and for subslot-PDSCH), then rounded to the closest valid transport block size in - Table 7.1.7.2.1-1 when the transport block is mapped to one spatial layer, - Table 7.1.7.2.2-1 when the transport block is mapped to two spatial layers, - Table 7.1.7.2.4-1when the transport block is mapped to three spatial layers, - Table 7.1.7.2.5-1when the transport block is mapped to four spatial layers. If the scaled TBS is closest to two valid transport block sizes, it is rounded to the larger transport block size.

# 2	Discussion

## 2.1	Confirmation of TBS selection procedure for slot/subslot-PDSCH

First UE extracts the modulation and coding scheme information IMCS, given by 5bits, from DCI 7-1x. UE then gets the TBS index ITBS from, for example, Table 7.1.7.1-1 in TS36.213. UE derives the TBS based on ITBS and the allocated resource block size, NPRB, from Table 7.1.7.2.1-1 Transport block size table. For example, If IMCS=2 and NPRB=2, the corresponding ITBS is 2 and TBS=72bits.

Consider the slot-PDSCH transmission with one layer. According to the description in TS36.213 7.1.7, TBS is scaled by α=0.5, and TBSscaled= 72*0.5 = 36, and then rounded to the closest valid transport block size. According to the TBS table (Table 7.1.7.2.1-1 in TS36.213), the closet valid TBS is 32 or 40. If the scaled TBS is closest to two valid transport block sizes, it is rounded to the larger transport block size. Therefore the TBS for slot-PDSCH is set to 40.

## 2.2	TBS used for sTTI UE demodulation requirements

RAN4#86bis agreed with the simulation assumption for sTTI [2], and it uses QPSK or 16QAM with 50PRB. According the procedure above, the TBS for slot/subslot-PDSCH is given as Table 1. We also give the TBS table for 41PRB in Table 2 since it is used for subframe 0 in TM9.

Table 1	TBS table for slot/subslot-PDSCH for UE demodulation requirements (50PRB).

|  |  |  |  | Slot-PDSCH |  | Subslot-PDSCH |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IMCS | Qm | ITBS | TBS with 50PRB | TBSscaled | TBSslot | TBSscaled | TBSsubslot |
| 0 | 2 | 0 | 1384 | 692 | 696 | 230.7 | 224 |
| 1 | 2 | 1 | 1800 | 900 | 904 | 300 | 296 |
| 2 | 2 | 2 | 2216 | 1108 | 1096 | 369.3 | 376 |
| 3 | 2 | 3 | 2856 | 1428 | 1416 | 476 | 472 |
| 4 | 2 | 4 | 3624 | 1812 | 1800 | 604 | 600 |
| 5 | 2 | 5 | 4392 | 2196 | 2216 | 732 | 744 |
| 6 | 2 | 6 | 5160 | 2580 | 2600 | 860 | 872 |
| 7 | 2 | 7 | 6200 | 3100 | 3112 | 1033.3 | 1032 |
| 8 | 2 | 8 | 6968 | 3484 | 3496 | 1161.3 | 1160 |
| 9 | 2 | 9 | 7992 | 3996 | 4008 | 1332 | 1320 |
| 10 | 4 | 9 | 7992 | 3996 | 4008 | 1332 | 1320 |
| 11 | 4 | 10 | 8760 | 4380 | 4392 | 1460 | 1480 |
| 12 | 4 | 11 | 9912 | 4956 | 4968 | 1652 | 1672 |
| 13 | 4 | 12 | 11448 | 5724 | 5736 | 1908 | 1928 |
| 14 | 4 | 13 | 12960 | 6480 | 6456 | 2160 | 2152 |
| 15 | 4 | 14 | 14112 | 7056 | 6968 | 2352 | 2344 |
| 16 | 4 | 15 | 15264 | 7632 | 7736 | 2544 | 2536 |
| 17 | 6 | 15 | 15264 | 7632 | 7736 | 2544 | 2536 |
| 18 | 6 | 16 | 16416 | 8208 | 8248 | 2736 | 2728 |
| 19 | 6 | 17 | 18336 | 9168 | 9144 | 3056 | 3112 |
| 20 | 6 | 18 | 19848 | 9924 | 9912 | 3308 | 3368 |
| 21 | 6 | 19 | 21384 | 10692 | 10680 | 3564 | 3624 |
| 22 | 6 | 20 | 22920 | 11460 | 11448 | 3820 | 3880 |
| 23 | 6 | 21 | 25456 | 12728 | 12576 | 4242.7 | 4264 |
| 24 | 6 | 22 | 27376 | 13688 | 13536 | 4562.7 | 4584 |
| 25 | 6 | 23 | 28336 | 14168 | 14112 | 4722.7 | 4776 |
| 26 | 6 | 24 | 30576 | 15288 | 15264 | 5096 | 5160 |
| 27 | 6 | 25 | 31704 | 15852 | 15840 | 5284 | 5352 |
| 28 | 6 | 26 | 36696 | 18348 | 18336 | 6116 | 6200 |

Table 2	TBS table for slot/subslot-PDSCH for UE demodulation requirements (41PRB).

|  |  |  |  | Slot-PDSCH |  | Subslot-PDSCH |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IMCS | Qm | ITBS | TBS with 41PRB | TBSscaled | TBSslot | TBSscaled | TBSsubslot |
| 0 | 2 | 0 | 1128 | 564 | 568 | 188 | 176 |
| 1 | 2 | 1 | 1480 | 740 | 744 | 246.7 | 256 |
| 2 | 2 | 2 | 1800 | 900 | 904 | 300 | 296 |
| 3 | 2 | 3 | 2408 | 1204 | 1192 | 401.3 | 408 |
| 4 | 2 | 4 | 2984 | 1492 | 1480 | 497.3 | 504 |
| 5 | 2 | 5 | 3624 | 1812 | 1800 | 604 | 600 |
| 6 | 2 | 6 | 4264 | 2132 | 2152 | 710.7 | 712 |
| 7 | 2 | 7 | 4968 | 2484 | 2472 | 828 | 840 |
| 8 | 2 | 8 | 5736 | 2868 | 2856 | 956 | 968 |
| 9 | 2 | 9 | 6456 | 3228 | 3240 | 1076 | 1064 |
| 10 | 4 | 9 | 6456 | 3228 | 3240 | 1076 | 1064 |
| 11 | 4 | 10 | 7224 | 3612 | 3624 | 1204 | 1192 |
| 12 | 4 | 11 | 8248 | 4124 | 4136 | 1374.7 | 1384 |
| 13 | 4 | 12 | 9528 | 4764 | 4776 | 1588 | 1608 |
| 14 | 4 | 13 | 10680 | 5340 | 5352 | 1780 | 1800 |
| 15 | 4 | 14 | 11832 | 5916 | 5992 | 1972 | 1992 |
| 16 | 4 | 15 | 12576 | 6288 | 6200 | 2096 | 2088 |

# 3	Conclusion

Proposal: RAN4 confirms the method to derive TBS and the TBS tables used for slot/subslot-PDSCH demodulation requirements as shown in this paper.

# 4	References

TS36.213, “Physical layer procedures”.

R4-1805493, “Update of simulation assumption for sTTI UE demodulation and CSI requirements”, Ericsson.