3GPP TSG-RAN WG4 Meeting #87	R4-1806763

Busan, Korea (Republic of), 21 – 25 May 2018

Agenda Item:	6.20.7.1.1

Source:	Ericsson

Title:	Simulation results of PDSCH for sTTI UE demodulation requirements

Document for:	Discussion

# 1	Introduction

RAN4#86bis agreed with the simulation assumption for PDSCH demodulation for sTTI [1]. In this contribution we provide the ideal simulation results for alignment.

# 2	FRC used for slot/subslot-PDSCH demodulation

This section discusses the FRC used for slot/subslot-PDSCH demodulation. We first discuss the channel bits and then TBS to satisfy the target coding rate.

## 2.1	CRS-based transmission

### 2.1.1	Channel bits

Figure 1 illustrates the resource allocation for slot-PDSCH without SPDCCH transmission (left) and with SPDCCH transmission (right). Figure 2 illustrates the resource allocation for subslot-PDSCH without SPDCCH transmission (left) and with SPDCCH transmission (right).

Table 1 and Table 2 give the calculation of the number of OFDM symbol per 12 subcarriers for slot-PDSCH and subslot-PDSCH, respectively. Table 3 and Table 4 give the calculation of the total number of OFDM symbols allocated for PDSCH. Note 6PRB is used by PSS/SSS/PBCH transmission in SF#0.

Figure 1	Resource allocation for CRS-based slot-PDSCH with/without SPDCCH (4 CRS ports, 1 OFDM symbol for SPDCCH).

Figure 2	Resource allocation for CRS-based subslot-PDSCH with/without SPDCCH (4 CRS ports, 1 OFDM symbol for SPDCCH).

Table 1 Number of OFDM symbols per 12 subcarriers for slot-PDSCH (4 CRS ports, 1 OFDM symbol for SPDCCH).

|  | Without SPDCCH | With SPDCCH | 12 subcarriers with PSS/SSS/PBCH |
| --- | --- | --- | --- |
| Slot 0 | 12*5-4 = 56 | 56 | 12*5-4-24 = 32 |
| Slot 1 | 12*7-12 = 72 | 12*7-12-8 = 64 | 12*3-4 = 32 |

Table 2 Number of OFDM symbols per 12 subcarriers for subslot-PDSCH (4 CRS ports, 1 OFDM symbol for SPDCCH).

|  | Without SPDCCH | With SPDCCH | 12 subcarriers with PSS/SSS/PBCH |
| --- | --- | --- | --- |
| Subslot 1 | 12*3-4 = 32 | 12*3-12-4 = 20 | 32 |
| Subslot 2 | 12*2 = 24 | 12*2-12 = 12 | 0 |
| Subslot 3 | 12*2-8 = 16 | 12*2-12-4 = 8 | 0 |
| Subslot 4 | 12*2 = 24 | 12*2-12 = 12 | 0 |
| Subslot 5 | 12*3-4 = 32 | 12*3-12 = 24 | 32 |

Table 3 Number of OFDM symbols with 50PRB for slot-PDSCH (8PRB for SPDCCH).

|  | SF#0 (SS/PBCH occupies 6PRBs) | SF#1,2,3,4,6,7,8,9 |
| --- | --- | --- |
| Slot 0 | 56*(50-8-6) + 56*8 + 32*6 = 2656 | 56*(50-8) + 56*8 = 2800 |
| Slot 1 | 72*(50-8-6) + 64*8 + 32*6 = 3296 | 72*(50-8) + 64*8 = 3536 |

Table 4 Number of OFDM symbols with 50PRB for subslot-PDSCH (8PRB for SPDCCH).

|  | SF#0 (SS/PBCH occupies 6PRBs) | SF#1,2,3,4,6,7,8,9 |
| --- | --- | --- |
| Subslot 1 | 32*(50-8-6) + 20*8 + 32*6 = 1504 | 32*(50-8) + 20*8 = 1504 |
| Subslot 2 | 24*(50-8-6) + 12*8 = 960 | 24*(50-8) + 12*8 = 1104 |
| Subslot 3 | 16*(50-8-6) + 8*8 = 640 | 16*(50-8) + 8*8 = 736 |
| Subslot 4 | 24*(50-8-6) + 12*8 = 960 | 24*(50-8) + 12*8 = 1104 |
| Subslot 5 | 32*(50-8-6) + 24*8 + 32*6 = 1536 | 32*(50-8) + 24*8 = 1536 |

### 2.2.2	FRC tables

Table 5 and Table 6 show the FRC tables used for slot-PDSCH and subslot-PDSCH, respectively. The TBS for slot/subslot-PDSCH is derived according to [3], and suitable TBS is chosen according to TS36.101 A.3.1.

In RAN4#86bis, some company concerned the coding rate at retransmission. For slot-PDSCH, the maximum coding rate is achieved when TBS=6968 is re-transmitted in SF#0 slot#0 (channel bits=10264), and coding rate is 0.68. We think it is still acceptable.

For subslot-PDSCH, on the other hand, the maximum code rate becomes 1.0 when TBS=2536 is re-transmitted in SF#0 subslot#3 (channel bits=2560). According to the RAN1 specification, the code rate should be less than 0.93 and it is not allowed. On simple solution is to set DTX for subslot 3. In this case, the maximum code rate becomes 0.67 when TBS=2536 is re-transmitted in SF#0 Subslot#2/4 (channel bits=3840).

Table 5	FRC table for CRS-based slot-PDSCH (1/2 16QAM)

|  | Unit | Information bit payload per slot | Binary channel bits per slot |
| --- | --- | --- | --- |
| For Sub-Frame 1,2,3,4,6,7,8,9 | Bits |  |  |
| Slot index 0 | Bits | 5736 | 11200 |
| Slot index 1 | Bits | 6968 | 14114 |
| For Sub-Frame 5 |  | N/A |  |
| For Sub-Frame 0 |  |  |  |
| Slot index 0 | Bits | 4968 | 10264 |
| Slot index 1 | Bits | 6456 | 13184 |

Table 6	FRC table for CRS-based subslot-PDSCH (1/2 16QAM).

|  | Unit | Information bit payload per subslot | Binary channel bits per subslot |
| --- | --- | --- | --- |
| For Sub-Frame 1,2,3,4,6,7,8,9 |  |  |  |
| Subslot index 1 | Bits | 2536 | 6016 |
| Subslot index 2 | Bits | 2152 | 4416 |
| Subslot index 3 | Bits | 1480 => DTX | 2944 => DTX |
| Subslot index 4 | Bits | 2152 | 4416 |
| Subslot index 5 | Bits | 2536 | 6144 |
| For Sub-Frame 5 |  | N/A |  |
| For Sub-Frame 0 |  |  |  |
| Subslot index 1 | Bits | 2536 | 6016 |
| Subslot index 2 | Bits | 1928 | 3840 |
| Subslot index 3 | Bits | 1320 => DTX | 2560 => DTX |
| Subslot index 4 | Bits | 1928 | 3840 |
| Subslot index 5 | Bits | 2536 | 6144 |

## 2.2	DMRS-based transmission

### 2.2.1	Channel bits

Figure 3 illustrates the resource allocation for slot-PDSCH without SPDCCH transmission (left) and with SPDCCH transmission (right). Figure 4 illustrates the resource allocation for subslot-PDSCH without SPDCCH transmission (left) and with SPDCCH transmission (right). According to the simulation assumption [1], we assume DMRS is transmitted in sTTI#1, #3, and #5. DMRS mapping is based on baseline for sTTI#1 but based on v0 for sTTI#3 and sTTI#5 to avoid collision with CRS symbols.

Table 7 and Table 8 show the calculation of the number of OFDM symbol per 12 subcarriers for slot-PDSCH and subslot-PDSCH, respectively. Table 9 and Table 10 show the calculation of the total number of OFDM symbols allocated for PDSCH.

Figure 3	Resource allocation for DMRS-based slot-PDSCH with CSI-RS with/without SPDCCH.

Figure 4	Resource allocation for DMRS-based subslot-PDSCH with CSI-RS with/without SPDCCH.

Table 7 Number of OFDM symbols per 12 subcarriers for slot-PDSCH (2 CRS ports, 1 or 2 DMRS ports, 4 CRI-RS ports, 2 OFDM symbol for SPDCCH).

|  | Without SPDCCH Without NZP-CSI-RS | With SPDCCH Without NZP-CSI-RS | Without SPDCCH With NZP-CSI-RS | With SPDCCH With NZP-CSI-RS |
| --- | --- | --- | --- | --- |
| Slot 0 | 12*5-4-6 = 50 | 50 | 12*5-4-6-2 = 48 | 48 |
| Slot 1 | 12*7-8-6 = 70 | 12*7-24-4-6 = 50 | 70 | 50 |

Table 8 Number of OFDM symbols per 12 subcarriers for subslot-PDSCH (2 CRS ports, 1 or 2 DMRS ports, 4 CRI-RS ports, 2 OFDM symbol for SPDCCH).

|  | Without SPDCCH Without NZP-CSI-RS | With SPDCCH Without NZP-CSI-RS | Without SPDCCH With NZP-CSI-RS | With SPDCCH With NZP-CSI-RS |
| --- | --- | --- | --- | --- |
| Subslot 1 | 12*3-4-4 = 28 | 12*3-24-4 = 8 | 28 | 8 |
| Subslot 2 | 12*2 = 24 | 0 | 12*2-2 = 22 | 0 |
| Subslot 3 | 12*2-4-4 = 16 | 0 | 16 | 0 |
| Subslot 4 | 12*2 = 24 | 0 | 24 | 0 |
| Subslot 5 | 12*3-4-4 = 28 | 12*3-24 = 12 | 28 | 12 |

Table 9 Number of OFDM symbols with 50PRB for slot-PDSCH (8PRB for SPDCCH).

|  | SF#0 (41PRB) | SF#1,3,4,6,8,9 (non NZP-CSI-RS) | SF#2,7 (NZP-CSI-RS) |
| --- | --- | --- | --- |
| Slot 0 | 50*(41-8)+50*8 = 2050 | 50*(50-8)+50*8 = 2500 | 48*(50-8)+48*8 = 2400 |
| Slot 1 | 70*(41-8)+50*8 = 2710 | 70*(50-8)+50*8 = 3340 | 70*(50-8)+50*8 = 3340 |

Table 10 Number of OFDM symbols with 50PRB for subslot-PDSCH (8PRB for SPDCCH).

|  | SF#0 (41PRB) | SF#1,3,4,6,8,9 (non NZP-CSI-RS) | SF#2,7 (NZP-CSI-RS) |
| --- | --- | --- | --- |
| Subslot 1 | 28*(41-8)+8*8 = 988 | 28*(50-8)+8*8 = 1240 | 28*(50-8)+8*8 = 1240 |
| Subslot 2 | 24*(41-8)+0 = 792 | 24*(50-8)+0 = 1008 | 22*(50-8)+0 = 924 |
| Subslot 3 | 16*(41-8)+0 = 528 | 16*(50-8)+0 = 672 | 16*(50-8)+0 = 672 |
| Subslot 4 | 24*(41-8)+0 = 792 | 24*(50-8)+0 = 1008 | 24*(50-8)+0 = 1008 |
| Subslot 5 | 28*(41-8)+12*8 = 1020 | 28*(50-8)+12*8 = 1272 | 28*(50-8)+12*8 = 1272 |

### 2.2.2	FRC tables

Table 11 and Table 12 show the FRC tables used for slot-PDSCH and subslot-PDSCH, respectively. The TBS for slot/subslot-PDSCH is derived according to [3], and suitable TBS is chosen according to TS36.101 A.3.1.

For slot-PDSCH, the maximum coding rate at retransmission becomes 0.55 when TBS=2216 is re-transmitted in SF#0 slot#0 (channel bits=4100). We think this is acceptable.

For subslot-PDSCH, the maximum coding rate becomes 0.73 when TBS=744 is transmitted in SF#0 subslot#3 (Channel bits=1056). Although the coding rate does not exceed 0.93, it may be high considering the target coding rate of 0.33 for the TM9 test. As same as CRS-based transmission, one possibility is to avoid the transmission in subslot#3 and Table 13 shows the FRC table when subslot#3 is DRXed. Since DMRS is transmitted only in subslot#1, #3, and #5, UE cannot receive subslot-PDSCH in subslot#3 if subslot#3 is DTXed. Therefore in this table DMRS is transmitted in subslot#1 and #4. With this change, the maximum coding rate becomes 0.58 when TBS=744 is transmitted in SF#0 subslot#4 (channel bits=1320).

Table 11	FRC table for DMRS-based slot-PDSCH (1/3 QPSK).

|  | Unit | Information bit payload per slot | Binary channel bits per slot |
| --- | --- | --- | --- |
| For Sub-Frames 1,3,4,6,8,9 |  |  |  |
| Slot index 0 | Bits | 1800 | 5000 |
| Slot index 1 | Bits | 2216 | 6680 |
| For Sub-Frames 2,7 (2 NZP-CSI-RS ports) |  |  |  |
| Slot index 0 | Bits | 1416 | 4800 |
| Slot index 1 | Bits | 2216 | 6680 |
| For Sub-Frame 5 |  | N/A |  |
| For Sub-Frames 0 (RB0–RB20 and RB30–RB49 => 41RBs) |  |  |  |
| Slot index 0 | Bits | 1480 | 4100 |
| Slot index 1 | Bits | 1800 | 5420 |

Table 12	FRC table for DMRS-based subslot-PDSCH (1/3 QPSK).

|  | Unit | Information bit payload per subslots | Binary channel bits per subslots |
| --- | --- | --- | --- |
| For Sub-Frames 1,3,4,6,8,9 |  |  |  |
| Subslot index 1 (With DMRS) | Bits | 744 | 2480 |
| Subslot index 2 | bits | 600 | 2016 |
| Subslot index 3 (With DMRS) | Bits | 376 | 1344 |
| Subslot index 4 | Bits | 600 | 2016 |
| Subslot index 5 (With DMRS) | Bits | 872 | 2544 |
| For Sub-Frames 2,7 (2 NZP-CSI-RS ports) |  |  |  |
| Subslot index 1 (With DMRS) | Bits | 744 | 2480 |
| Subslot index 2 | bits | 600 | 1848 |
| Subslot index 3 (With DMRS) | Bits | 376 | 1344 |
| Subslot index 4 | Bits | 600 | 2016 |
| Subslot index 5 (With DMRS) | Bits | 872 | 2544 |
| For Sub-Frame 5 |  |  |  |
| For Sub-Frames 0 (41PRB) |  |  |  |
| Subslot index 1 (With DMRS) | Bits | 600 | 1976 |
| Subslot index 2 | bits | 504 | 1584 |
| Subslot index 3 (With DMRS) | Bits | 296 | 1056 |
| Subslot index 4 | Bits | 504 | 1584 |
| Subslot index 5 (With DMRS) | Bits | 600 | 2040 |

Table 13	FRC table for DMRS-based subslot-PDSCH (1/3 QPSK) with subslot#3 DTXed.

|  | Unit | Information bit payload per subslots | Binary channel bits per subslots |
| --- | --- | --- | --- |
| For Sub-Frames 1,3,4,6,8,9 |  |  |  |
| Subslot index 1 (With DMRS) | Bits | 744 | 2480 |
| Subslot index 2 | bits | 600 | 2016 |
| Subslot index 3 (Without DMRS) | Bits | N/A | DTX |
| Subslot index 4 (With DMRS) | Bits | 472 | 1680 |
| Subslot index 5 | Bits | 872 | 2880 |
| For Sub-Frames 2,7 (2 NZP-CSI-RS ports) |  |  |  |
| Subslot index 1 (With DMRS) | Bits | 744 | 2480 |
| Subslot index 2 | bits | 600 | 1848 |
| Subslot index 3 (Without DMRS) | Bits | N/A | DTX |
| Subslot index 4 (With DMRS) | Bits | 472 | 1680 |
| Subslot index 5 | Bits | 872 | 2880 |
| For Sub-Frame 5 |  |  |  |
| For Sub-Frames 0 (41PRB) |  |  |  |
| Subslot index 1 (With DMRS) | Bits | 600 | 1976 |
| Subslot index 2 | bits | 504 | 1584 |
| Subslot index 3 (Without DMRS) | Bits | N/A | DTX |
| Subslot index 4 (With DMRS) | Bits | 408 | 1320 |
| Subslot index 5 | Bits | 712 | 2304 |

Proposal 1: For Subslot-PDSCH, subslot#3 should be DTX’ed to avoid very high coding rate at retransmission.

# 3	Simulation results

## 3.1	TM3

Figure 5 and Figure 6 show the simulation results of slot-PDSCH and subslot-PDSCH respectively. Figure 6 also compares the case subslot#3 is transmitted or DTX’ed.

Comparing between EPA5 and EVA30, there is no significant performance difference with regard to 70% of the maximum throughput. Since RAN4 has already agreed to set EPA5 for TM9, we propose to test with higher Doppler frequency, i.e., EVA30 for TM3 PDSCH demodulation requirements from the test coverage purpose.

For the comparison between subslot#3 transmission and DTXed in subslot-PDSCH, it is observed that the performance with subslot#3 DTX’ed is slight better than the case of subslot#3 transmission with regard to 70% of the maximum throughput. We think it is because the subslots with high coding rate are DTX’ed.

Proposal 2: RAN4 sets EVA30 for TM3 PDSCH demodulation requirements with sTTI.

Figure 5	Simulation results of slot-PDSCH TM3 dual layer.

Figure 6	Simulation results of subslot-PDSCH TM3 dual layer.

## 3.2	TM9

Figure 7 and Figure 8 show the simulation results for slot-PDSCH and subslot-PDSCH, respectively. Figure 8 also compares the scenario subslot#3 is transmitted or DTX’ed.

Our preference is to choose single layer. The reason is we think the purpose of this demodulation requirement is to verify DMRS-based PDSCH demodulation under slot/subslot-based processing. For this purpose, single layer is sufficient and dual layer can be verified with the existing non-STTI TM9 test cases.

For the comparison between subslot#3 transmission and DTXed in subslot-PDSCH, it is observed that the performance with subslot#3 DTX’ed is slight better than the case of subslot#3 transmission with regard to 70% of the maximum throughput.

Proposal 3: RAN4 specifies TM9 single layer PDSCH demodulation requirements for sTTI.

Figure 7	Simulation results of slot-PDSCH TM9.

Figure 8	Simulation results of subslot-DPSCH TM9.

# 4	Conclusion

Proposal 1: For Subslot-PDSCH, subslot#3 should be DTX’ed to avoid very high coding rate at retransmission.

Proposal 2: RAN4 sets EVA30 for TM3 PDSCH demodulation requirements with sTTI.

Proposal 3: RAN4 specifies TM9 single layer PDSCH demodulation requirements for sTTI.

# References

- R4-1805493, “Update of simulation assumption for sTTI UE demodulation and CSI requirements”, Ericsson.

- R4-1805506, “Way forward on sTTI DMRS pattern and CQI2MCS mapping table”, Huawei, Ericsson, Qualcomm.

- R4-1806762, “TBS tables used for sTTI UE demodulation requirements”, Ericsson.