3GPP TSG-RAN WG4 Meeting #87	R4-1806882

## Busan, Korea, 21st May  – 25th May, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.113 | CR | 0082 | rev | 0 | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 37.113 (MSR EMC): NR EMC Core Requrements updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-05-14 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In this CR, introduction of the NR (Core aspects) into the MSR EMC specification is proposed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added some symbols for NR BS. - Deleted some abbreviations. - Added exclusion bands for NR BS. - Updated radiated emission for NR BS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not aligned with NR BS EMC specifications TS 38.113. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 3.3, 4.4, 8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

--------------Start of text proposal-------------

3.2	Symbols

For the purposes of the present document, the following symbols apply:

BWChannel	Channel bandwidth

foffset	Frequency offset used for discovering narrowband response for receivers

ΔfOBUE 	Maximum offset of the operating band unwanted emissions mask from the operating band edge

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AC	Alternating Current

AMN	Artificial Mains Network

BC	Band Category

BER	Bit Error Ratio

BLER	Block Error Ratio

CDN	Coupling/Decoupling Network

CS	Capability Set

DC	Direct Current

E-UTRA	Evolved Universal Terrestrial Radio Access

EMC	Electromagnetic Compatibility

EPC	Evolved Packet Core

ESD	Electrostatic discharge

EUT	Equipment Under Test

FC,high		Center frequency of the highest transmitted/received carrier.

FC,low		Center frequency of the lowest transmitted/received carrier.

Foffset, RAT	Frequency offset from FC,high to the upper  RF bandwidth edge or FC,low to the lower RF bandwidth edge for a specific RAT.

FRC	Fixed Reference Channel

MB-MSR	Multi-Band Multi-Standard Radio

MSR	Multi-Standard Radio

NB-IoT	Narrowband – Internet of Things

NR	New Radio

NTC	Test Configuration for Non-contiguous operation

RAT	Radio Access Technology

RF	Radio frequency

rms	root mean square

TC	Test Configuration

UTRA	Universal Terrestrial Radio Access

*********************End of change*****************

*********************Next changed section*****************

4.4	Exclusion bands

An exclusion band is a band of frequencies over which no tests of radiated immunity are made.

4.4.1	Transmitter exclusion band

For testing of radiated immunity there shall be no transmitter exclusion band.

4.4.2	Receiver exclusion band

The receiver exclusion band for base stations extends from the lower frequency of the Base Station receive band minus 20 MHz to the upper frequency of the Base Station receive band plus 20 MHz for BS in an operating band less than 200 MHz wide, or from the lower frequency of the receive band minus 60 MHz to the upper frequency of the receive band plus 60 MHz for BS in an operating band more than or equal to 200 MHz wide. The exclusion bands for the MSR, NR, E-UTRA, UTRA and GSM/EDGE paired and unpaired operating bands are as set out in Tables 4.4-1 , Table 4.4-2  and Table 4.4-3 respectively.

For BS capable of multi-band operation, the total receiver exclusion band shall be the combination of the exclusion bands for each operating band supported by the BS.

Table 4.4-1: Receiver exclusion band for base stations (paired bands).

| MSR and E-UTRA Band number | UTRA Band number | GSM/EDGE Band designation |  | Receiver exclusion band |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | I | - |  | 1900 MHz |  | – | 2000 MHz |  |  |
| 2 | II | PCS 1900 |  | 1830 MHz |  | – | 1930 MHz |  |  |
| 3 | III | DCS 1800 |  | 1690 MHz |  | – | 1805 MHz |  |  |
| 4 | IV | - |  | 1690 MHz |  | – | 1775 MHz |  |  |
| 5 | V | GSM 850 |  | 804 MHz |  | – | 869 MHz |  |  |
| 6(1) | VI | - |  | 810 MHz |  | – | 860 MHz |  |  |
| 7 | VII | - |  | 2480 MHz |  | – | 2590 MHz |  |  |
| 8 | VIII | E-GSM |  | 860 MHz |  | – | 935 MHz |  |  |
| 9 | IX | - |  | 1729.9 MHz |  | – | 1804.9 MHz |  |  |
| 10 | X | - |  | 1690 MHz |  | – | 1790 MHz |  |  |
| 11 | XI | - |  | 1407.9 MHz |  | – | 1467.9 MHz |  |  |
| 12 | XII | - |  | 679 MHz |  | – | 736 MHz |  |  |
| 13 | XIII | - |  | 757 MHz |  | – | 807 MHz |  |  |
| 14 | XIV | - |  | 768 MHz |  | – | 818 MHz |  |  |
| 15 | XV | - |  | Reserved |  |  |  |  |  |
| 16 | XVI | - |  | Reserved |  |  |  |  |  |
| 17 | - | - |  | 684 MHz |  | – | 736 MHz |  |  |
| 18 | - | - |  | 795 MHz |  | – | 850 MHz |  |  |
| 19 | XIX | - |  | 810 MHz |  | – | 865 MHz |  |  |
| 20 | XX | - |  | 812 MHz |  | – | 882 MHz |  |  |
| 21 | XXI | - |  | 1427.9 MHz |  | – | 1482.9 MHz |  |  |
| 22 | XXII | - |  | 3390 MHz |  | – | 3510 MHz |  |  |
| 23 | - | - |  | 1980 MHz |  | – | 2040 MHz |  |  |
| 24 | - | - | 1606.5 MHz |  | – |  |  | 1680.5 MHz |
| 25 | XXV | - | 1830 MHz |  | – |  |  | 1935 MHz |
| 26 | XXVI | - |  | 794 MHz |  | – | 869 MHz |  |  |
| 27 | - | - |  | 787 MHz |  | – | 844 MHz |  |  |
| 28 | - | - |  | 683 MHz |  | – | 768 MHz |  |  |
| 29 | - | - |  | N/A |  |  |  |  |  |
| 30 | - |  |  | 2285 MHz |  | – | 2335 MHz |  |  |
| 31 | - | - |  | 432.5 MHz |  | – | 477.5 MHz |  |  |
| 32 | XXXII | - |  | N/A |  |  |  |  |  |
| 65 | - | - |  | 1900 MHz |  | – | 2030 MHz |  |  |
| 66 | - | - |  | 1690 MHz |  | – | 1800 MHz |  |  |
| 67 | - | - |  | N/A |  |  |  |  |  |
| 68 | - | - |  | 678 MHz |  | – | 748 MHz |  |  |
| 69 | - | - |  |  | N/A |  |  |  |
| 70 | - | - |  | 1675 MHz |  | – | 1730 MHz |  |  |
| 71 | - | - |  | 643 MHz |  | - | 718 MHz |  |  |
| 72 | - | - |  | 431 MHz |  | – | 476 MHz |  |  |
| 73 | - | - |  | 430 MHz |  | – | 475 MHz |  |  |
| 74 | - | - |  | 1407 MHz |  | – | 1490 MHz |  |  |
| 75 | - | - |  | N/A |  |  |  |  |  |
| 76 | - | - |  | N/A |  |  |  |  |  |
| NOTE 1: The band is for UTRA only. |  |  |  |  |  |  |  |  |  |

Table 4.4-2: Receiver exclusion band for base stations (unpaired bands)

| MSR and E-UTRA Band number | UTRA Band number | Receiver exclusion band |  |  |
| --- | --- | --- | --- | --- |
| 33 | a) | 1880 MHz | – | 1940 MHz |
| 34 | a) | 1990 MHz | – | 2045 MHz |
| 35 | b) | 1830 MHz | – | 1930 MHz |
| 36 | b) | 1910 MHz | – | 2010 MHz |
| 37 | c) | 1890 MHz | – | 1950 MHz |
| 38 | d) | 2550 MHz | – | 2640 MHz |
| 39 | f) | 1860 MHz | – | 1940 MHz |
| 40 | e) | 2280 MHz | – | 2420 MHz |
| 41 | - | 2476 MHz | – | 2710 MHz |
| 42 | - | 3380 MHz | – | 3620 MHz |
| 43 | - | 3580 MHz | – | 3820 MHz |
| 44 | - | 683 MHz | – | 823 MHz |
| 45 | - | 1427 MHz | – | 1487 MHz |
| 48 | - | 3530 MHz | – | 3720 MHz |
| 50 | - | 1412 MHz | – | 1537 MHz |
| 51 | - | 1407 MHz | – | 1452 MHz |

Table 4.4-3: Receiver exclusion band for NR base stations

| NR operating band | Receiver exclusion band |
| --- | --- |
| n1 | 1900 MHz – 2000 MHz |
| n2 | 1830 MHz – 1930 MHz |
| n3 | 1690 MHz – 1805 MHz |
| n5 | 804 MHz – 869 MHz |
| n7 | 2480 MHz – 2590 MHz |
| n8 | 860 MHz – 935 MHz |
| n20 | 812 MHz – 882 MHz |
| n28 | 683 MHz – 768 MHz |
| n38 | 2550 MHz – 2640 MHz |
| n41 | 2476 MHz – 2710 MHz |
| n50 | 1412 MHz – 1537 MHz |
| n51 | 1407 MHz – 1452 MHz |
| n66 | 1690 MHz – 1800 MHz |
| n70 | 1675 MHz – 1730 MHz |
| n71 | 643 MHz – 718 MHz |
| n74 | 1407 MHz – 1490 MHz |
| n75 | N/A |
| n76 | N/A |
| n77 | 3240 MHz – 4260 MHz |
| n78 | 3240 MHz – 3860 MHz |
| n79 | 4340 MHz – 5060 MHz |
| n80 | 1690 MHz – 1805 MHz |
| n81 | 860 MHz – 935 MHz |
| n82 | 812 MHz – 882 MHz |
| n83 | 683 MHz – 768 MHz |
| n84 | 1900 MHz – 2000 MHz |

*********************End of change*****************

*********************Next changed section*****************

8.2	Radiated emission from Base Station and ancillary equipment

8.2.1	Radiated emission for Base Stations

This test is applicable to Base stations, except for BS that are only single-RAT GSM/EDGE capable. This test shall be performed on a representative configuration of the Base station.

For Base Stations that are only single-RAT GSM/EDGE capable, the test method and limits in clause 8 of TS 51.021 [10] apply.

8.2.1.1	Definition

This test assesses the ability of BS to limit unwanted emission from the enclosure port.

8.2.1.2	Test method

a)	A test site fulfilling the requirements of ITU-R SM. 329 [15] shall be used. The BS shall be placed on a non-conducting support and shall be operated from a power source via a RF filter to avoid radiation from the power leads.

Mean power of any spurious components shall be detected by the test antenna and measuring receiver (e.g. a spectrum analyser). At each frequency at which a component is detected, the BS shall be rotated and the height of the test antenna adjusted to obtain maximum response, and the effective radiated power (e.r.p.) of that component determined by a substitution measurement. The measurement shall be repeated with the test antenna in the orthogonal polarization plane.

NOTE:	Effective radiated power (e.r.p.) refers to the radiation of a half wave tuned dipole instead of an isotropic antenna. There is a constant difference of 2,15 dB between e.i.r.p. and e.r.p.

e.r.p. (dBm)  e.i.r.p. (dBm)  2,15			Ref: ITU-R SM.329 ANNEX 1 [15].

b)	The BS shall transmit with maximum power declared by the manufacturer with all transmitters active. Set the base station to transmit a signal as stated in subclause 4.5.

c)	The received power shall be measured over the frequency range 30 MHz to 12.75 GHz, excluding 10 MHz below the lower RF bandwidth edge to 10 MHz above the upper RF bandwidth edge The measurement bandwidth shall be 100 kHz between 30 MHz and 1 GHz and 1 MHz above 1 GHz as given in ITU-R SM.329 [15]. The video bandwidth shall be approximately three times the resolution bandwidth. If this video bandwidth is not available on the measuring receiver, it shall be the maximum available and at least 1 MHz. Unless otherwise stated, all measurements are done as mean power (RMS).

8.2.1.3	Limits

The frequency boundary and reference bandwidths for the detailed transitions of the limits between the requirements for out of band emissions and spurious emissions are based on ITU-R Recommendations SM.329 [10] and SM.1539 [23].

The BS shall meet the limits below:

Table 8.2.1.3-1: Limits for radiated emissions from BS

| Frequency range | Minimum requirement (e.r.p.)/Reference Bandwidth |
| --- | --- |
| 30 MHz f <1000 MHz | -36 dBm/100 kHz |
| 1 GHz f <12,75 GHz | -30 dBm/ 1MHz |
| FDL_low - ΔfOBUE < f < FDL_high +ΔfOBUE (Note 1, Note 2) | Not defined |
| NOTE 1: For BS capable of multi-band operation, the frequency ranges relating to the RF bandwidths of all supported bands apply. NOTE 2: ΔfOBUE as defined in Table 8.2.1.3-2. |  |

Table 8.2.1.3-2: Maximum offset of OBUE outside the downlink operating band

| Operating band characteristics | ΔfOBUE [MHz] |
| --- | --- |
| FDL_high – FDL_low < 200 MHz | 10 |
| 200 MHz < FDL_high – FDL_low 900 MHz | 40 |

8.2.1.4	Interpretation of the measurement results

The interpretation of the results recorded in a test report for the radiated emission measurements described in the present document shall be as follows:

-	the measured value related to the corresponding limit will be used to decide whether an equipment meets the requirements of the present document;

-	the value of the measurement uncertainty for the measurement of each parameter shall be included in the test report;

-	the recorded value of the measurement uncertainty shall be, for each measurement, equal to or lower than the figures in Table 8.2.1-4-1 for BS.

Table 8.2.2 specifies the Maximum measurement uncertainty of the Test System. The Test System shall enable the equipment under test to be measured with an uncertainty not exceeding the specified values. All tolerances and uncertainties are absolute values, and are valid for a confidence level of 95 %, unless otherwise stated.

A confidence level of 95% is the measurement uncertainty tolerance interval for a specific measurement that contains 95% of the performance of a population of test equipment.

Table 8.2.1.4-1: Maximum measurement uncertainty (BS)

| Parameter | Uncertainty for EUT dimension ≤ 1 m | Uncertainty for EUT dimension >1 m |
| --- | --- | --- |
| Effective radiated RF power between 30 MHz to 180 MHz | 6 dB | 6 dB |
| Effective radiated RF power between 180 MHz to 4 GHz | 4 dB | 6 dB |
| Effective radiated RF power between 4 GHz to 12,75 GHz | 6 dB | 9 dB (Note 1) |
| NOTE 1: This value may be reduced to 6 dB when further information on the potential radiation characteristic of the EUT is available. |  |  |

NOTE 2:	If the Test System for a test is known to have a measurement uncertainty greater than that specified in Table 8.2.2, this equipment can still be used, provided that an adjustment is made follows:

Any additional uncertainty in the Test System over and above that specified in Table 8.2.1.4-1 is used to tighten the Test Requirements - making the test harder to pass. This procedure will ensure that a Test System not compliant with Table 8.2.1.4-1 2 does not increase the probability of passing an EUT that would otherwise have failed a test if a Test System compliant with Table 8.2.1.4-1 had been used.

8.2.2	Radiated emission, ancillary equipment

This test is applicable to ancillary equipment. This test shall be performed on a representative configuration of the ancillary equipment.

8.2.2.1	Definition

This test assesses the ability of ancillary equipment to limit unwanted emission from the enclosure port.

8.2.2.2	Test method

The test method shall be in accordance with CISPR 32 34.

8.2.2.3	Limits

The ancillary equipment shall meet the limits according to CISPR 32 34 shown in Table 8.2.2.3-1 and Table 8.2.2.3-2.

Table 8.2.2.3-1: Limits for radiated emissions from ancillary equipment, measured on a stand-alone basis (10 m measuring distance)

| Frequency range | Quasi-peak |
| --- | --- |
| 30 MHz-230 MHz | 30 dBµV/m |
| 230 MHz-1000 MHz | 37 dBµV/m |

Table 8.2.2.3-2: Limits for radiated emissions from ancillary equipment, measured on a stand-alone basis (3 m measuring distance)

| Frequency range GHz | Average limit dBµV/m | Peak limit dBµV/m |
| --- | --- | --- |
| 1 to 3 | 50 | 70 |
| 3 to 6 | 54 | 74 |
| NOTE: The lower limit applies at the transition frequency. |  |  |

-------------End of text proposal-------------