3GPP TSG-RAN WG4 Meeting #87	R4-186950

# Busan, KR, 21st – 25th May, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TR37.843 -Indoor anechoic chamber test method procedure for OTA adjacent channel selectivity, narrow-band blocking, and in-band blocking |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA |  |  |  |  |  |  | Date: |  | 2018-05-14 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For adding an indoor anechoic chamber test method on OTA adjacent channel selectivity, narrow-band blocking, and in-band blocking requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | An anechoic chamber test method and MU for adjacent channel selectivity, narrow-band blocking, and in-band blocking is described |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An indoor anechoic chamber test method is not available for adjacent channel selectivity, narrow-band blocking, and in-band blocking OTA test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

10.3.5  OTA adjacent channel selectivity, general blocking and narrowband blocking

10.3.5.1 General

10.3.5.2 Indoor anechoic chamber test method

10.3.5.2.1 General

This method measures the adjacent channel selectivity, narrow-band blocking, and in-band blocking in an anechoic chamber with the separation between the manufacturer declared coordinate system reference point of the AAS BS and the phase centre of the receiving antenna of no less than 2D2/λ, where D is the largest dimension of the antenna of AAS BS and λ is the wavelength. The measurement system setup is as depicted in figure 10.3.5.2.1.

# Figure 10.3.5.2.1: Indoor Anechoic Chamber measurement system setup for  adjacent channel selectivity, general blocking and narrowband blocking

10.3.5.2.2 Calibration

Calibration shall be done with the procedure shown in 10.3.2.1.2 for the frequencies of wanted and unwanted signals with power combier/coupler. Calculate the calibration value for wanted and unwanted signal from A to D as well as from A to E.

LWanted_cal, A→D:  Calibration value for wanted signal between A and D in figure 10.3.5.2.1.

LUnanted_cal, A→E:  Calibration value for unwanted signal between A and E in figure 10.3.5.2.1.

10.3.5.2.3 Procedure

1)	Uninstall the reference antenna and install the AAS with its manufacturer declared coordinate system reference point in the same place as the phase centre of the reference antenna. The manufacturer declared coordinate system orientation of the AAS is set to be aligned with testing system.

2)	Set the AAS BS to be satisfied with the RoAoA desired by the requirement covering conformance testing receiving direction.

3)	Rotate the AAS to make the conformance testing receiving direction aligned with the beam peak direction of the reference antenna at the calibration stage.

4)	For FDD AAS BS start BS transmission at the required condition.

5)	Set the test signal mean power at the RF signal source generator for wanted signal as the required level plus LWanted_cal, A→D  and the reference measurement channel.

6) Set the test signal mean power at the RF signal source generator for unwanted signal as the required level plus LUnwanted_cal, A→E  and the intereferece signal channl.

6)	Measure the throughput according to annex E of the 3GPP TS 36.141 [??].

7)	Repeat the above steps 1)~6) per conformance testing direction.

10.3.5.2.4 MU assesment

10.3.5.2.4.1 MU Budget

Table 10.3.5.2.4.1 MU Budget for ACS, narrow band blocking, and in-band blocking

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Received wanted/unwanted signal level |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | C1-1 |
| 2 | Pointing misalignment between the AAS BS and the transmitting antenna. | C1-2 |
| 3 | Quality of quiet zone | C1-3 |
| 4 | Polarization mismatch between the AAS BS and the transmitting antenna | C1-4 |
| 5 | Mutual coupling between the AAS BS and the transmitting antenna | C1-5 |
| 6 | Phase curvature | C1-6 |
| 7 | Uncertainty of the RF signal source generator | E |
| 8 | Impedance mismatch in the transmitting chain | C1-8 |
| 9 | Random uncertainty | C1-9 |
| Calibration measurement |  |  |
| 10 | Impedance mismatch between the transmitting antenna and the network analyzer | C1-10 |
| 11 | Positioning and pointing misalignment between the reference antenna and the transmitting antenna | C1-11 |
| 12 | Impedance mismatch between the reference antenna and network analyzer. | C1-12 |
| 13 | Quality of quiet zone | C1-3 |
| 14 | Polarization mismatch for reference antenna | C1-4 |
| 15 | Mutual coupling between the reference antenna and the transmitting antenna | C1-5 |
| 16 | Phase curvature | C1-6 |
| 17 | Uncertainty of the network analyzer | E |
| 18 | Influence of the reference antenna feed cable a) Flexing cables, adapters, attenuators, and connector repeatability | C1-14 |
| 19 | Reference antenna feed cable loss measurement uncertainty | C1-15 |
| 20 | Influence of the transmitting antenna feed cable a) Flexing cables, adapters, attenuators, and connector repeatability | C1-16 |
| 21 | Uncertainty of the absolute gain of the reference antenna | E |
| 22 | Uncertainty of the absolute gain of the transmitting antenna | C1-18 |

10.3.5.2.4.2 MU Value

Table 10.3.5.2.4.2-1 MU value of singal level and calibration for ACS, narrow band blocking, and in-band blocking

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reveived wanted/unwanted signal level |  |  |  |  |  |  |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | 0.03 | 0.03 | Rectangular | 1.73 | 1 | 0.02 | 0.02 |
| 2 | Pointing misalignment between the AAS BS and the transmitting antenna | 0.3 | 0.3 | Rectangular | 1.73 | 1 | 0.17 | 0.17 |
| 3 | Quality of quiet zone | 0.1 | 0.1 | Gaussian | 1.00 | 1 | 0.10 | 0.10 |
| 4 | Polarization mismatch between the AAS BS and the transmitting antenna | 0.01 | 0.01 | Rectangular | 1.73 | 1 | 0.01 | 0.01 |
| 5 | Mutual coupling between the AAS BS and the transmitting antenna | 0 | 0 | Rectangular | 1.73 | 1 | 0.00 | 0.00 |
| 6 | Phase curvature | 0.05 | 0.05 | Gaussian | 1.00 | 1 | 0.05 | 0.05 |
| 7 | Uncertainty of the RF signal generator | 0.46 | 0.46 | Gaussian | 1.00 | 1 | 0.46 | 0.46 |
| 8 | Impedance mismatch in the transmitting chain | 0.14 | 0.23 | U-shaped | 1.41 | 1 | 0.10 | 0.16 |
| 9 | Random uncertainty | 0.1 | 0.1 | Rectangular | 1.73 | 1 | 0.06 | 0.06 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.52 | 0.53 |
| Calibration measurement |  |  |  |  |  |  |  |  |
| 10 | Impedance mismatch between the transmitting antenna and the network analyzer | 0.05 | 0.05 | U-shaped | 1.41 | 1 | 0.04 | 0.04 |
| 11 | Positioning and pointing misalignment between the reference antenna and the transmitting antenna | 0.01 | 0.01 | Rectangular | 1.73 | 1 | 0.01 | 0.01 |
| 12 | Impedance mismatch between the reference antenna and network analyzer | 0.05 | 0.05 | U-shaped | 1.41 | 1 | 0.04 | 0.04 |
| 13 | Quality of quiet zone | 0.1 | 0.1 | Gaussian | 1.00 | 1 | 0.10 | 0.10 |
| 14 | Polarization mismatch for reference antenna | 0.01 | 0.01 | Rectangular | 1.73 | 1 | 0.01 | 0.01 |
| 15 | Mutual coupling between the reference antenna and the transmitting antenna | 0 | 0 | Rectangular | 1.73 | 1 | 0.00 | 0.00 |
| 16 | Phase curvature | 0.05 | 0.05 | Gaussian | 1.00 | 1 | 0.05 | 0.05 |
| 17 | Uncertainty of the network analyzer | 0.13 | 0.20 | Gaussian | 1.00 | 1 | 0.13 | 0.20 |
| 18 | Influence of the reference antenna feed cable | 0.05 | 0.05 | Rectangular | 1.73 | 1 | 0.03 | 0.03 |
| 19 | Reference antenna feed cable loss measurement uncertainty | 0.06 | 0.06 | Gaussian | 1.00 | 1 | 0.06 | 0.06 |
| 20 | Influence of the transmitting antenna feed cable | 0.05 | 0.05 | Rectangular | 1.73 | 1 | 0.03 | 0.03 |
| 21 | Uncertainty of the absolute gain of the reference antenna | 0.50 | 0.25 | Rectangular | 1.73 | 1 | 0.29 | 0.25 |
| 22 | Uncertainty of the absolute gain of the transmitting antenna | 0 | 0 | Rectangular | 1.73 | 1 | 0.00 | 0.00 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.35 | 0.35 |

The test system uncertainty is obtained by the following equation based on sub-clause 4.1.2.2 of 3GPP TS 36.141[22]:

Test system standard uncertainty= (wanted_signal_level_error)2+(unwanted_signal_level_error)2+(calibration_measurement_error)2+ACLR effect

Here, ACLR effect for ACS, narrow-band blocking, in-band blocking is 0.4 as shown in sub-clause 4.1.1.2 of 3GPP TS 36.141[22].

The wanted_signal_level_error and unwanted_signal_error , and calibration_measuremnt_error are standard uncertainty of the received signal level by AAS BS for OTA as shown in Table10.3.5.2.4.2-1.

Total test system MU for ACS, narrow band blocking, and in-band blocking is shown in Table 10.3.5.2.4.2-2

Table 10.3.5.2.4.2-2 MU Value of ACS, narrow band blocking, and in-band blocking

.

| OTA Requirements | ACS, narrow-band blocking, and In-band blocking |  |
| --- | --- | --- |
| Frequency range | f ≦ 3 GHz | 3 GHz < f ≦ 4.2 GHz |
| Test system standard uncertainty (1) | 1.21 | 1.23 |
| Test system expand standard uncertainty (1.96) | 2.37 | 2.41 |