3GPP TSG-RAN WG4 #87	R4- 1807444

Busan, Korea 21st April –25th May 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR |  | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.104: Annex B.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-05-21 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | EVM window parameters needs to be defined for FR1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Tables are included for 15 kHz, 30 kHz, and 60 kHz SCS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current specification does not have any information regarding to EVM window length. This is needed to complete the full requirement of transmit signal quality. An EVM requirement level alone is insufficient in defining the full signal quality requirement. The window lengths are noticeably smaller than that of LTE, however the higher spectral utilization and therefore the relative bandwidth is larger in NR. As one impact of high spectral utilization is the need for sharp filter implementation for to meet spectral confinement requirements, implying longer filters. As a result, this leads to higher inter-symbol interference which reveals itself as EVM. Although other RF impairments contribute to the overall EVM level, the larger spectral utilization defined in NR would require stricter spectral confinement filtering and as such introduce a larger portion of ISI to neighbouring symbols in the time domain. The impact due to filtering is prevalent with a window length greater than 85% has shown in past contributions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex B.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | Only window lengths for normal CP lengths have been proposed, window length for extended CP lengths are FFS. |  |  |  |  |  |  |  |  |

# B.5	Estimation of time offset

## B.5.1	General

The observation period for determining the sample timing difference shall be 1 ms.

In the following   represents the middle sample of the EVM window of length  (defined in B.5.2) or the last sample of the first window half if is even.

is estimated so that the EVM window of length  is centred on the measured cyclic prefix of the considered OFDM symbol. To minimize the estimation error the timing shall be based on the primary synchronization signal and reference signals. To limit time distortion of any transmit filter the reference signals in the 1 outer RBs are not taken into account in the timing estimation

Two values for  are determined:

and

where  if  is odd and  if is even.

When the cyclic prefix length varies from symbol to symbol (e.g. time multiplexed MBMS and unicast) then   shall be further restricted to the subset of symbols with the considered modulation scheme being active and with the considered cyclic prefix length type.

## B.5.2	Window length

Table E.5.2-1, E.5.2-2, E.5.2-3 below specify EVM window length (W) for normal CP, the cyclic prefix length for each corresponding bandwidth and subcarrier spacing.

# Table E.5.2-1: EVM window length for normal CP for NR (15kHz SCS)

| Channel Bandwidth MHz | FFT size | Cyclic prefix length for symbols 1-6 in FFT samples | EVM window length W | Ratio of W to total CP for symbols 1-6(Note 1) [%] |
| --- | --- | --- | --- | --- |
| 5 | 512 | 36 | [14] | [40] |
| 10 | 1024 | 72 | [29] | [40] |
| 15 | 1526 | 108 | [43] | [40] |
| 20 | 2048 | 144 | [58] | [40] |
| 25 | 3072 | 216 | [86] | [40] |
| 40 | 4096 | 288 | [179] | [62] |
| 50 | 4096 | 288 | [179] | [62] |

# Table E.5.2-2: EVM window length for normal CP for NR (30kHz SCS)

| Channel Bandwidth MHz | FFT size | Cyclic prefix length for symbols 1-6 in FFT samples | EVM window length W | Ratio of W to total CP for symbols 1-6(Note 1) [%] |
| --- | --- | --- | --- | --- |
| 5 | 256 | 18 | [7] | [40] |
| 10 | 512 | 36 | [14] | [40] |
| 15 | 768 | 54 | [22] | [40] |
| 20 | 1024 | 72 | [29] | [40] |
| 25 | 1024 | 72 | [29] | [40] |
| 30 | 1536 | 108 | [54] | [50] |
| 40 | 2048 | 144 | [89] | [62] |
| 50 | 2048 | 144 | [89] | [62] |
| 60 | 3072 | 216 | [162] | [75] |
| 70 | 3072 | 216 | [162] | [75] |
| 80 | 4096 | 288 | [233] | [81] |
| 90 | 4096 | 288 | [233] | [81] |
| 100 | 4096 | 288 | [233] | [81] |

# Table E.5.2-2: EVM window length for normal CP for NR (60kHz SCS)

| Channel Bandwidth MHz | FFT size | Cyclic prefix length for symbols 1-6 in FFT samples | EVM window length W | Ratio of W to total CP for symbols 1-6(Note 1) [%] |
| --- | --- | --- | --- | --- |
| 10 | 256 | 18 | [7] | [40] |
| 15 | 512 | 36 | [14] | [40] |
| 20 | 768 | 54 | [22] | [40] |
| 25 | 1024 | 72 | [29] | [40] |
| 40 | 1024 | 72 | [29] | [40] |
| 50 | 1536 | 108 | [54] | [50] |
| 60 | 2048 | 144 | [89] | [62] |
| 80 | 2048 | 144 | [89] | [62] |
| 100 | 3072 | 216 | [162] | [75] |