3GPP TSG-RAN WG4 #87	R4- 1807447

Busan, Korea 21st April –25th May 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR |  | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 37.843: Section 10 OTA Transmitted signal quality – Frequency Error |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-05-21 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Adding CATR test method procedure and calibration steps for OTA Transmitted signal quality – frequency error requirement |  |  |  |  |  |  |  |  |
| Summary of change: |  | The addition of text for OTA Transmitted signal quality – frequency error requirement for TX directional requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test procedure is needed for OTA frequency error requirement with respect to the CATR test method. If not included, then CATR test method is excluded from this OTA requirement for conformance requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 10.2.4.3	CATR

##### 10.2.4.3.1	General

##### 10.2.4.3.2 	Calibration

{editors note: if calibration is same as previous req then refernce}

The calibration measurement is done by using a reference antenna (SGH used in Figure 10.2.4.3.2-1) with known efficiency or gain values. In the calibration measurement the reference antenna is measured in the same place as the DUT, and the attenuation of the complete transmission path (C↔A, as in Figure 10.2.4.3.2-1) from the DUT to the measurement receiver is calibrated out. Figure 10.2.4.3.2-1 presents a setup of a typical compact antenna test range.

Figure 10.2.4.3.2-1: CATR calibration system setup.

Calibration procedure:

1)	Path loss calibration C→A:

a)	Measure SGH (or other calibrated reference antenna) reflection coefficient separately at the antenna's connector with a network analyser (or equivalent measurement equipment) to obtain ΓSGH.

b)	Measure cable loss from point C to input of SGH, call this LC↔SGH which is the equivalent of 20log|S21| from the use of a network analyser.

c)	Calculate the combined total path loss from C→A by using the following expression:

-	LSGHcal = LC,SGH + 10log(1 - |ΓSGH|2) - GSGH;

-	where10log1-ΓSGH2  10log(1 - |ΓSGH|2)  is the compensation for SGH connector return loss, GSGH  is the known gain of the reference SGH.

2)	Connect SGH and C↔A cable.

3)	To remove polarization(s) mismatch between range antenna (labelled as feeder antenna in diagram) and SGH use positions to position the SGH in the beam peak direction of range antenna.

4)	Measure path loss C→B with network analyzer LC→B = 20log|S21|.

5)  Calculate the test path loss compensation factor.  This is the total path loss between A↔B using the results from step 1c and 4. L = LSGHcal LSGHcal  - LC→B.

Where ΓSGH is the reflection coefficient (or mismatch) seen at the SGH connector (S11 with a network analyzer).

##### 10.2.4.3.3 	Procedure

As the frequency error is tested together with EVM, the measurement procedure is same with EVM.

1) Align DUT with beam peak direction of range antenna.

2) Configure TX branch and carrier at a time according to the manufacturer's declared rated output power.

3) Set the AAS BS to transmit the test signal according to E-TM3.1 at 5 MHz bandwidth configuration.

4) Measure frequency error of each carrier arriving at the measurement equipment (such as a spectrum analyzer or equivalent instrument) denoted in figure [10.2.5.3.1-1].

5) Repeat steps 3-4 for all conformance test beam direction pairs as described in 3GPP TS 37.145-2 [1], subclause 6.2.

6) Repeat steps 3-5 for E-TM3.2, E-TM3.3 and E-TM2. Note: all the discussions above are based on the measurement pre-condition of reference clock synchronized between measurement system with RF frequency signal denoted in figure [10.2.5.3.1-1].