3GPP TSG-RAN WG4 Meeting #87                 	R4-1807762

Busan, Korea (Republic of), 21-25 May 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 38.133 on Interruptions for EN-DC and SA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2017-05-14 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Requirements for EN-DC and SA interruptions when NR cell is the victim are incomplete in 38.133 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Requirements for interruptions during measurements on deactivated NR and E-UTRA SCells are introduced. - Interruption duration for inter-band interruption is updated for SCell activation/deactivation to be based on agreements during RAN4#86bis. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1, 8.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 8.2	Interruption

### 8.2.1	NSA: Interruptions with EN-DC

#### 8.2.1.1	Introduction

This section contains the requirements related to the interruptions on PSCell, and SCell, when

E-UTRA PCell transitions between active and non-active during DRX, or

E-UTRA PCell transitions from non-DRX to DRX, or

E-UTRA SCell in MCG or SCell in SCG is added or released, or

E-UTRA SCell in MCG or SCell in SCG is activated or deactivated, or

measurements on SCC with deactivated E-UTRA SCell in MCG or with deactivated SCell in SCG

The requirements shall apply for E-UTRA-NR DC with an E-UTRA PCell.

This section contains interruptions where victim cell is PSCell or SCell belonging to SCG. Requirements for interruptions requirements when the victim cell is E-UTRA PCell or E-UTRA SCell belonging to MCG are specified in [TS36.133].

#### 8.2.1.2	Requirements

##### 8.2.1.2.1	Interruptions at transitions between active and non-active during DRX

Interruption on PSCell and the activated SCell if configured due to E-UTRA PCell transitions between active and non-active during DRX when PSCell or SCell is in non-DRX are allowed with up to [1%] probability of missed ACK/NACK when the configured PSCell DRX cycle is less than [640] ms, and [0.625%] probability of missed ACK/NACK is allowed when the configured PSCell DRX cycle is [640] ms or longer. Each interruption shall not exceed [X] slot as defined in table 8.2.1.2.1-1.

Each interruption shall not exceed [X] slot as defined in table 8.2.1.2.1-1.

Table 8.2.1.2.1-1 Interruption length X at transition between active and non-active during DRX

|  | NR Slot length (ms) | Interruption length X |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 0 | 1 | [1] | [2] |
| 1 | 0.5 | [1] | [2] |
| 2 | 0.25 | [3] |  |
| 3 | 0.125 | [5] |  |

When both PCell and PSCell are in DRX, no interruption is allowed.

##### 8.2.1.2.2	Interruptions at transitions from non-DRX to DRX

Interruption on PSCell and the activated SCell if configured due to E-UTRA PCell transitions from non-DRX to DRX when PSCell or SCell is in non-DRX shall not exceed [X] slot as defined in table 8.2.1.2.1-1.

##### 8.2.1.2.3	Interruptions at SCell addition/release

The requirements in this clause shall apply for the UE configured with PSCell.

When one E-UTRA SCell is added or released:

-	an interruption on PSCell:

-	of up to [X1] slot, if the PSCell is not in the same band as any of the E-UTRA SCells being added or released, or

-	of up to [Y1] slot, if the PSCell is in the same band as any of the E-UTRA SCells being added or released;

-	an interruption on any activated SCell in SCG:

-	of up to [X1] slot, if the activated SCell is not in the same band as any of the E-UTRA SCells being added or released, or

-	of up to [Y1] slot, if the activated SCell is in the same band as any of the E-UTRA SCells being added or released.

When one SCell is added or released:

-	an interruption on PSCell:

-	of up to [X1] slot, if the PSCell is not in the same band as any of the SCells being added or released, or

-	of up to [Y1] slot, if the PSCell is in the same band as any of the SCells being added or released;

-	an interruption on any activated SCell in SCG:

-	of up to [X1] slot, if the activated SCell is not in the same band as any of the SCells being added or released, or

-	of up to [Y1] slot, if the activated SCell is in the same band as any of the SCells being added or released.

Table 8.2.1.2.3-1 Interruption length X1 and Y1 at SCell addition/Release

|  | NR Slot length (ms) | Interruption length X1 slot |  | Interruption length Y1 slot |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Sync | Async | Sync | Async |
| 0 | 1 | [1] | [2] | [TBD] | [TBD] |
| 1 | 0.5 | [2] | [3] | [TBD] | [TBD] |
| 2 | 0.25 | [5] |  | [TBD] |  |
| 3 | 0.125 | [9] |  | [TBD] |  |

##### 8.2.1.2.4	Interruptions at SCell activation/deactivation

The requirements in this clause shall apply for the UE configured with PSCell and one SCell.

When one E-UTRA SCell is activated or deactivated:

-	an interruption on PSCell:

-	of up to [X2] slot, if the PSCell is not in the same band as any of the E-UTRA SCells being activated or deactivated, or

-	of up to [Y2] slot, if the PSCell is in the same band as any of the E-UTRA SCells being activated or deactivated;

-	an interruption on any activated SCell in SCG:

-	of up to [X2] slot, if the activated SCell is not in the same band as any of the E-UTRA SCells being activated or deactivated, or

-	of up to [Y2] slot, if the activated SCell is in the same band as any of the E-UTRA SCells being activated or deactivated.

When one SCell is activated or deactivated:

-	an interruption on PSCell:

-	of up to [X2] slot, if the PSCell is not in the same band as any of the SCells being activated or deactivated, or

-	of up to [Y2] slot, if the PSCell is in the same band as any of the SCells being activated or deactivated;

-	an interruption on any activated SCell in SCG:

-	of up to [X2] slot, if the activated SCell is not in the same band as any of the SCells being activated or deactivated, or

-	of up to [Y2] slot, if the activated SCell is in the same band as any of the SCells being activated or deactivated.

Table 8.2.1.2.4-1 Interruption length X2 and Y2 at SCell activation/deactivation

|  | NR Slot length (ms) | Interruption length X2 slot |  | Interruption length Y2 slot |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Sync | Async | Sync | Async |
| 0 | 1 | [1] | [2] | [TBD] | [TBD] |
| 1 | 0.5 | [1] | [2] | [TBD] | [TBD] |
| 2 | 0.25 | [3] |  | [TBD] |  |
| 3 | 0.125 | [5] |  | [TBD] |  |

##### 8.2.1.2.5	Interruptions during measurements on SCC

When one E-UTRA SCell in MCG is deactivated, the UE is allowed due to measurements on the SCC with the deactivated SCell:

-	an interruption on PSCell or any activated SCell with up to 0.5% probability of missed ACK/NACK when any of the configured measCycleSCell [2] for the deactivated E-UTRA SCells is 640 ms or longer.

-	an interruption on PSCell or any activated SCell with up to 0.5% probability of missed ACK/NACK regardless of the configured measCycleSCell [2] for the deactivated E-UTRA SCells if indicated by the network using IE allowInterruptions [2].

Each interruption shall not exceed

-	 [X3] slot, if the PSCell or activated SCell is not in the same band as the E-UTRA SCell being deactivated, or

-	 [Y3] slot, if the PSCell or activated SCell is in the same band as the E-UTRA SCell being deactivated.

When one SCell in SCG is deactivated, the UE is allowed on PSCell or any activated SCell due to measurements on the SCC with the deactivated SCell immediately before and immediately after the measured SMTC as indicated to the UE for measurements by the network, an interruption of:

-	 [X3] slot, if the PSCell or activated SCell is not in the same band as the E-UTRA SCell being deactivated, or

-	 [Y3] slot, if the PSCell or activated SCell is in the same band as the E-UTRA SCell being deactivated.

Table 8.2.1.2.5-1 Interruption length X3 and Y3 at measurements on deactivated SCC

|  | NR Slot length (ms) | Interruption length X3 slot |  | Interruption length Y3 slot |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Sync | Async | Sync | Async |
| 0 | 1 | [1] | [2] | [TBD] | [TBD] |
| 1 | 0.5 | [1] | [2] | [TBD] | [TBD] |
| 2 | 0.25 | [3] |  | [TBD] |  |
| 3 | 0.125 | [5] |  | [TBD] |  |

##### 8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration

The requirements in this clause shall apply when a supplementary UL carrier or an UL carrier is configured or de-configured in NR non-standalone operation as defined in [2].

When an UL carrier or supplementary UL carrier is configured or deconfigured, an interruption on LTE PCell, all activated LTE SCells, PSCell and all activated SCells within the same FR as the reconfigured uplink carrier of up to [1] slot, is allowed immediately after the RRC reconfiguration procedure [2]. The interruption is for both uplink and downlink of LTE PCell, all LTE activated SCells, PSCell and all activated SCells within the same FR as the configured or de-configured UL.

### 8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation

#### 8.2.2.1	Introduction

This section contains the requirements related to the interruptions on PCell and activated SCell if configured, when up to TBD SCells are configured, deconfigured, activated or deactivated.

Note:	interruptions at SCell addition/release, activation/deactivation and during measurements on SCC may not be required by all UEs.

#### Editor’s Note:	The interruptions shall not interrupt RRC signalling or ACK/NACKs related to RRC reconfiguration procedure [2] for SCell addition/release or MAC control signalling [17] for SCell activation/deactivation command. How to specify this is FFS.

#### 8.2.2.2	Requirements

##### 8.2.2.2.1	Interruptions at SCell addition/release

When any number of SCells between one and TBD is added or released using the same RRCConnectionReconfiguration message as defined in [2], the UE is allowed an interruption on PCell and on any activated SCell during the RRC reconfiguration procedure as follows:

-	an interruption on PCell:

-	of up to the duration shown in table 8.2.2.2.1-1, if the PCell is not in the same band as any of the SCells being added or released, or

-	of up to the duration shown in table 8.2.2.2.1-2, if the PCell is in the same band as any of the SCells being added or released;

-	an interruption on any activated SCell:

-	of up to to the duration shown in table 8.2.2.2.1-1, if the activated SCell is not in the same band as any of the SCells being added or released, or

-	of up to the duration shown in table 8.2.2.2.1-2, if the activated SCell is in the same band as any of the SCells being added or released.

Table 8.2.2.2.1-1: Interruption duration for SCell addition/release for inter-band CA

|  | NR Slot length (ms) | Interruption length |
| --- | --- | --- |
| 0 | 1 | [1] |
| 1 | 0.5 | [2] |
| 2 | 0.25 | [5] |
| 3 | 0.125 | [9] |

Table 8.2.2.2.1-2: Interruption duration for SCell addition/release for intra-band CA

|  | NR Slot length (ms) | Interruption length |
| --- | --- | --- |
| 0 | 1 | [TBD] |
| 1 | 0.5 | [TBD] |
| 2 | 0.25 | [TBD] |
| 3 | 0.125 | [TBD] |

##### 8.2.2.2.2	Interruptions at SCell activation/deactivation

When an intra-band SCell is activated or deactivated as defined in [17] the UE is allowed

-	an interruption on PCell:

-	of up to the duration shown in table 8.2.2.2.2-1, if the PCell is not in the same band as any of the SCells being activated or deactivated, or

-	of up to the duration shown in table 8.2.2.2.2-2 if the PCell is in the same band as any of the SCells being activated or deactivated;

-	an interruption on any activated SCell:

-	of up to the duration shown in table 8.2.2.2.2-1, if the activated SCell is not in the same band as any of the SCells being activated or deactivated, or

-	of up to the duration shown in table 8.2.2.2.2-2, if the activated SCell is in the same band as any of the SCells being activated or deactivated.

Table 8.2.2.2.2-1: Interruption duration for SCell activation/deactivation for inter-band CA

|  | NR Slot length (ms) | Interruption length |
| --- | --- | --- |
| 0 | 1 | [1] |
| 1 | 0.5 | [1] |
| 2 | 0.25 | [3] |
| 3 | 0.125 | [5] |

Table 8.2.2.2.2-2: Interruption duration for SCell activation/deactivation for intra-band CA

|  | NR Slot length (ms) | Interruption length |
| --- | --- | --- |
| 0 | 1 | [TBD] |
| 1 | 0.5 | [TBD] |
| 2 | 0.25 | [TBD] |
| 3 | 0.125 | [TBD] |

##### 8.2.2.2.3	Interruptions during measurements on SCC for intra-band CA

When one SCell in MCG is deactivated, the UE is allowed on PSCell or any activated SCell due to measurements on the SCC with the deactivated SCell immediately before and immediately after the measured SMTC as indicated to the UE for measurements by the network, an interruption of:

-	up to the duration shown in table 8.2.2.2.3-1, if the PSCell or activated SCell is not in the same band as the E-UTRA SCell being deactivated, or

-	up to the duration shown in table 8.2.2.2.3-2, if the PSCell or activated SCell is in the same band as the E-UTRA SCell being deactivated.

Table 8.2.2.2.3-1: Interruption duration for measurements on deactivated SCC

|  | NR Slot length (ms) | Interruption length |
| --- | --- | --- |
| 0 | 1 | [1] |
| 1 | 0.5 | [1] |
| 2 | 0.25 | [3] |
| 3 | 0.125 | [5] |

Table 8.2.2.2.3-2: Interruption duration for measurements on deactivated SCC

|  | NR Slot length (ms) | Interruption length |
| --- | --- | --- |
| 0 | 1 | [TBD] |
| 1 | 0.5 | [TBD] |
| 2 | 0.25 | [TBD] |
| 3 | 0.125 | [TBD] |

##### 8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration

The requirements in this clause shall apply when a supplementary UL carrier or an UL carrier is configured or de-configured in NR standalone carrier aggregation as defined in [2].

When an UL carrier or supplementary UL carrier is configured or deconfigured, an interruption on PCell and all activated SCells within the same FR as the reconfigured uplink carrier of up to [1] slot, is allowed immediately after the RRC reconfiguration procedure [2]. The interruption is for both uplink and downlink of PCell and all the activated SCells within the same FR as the configured or de-configured UL.