3GPP TSG-RAN WG4 Meeting #87						R4-1807845

Busan, Korea, 21 - 25 May 2018

Source: 	NXP Semiconductors

Title: 	Peak EIRP of Type 2 and 3 UE

Agenda item:	7.5.7.2

Document for:	Approval

# Introduction

In WG4#86-bis a WF on types of UE in FR2 was captured [1]. All FR2 UEs were segregated broadly into 4 different types while distinguishing them by minimum peak EIRP. Alternatively, it may be stated that the UEs were agreed to be of 4 different power classes.

Table 1: NR FR2 UE Type summary [1]

*Note: these notes are meant to illustrate examples that have the requirements listed, other examples are not precluded

Additional requirement on Spherical coverage was also depicted in this table to each power class. Email discussion has proposed that each UE type has one-to-one correspondence with a min peak EIRP (power class) in Rel-15 and thus may be distinguished without “Spherical class”. It may also be noted that the 4 UE types would not preclude examples other than that stated in “Comments” columns.  In this contribution, minimum peak EIRP of types 2 and 3 UE (power class 2 and 3) have been proposed for approval.

# Simulations Results

Table 2 shows peak EIRP budget for UE types 2 and 3 for various array configurations at bands n257 and n261. Peak EIRP of type 2 UE with 4 and 8 elements and 16dBm and 18dBm output power of PA (for QPSK modulation) are shown. Radiation pattern and gain of each patch that has been used in the budget are shown in [1]. Measured PA data file has been used with simulation of other sub-systems to simulate eirp of complete system. It may be observed that a wide range of peak EIRP could be achieved because constraints due to form factor are not anticipated in this application. While converging on a minimum peak EIRP value UMa use-case and 5G datarate enablement should be considered.

Proposal 1: for QPSK modulation, Type 2 UE has minimum peak EIRP of 29.5dBm in bands n257 and n261.

Proposal 2: for QPSK modulation, Type 2 UE has nominal peak EIRP of 36dBm in bands n257 and n261.

Proposal 3: for QPSK modulation, Type 3 UE has minimum peak EIRP of 35.5dBm in bands n257 and n261.

Proposal 4: for QPSK modulation, Type 3 UE has nominal peak EIRP of 39dBm in bands n257 and n261.

Table 2: Peak EIRP in boresight of various Array dimensions for bands n257 and n261

The author would like to state that although there is a breakdown of EIRP budget in table 2, it should be considered as few of several plausible solutions.

With a similar approach in band n260 table 3 has been derived.

Table 3: Peak EIRP in boresight of various Array dimensions for band n260

Proposal 5: for QPSK modulation, Type 2 UE has minimum peak EIRP of 28dBm in band n260.

Proposal 6: for QPSK modulation, Type 2 UE has nominal peak EIRP of 32dBm in band n260.

Proposal 7: for QPSK modulation, Type 3 UE has minimum peak EIRP of 34dBm in band n260.

Proposal 8: for QPSK modulation, Type 3 UE has nominal peak EIRP of 38dBm in band n260.

# Conclusions

Proposal 1: for QPSK modulation, Type 2 UE has minimum peak EIRP of 29.5dBm in bands n257 and n261.

Proposal 2: for QPSK modulation, Type 2 UE has nominal peak EIRP of 36dBm in bands n257 and n261.

Proposal 3: for QPSK modulation, Type 3 UE has minimum peak EIRP of 35.5dBm in bands n257 and n261.

Proposal 4: for QPSK modulation, Type 3 UE has nominal peak EIRP of 39dBm in bands n257 and n261.

Proposal 5: for QPSK modulation, Type 2 UE has minimum peak EIRP of 28dBm in band n260.

Proposal 6: for QPSK modulation, Type 2 UE has nominal peak EIRP of 32dBm in band n260.

Proposal 7: for QPSK modulation, Type 3 UE has minimum peak EIRP of 34dBm in band n260.

Proposal 8: for QPSK modulation, Type 3 UE has nominal peak EIRP of 38dBm in band n260.

# Reference

[1]	“WF on How to Distinguish UE Types in FR2”, R4-1805926, 3GPP RAN4#86-Bis.

[2]	“Follow-up on R4-1800944: Boresight and 40deg scan angle EIRP of an antenna module”, R4-1802158, NXP Semiconductors, 3GPP RAN4#86.