3GPP TSG-RAN WG4 #87	R4-1807860

Busan, Republic of South Korea, 21st – 25th May 2018

Source:	MVG Industries

Title:	TP to Draft CR 37.843 - OTA Reference Sensitivity in a Near Field Test Range

Agenda item:	6.28.3.1.3

Document for:	Approval

# Introduction

During 3GPP RAN4 #86-bis, the skeleton for TR 37.843 clause 10 conformance testing aspect was agreed. A number of contributions have been then approved for inclusion of specific testing methodologies in clause 10. This contribution is providing the measurement and calibration procedures along with MU assessment for OTA Reference Sensitivity measurement in Near Field test Range.

# Discussion

Near Field is a testing methodology which can be used for OTA sensitivity measurement, EIS measurement [1]. OTA Sensitivity in TR 37.843 is the same as the OTA sensitivity measurement in TR37.842. It means that same measurement, and calibration procedures can be used along with the MU assessment already agreed in TR37.842. Test scope and limitations apply here as in TR 37.842.

# Proposal

<< Unchanged sections omitted >>

<< Start of proposal >>

##### 10.3.3.4	Near Field Test range

10.3.3.4.1	Description

The near field measurement techniques consist in measuring amplitude and phase of the modulated signal at the AAS BS under test, on some specific surfaces such as planar, cylindrical, and spherical. The measurement setup is as depicted in clause 10.3.2.4.1.

# 10.3.3.4.2	Test Method limitations and scope

Same as in clause 10.3.2.4.2.

10.3.3.4.3	Calibration

Calibration shall be performed with the same procedure as in clause 10.3.2.4.3.

10.3.3.4.4	Procedure

Measurement procedure is depicted in clause 10,3,2,4,4 where in step 4) the sensitivity level is EISREFSENS.

##### 10.3.3.4.4	MU assessment

##### 10.3.3.4.4.1	MU budget

# Table 10.3.3.4.4.1-1: Near field test range uncertainty contributors in AAS OTA Reference Sensitivity measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
|  | Stage 2: EIS near field radiation pattern measurement and EIS near field DUT power measurement |  |
| 1 | Axes Intersection | E.5 |
| 2 | Axes Orthogonality | E.5 |
| 3 | Horizontal Pointing | E.5 |
| 4 | Probe Vertical Position | E.5 |
| 5 | Probe H/V pointing | E.5 |
| 6 | Measurement Distance | E.5 |
| 7 | Amplitude and Phase Drift | E.5 |
| 8 | Amplitude and Phase Noise | E.5 |
| 9 | Leakage and Crosstalk | E.5 |
| 10 | Amplitude Non-Linearity | E.5 |
| 11 | Amplitude and Phase Shift in rotary joints | E.5 |
| 12 | Channel Balance Amplitude and Phase | E.5 |
| 13 | Probe Polarization Amplitude and Phase | E.5 |
| 14 | Probe Pattern Knowledge | E.5 |
| 15 | Multiple Reflections | E.5 |
| 16 | Room Scattering | E.5 |
| 17 | DUT support Scattering | E.5 |
| 18 | Scan Area Truncation | E.5 |
| 19 | Sampling Point Offset | E.5 |
| 20 | Spherical Mode Truncation | E.5 |
| 21 | Positioning | E.5 |
| 22 | Probe Array Uniformity | E.5 |
| 23 | Mismatch of transmitter chain (i.e. between receiving antenna and vector signal generator) | E.5 |
| 24 | Insertion loss of transmitter chain | E.5 |
| 25 | Uncertainty of the absolute gain of the probe antenna | E.5 |
| 26 | Vector Signal Generator | E.5 |
| 27 | Measurement repeatability - Positioning Repeatability | E.5 |
|  | Stage 1:, Calibration measurement |  |
| 28 | Uncertainty of network analyser | E.5 |
| 29 | Mismatch of transmitter chain | E.5 |
| 30 | Insertion loss of transmitter chain | E.5 |
| 31 | Mismatch in the connection of the calibration antenna | E.5 |
| 32 | Influence of the calibration antenna feed cable | E.5 |
| 33 | Influence of the probe antenna cable | E.5 |
| 34 | Uncertainty of the absolute gain of the calibration antenna | E.5 |
| 35 | Short term repeatability | E.5 |

##### 10.3.3.4.4.2	MU value

# Table 10.3.3.4.4.2-1: Near field test range uncertainty assessment for EIS measurement

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Axes Intersection | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 2 | Axes Orthogonality | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 3 | Horizontal Pointing | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 4 | Probe Vertical Position | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 5 | Probe H/V pointing | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 6 | Measurement Distance | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 7 | Amplitude and Phase Drift | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 8 | Amplitude and Phase Noise | 0.02 | 0.02 | Gaussian | 1.00 | 1 | 0.02 | 0.02 |
| 9 | Leakage and Crosstalk | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 10 | Amplitude Non-Linearity | 0.04 | 0.04 | Gaussian | 1.00 | 1 | 0.04 | 0.04 |
| 11 | Amplitude and Phase Shift in rotary joints | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 12 | Channel Balance Amplitude and Phase | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 13 | Probe Polarization Amplitude and Phase | 0.0001 | 0.0001 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 14 | Probe Pattern Knowledge | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 15 | Multiple Reflections | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 16 | Room Scattering | 0.09 | 0.09 | Gaussian | 1.00 | 1 | 0.09 | 0.09 |
| 17 | DUT support Scattering | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 18 | Scan Area Truncation | 0.003 | 0.003 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 19 | Sampling Point Offset | 0.0058 | 0.0058 | Gaussian | 1.00 | 1 | 0.01 | 0.01 |
| 20 | Spherical Mode Truncation | 0.015 | 0.015 | Gaussian | 1.00 | 1 | 0.02 | 0.02 |
| 21 | Positioning | 0.03 | 0.03 | Rectangular | 1.73 | 1 | 0.02 | 0.02 |
| 22 | Probe Array Uniformity | 0.055 | 0.055 | Gaussian | 1.00 | 1 | 0.06 | 0.06 |
| 23 | Mismatch of transmitter chain | 0.284 | 0.284 | U-Shaped | 1.41 | 1 | 0.20 | 0.20 |
| 24 | Insertion loss of transmitter chain | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 25 | Uncertainty of the absolute gain of the probe antenna | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 26 | Vector Signal Generator | 0.46 | 0.46 | Gaussian | 1.00 | 1 | 0.46 | 0.46 |
| 27 | Measurement repeatability - positioning repeatability | 0.15 | 0.15 | Gaussian | 1.00 | 1 | 0.15 | 0.15 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 28 | Uncertainty of network analyser | 0.13 | 0.20 | Gaussian | 1.00 | 1 | 0.13 | 0.20 |
| 29 | Mismatch of transmitter chain | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 30 | Insertion loss of transmitter chain | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 31 | Mismatch in the connection of the calibration antenna | 0.02 | 0.02 | U-Shaped | 1.41 | 1 | 0.01 | 0.01 |
| 32 | Influence of the calibration antenna feed cable | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 33 | Influence of the probe antenna cable | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 34 | Reference antenna | 0.50 | 0.25 | Rectangular | 1.73 | 1 | 0.29 | 0.25 |
| 35 | Short term repeatability | 0.088 | 0.088 | Gaussian | 1.00 | 1 | 0.09 | 0.09 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.63 | 0.63 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1.24 | 1.24 |

NOTE:	Standard uncertainty values for the signal generator, network analyzer and reference antenna are according to the test equipment vendors proposal, as captured in annex E.

<< end of proposal >>

# Conclusion

Text proposal to TR 37.843 is presented in order to add the test procedure, and MU for OTA Reference Sensitivity measurement in a Near Field test range.

# Reference

[1] 3GPP TR 37.842 v 13.2.0, March 2017