3GPP TSG RAN WG4 Meeting #87bis	R4-1807884

Busan, Korea 21-25 May 2018

Source:		Keysight Technologies

Title:	Supplement to “On TRP Measurement Grids for mm-wave”

Agenda Item:		7.13.2

Document for:	Approval

# Introduction

Different options regarding the selection of a measurement grid for TRP measurements have been discussed in previous meetings. The WF on NR MU and test tolerance from 3GPP TSG-RAN WG4 AH-1801 lists several options [1]. TRP measurement uncertainties for constant density measurement grids with different number of measurement points have been already discussed in [2] and [3].

This contribution goes one step farther back and explores mathematical factors that impact the accuracy of TRP calculations with constant step size grids. We use the same simulated 8x2 path antenna array and simulation methodology as was used in [3] except with a different numerical approximation to the TRP surface integral.

# Discussion

## Evaluation Setup

The same simulated 8x2 patch antenna array as in [3] was used in the subsequent analysis.

This contribution focuses on the impact of integration on the simulated TRP errors due to sampling grids. The impact of mapping EIRP values to directions in spherical space will be addressed later, but is expected to show similar results.

Following the methodology in [3], we evaluate the reproducibility of TRP calculations over uniform angular spacing grids of different spacing, with random orientation of the DUT to the sampling grid.  The statics are derived from 10000 random orientations.

Figure 1 shows the composite antenna pattern for the 8x2 patch antenna array: 1a. is as in Fig.1 of [3], 1b. is our implementation function.

1a.   1b.

Figure 1:a. from [3], b. our version.

## Measurement Results

Figure 2 shows a histogram of TRP measurements for constant step size measurement grid of 15 degrees. Figure 2a is Figure 2 of [3]. Figure 2b is our simulation of under the same conditions. Figure 2b, is under the same conditions but we used an improved integration methodology.

2a) mean (-0.1dB), sigma 0.11 dB

2b) mean = 0.03 dB, sigma = 0.088 dB

Figure 2: a. from [3], and b. our simulations, showing a similar distribution, both mean and standard deviation.

Figure 3 shows a histogram of TRP measurements for constant step size measurement grid of 15 degrees, under the same conditions as above, but with the distribution peak directed to the pole (Z axis) under random orientations about the same z-axis. Figure 3a. is Figure 3 from [3]. Figure 3b, shows our result with a different integration methodology. This demonstrates that the integration methodology is equally, or more, important than the grid selection.

3 a) mean, -0.95, and sigma < 0.00 dB
b.          3 b) mean -0.212, and sigma 0.024

Figure 3: a) data form [3] b) is our integration method.

This shows that with an improved integration method, we reduce the ~ 1dB offset seen in other “uniform angular step” TRP calculations to -0.2dB with our integration method.

Figure 4 shows a histogram of TRP measurements for constant step size measurement grid of 10 degrees, under the same conditions as above.  4a is the plot from [3], 4b is our calculation with an improved integration method.

4a.  mean -0.29 dB, sigma 0.00 dB

4b.  mean 0.011dB, sigma 0.004 dB

Figure 4:  TRP for 10 deg. Constant steps, peak of profile toward pole, rotating about Z axis.

We show the elimination of the ~ 0.29dB offset with the standard integration method seen with 10 deg uniform angular steps.

Figure 5 shows a histogram of TRP measurements for constant step size measurement grid of 10 degrees, with the distribution oriented toward the equator of the sampling grid.  5a is the plot from [3], 5b is our calculation with an improved integration method.

5a.  mean -0.05 dB, sigma 0.06 dB

5b.  mean 0.001dB, sigma 0.057 dB

Figure 5: TRP for 10 deg. Constant steps, peak of profile on equator, but rotating about x axis.

Observations:

- The integration method is as strong a factor as the sampling grid in determining TRP errors.

Further considerations:

- We have shown above, that uniform sampling grids can provide reasonable TRP errors without having to go to very small sample sizes. Below we show this in more detail. We have performed simulations of the same test antenna with random orientation (over full 4-pi steradians) with different uniform sampling grid spacing (of 15, 12, 10 and 8 degrees). The plots are below in figure 6 as well as are summarized in table 1.

6a. 6b

6c. 6d.

Figure 6: a: 15 deg spacing, b. 12 deg spacing, c. 10 deg spacing, d. 8 deg spacing

Table 1: Uniform angular spacing, Mean and Std. Dev. Vs. angular spacing, simulated for 10 K random orientations

| Spacing (deg) | Mean TRP (dBm) | Std Dev. | Min | Max |
| --- | --- | --- | --- | --- |
| 15 | 0.03 | 0.085 | -0.282 | 0.618 |
| 12 | 0.03 | 0.042 | -0.410 | 0.394 |
| 10 | 0.03 | 0.024 | -0.251 | 0.279 |
| 8 | 0.03 | 0.008 | -0.073 | 0.122 |

Table 1: Uniform angular spacing, Mean and Std. Dev. Vs. angular spacing, simulated for 10 K random orientations

Note: we think the mean offset of 0.03 dB is a systematic offset in the simulation, and is present even with very fine sample size, and can be ignored.

These results show that a uniform sampling of 8 degrees is equivalent or better than a uniform sampling grid with 264 points.  As stated in [3], constant step size grids can be optimized for speed. In some cases, more samples can be taken faster in swept mode than with motion controllers navigating to specific points in a step-stop-step fashion. This work shows that TRP performance equivalent to that with a uniform area sampling grid can be obtained in these cases.

To further support these conclusions, additional simulations were run with random orientations, using uniform spacing in angle grids. The mean TRP, and std Dev (in linear, not in dB) using our integration method are given in Table 2 below.

| d = d (deg) | Mean TRP (mW) | StDev TRP (mW) | Error (%) {3/mean} |
| --- | --- | --- | --- |
| 0.5 | 1.0072 | 1.2e-8 | 3.5e-6 |
| 2.0 | 1.0072 | 6.3e-6 | 1.9e-3 |
| 4.0 | 1.0072 | 1.8e-4 | 5.4e-2 |
| 10 | 1.0068 | 1.5e-2 | 1.9 |
| 15 | 1.0068 | 7.3e-2 | 21.7 |

Table 2: TRP distributions from random orientations with uniform angular spaced grids.

Annexes A, B and C provide graphical results for  more cases showing that even with more extreme orientations, with proper integration and reasonable uniform sample spacing, reasonable error margins on TRP are obtained.

# Conclusion

This contribution demonstrated that valid TRP measurements can be extracted using sampling grids uniformly sampled in angle without resorting to very fine angular spacing.

Proposal: The requirement of uniform grid sampling to orient beam peak at the equator is removed from TRP measurement and calculation should based on our demonstrated alternative numerical integration algorithm, having improved mean and standard deviation for a given sample size.

We also would like to note that with modern wideband RF acquisition equipment, it is possible to capture instantaneous bandwidths of over 2 GHz bandwidth with a single trigger.  This reduces the benefit for stop-step motion in addressing measurement time for beam characterization over a broad (> 1 GHz) frequency range.

# References

- R4-1800745, CATR, Anritsu, Intel, “WF on NR MU and test tolerance”, 3GPP TSG RAN WG4 Meeting #AH-1801, San Diego, US, 22 - 26 January 2018

- R4-1802332, Rohde & Schwarz, “On Constant Density TRP Measurement Grids for mm-wave”, 3GPP TSG RAN WG4 Meeting #86, Athens, Greece, 26 February - 2 March 2018

- R4-1804463, Rohde & Schwarz, “On TRP Measurement Grids for mm-Wave”, 3GPP TSG RAN WG4 Meeting #86bis, 16-20 April 2018, Melbourne, Australia

Appendices: more test cases showing that even with more extreme orientations, with proper integration and reasonable uniform sample spacing, reasonable error margins on TRP are obtained.

# Annex A: TRP values for uniformly spaced angle grid with random rotations about z-axis, beam pointed always along Z-axis

Figure A1: 15 deg spacing: fan beam aligned along z-axis.

Figure A2: 10 degree spacing: fan beam aligned along z-axis

Figure A3: 8 degree spacing: Fan beam aligned along z-axis

| Spacing (deg) | Mean (dBm) | Sigma (dBm) | Min (dBm) | Max (dBm) |
| --- | --- | --- | --- | --- |
| 15 | -0.211 | 0.032 | -0.295 | -0.143 |
| 10 | 0.00 | 0.004 | -0.010 | 0.011 |
| 8 | 0.03 | 0.001 | 0.026 | 0.032 |

Table A1:  Statistics for Fan beam aligned along z-axis, for uniform spaced grids

# Annex B. TRP values for uniformly spaced angle grid with random rotations about x-axis, beam peak pointed always at equator along x-axis

Figure B1: 15 degrees spacing

Figure B2: 10 degrees spacing

Figure B3: 8 degrees spacing

Figure B4: 1 degree spacing: fan beam aligned along x axis, random rotations, and image.

Table B1:  Statistics for Fan beam aligned with X-axis, for uniform spaced grids

| Spacing (deg) | Mean (dBm) | Sigma (dBm) | Min (dBm) | Max (dBm) |
| --- | --- | --- | --- | --- |
| 15 | -0.041 | 0.180 | -0.472 | -0.289 |
| 10 | -0.012 | 0.085 | -0.262 | 0.032 |
| 8 | 0.016 | 0.031 | -0.076 | 0.032 |
| 1 | 0.031 | 0.000 | 0.031 | 0.031 |

Table B1:  Statistics for Fan beam aligned with X-axis, for uniform spaced grids

# Annex C: TRP values for uniformly spaced angle grid with random rotations about z-axis, beam pointed always along equator, fan beam aligned with longitude lines (worst case).

Figure C1: 15 degrees spacing, longitude

Figure C2: 10 degrees spacing, longitude

Figure C3: 8 degrees spacing, longitude

Figure C4:  sample 3D

Table C1:  Statistics for Fan beam pointed along equator, beam aligned with longitude lines, for uniform spaced grids  (worst case)

| Spacing (deg) | Mean (dBm) | Sigma (dBm) | Min (dBm) | Max (dBm) |
| --- | --- | --- | --- | --- |
| 15 | 0.022 | 0.252 | -0.472 | 0.675 |
| 10 | 0.031 | 0.094 | -0.262 | 0..281 |
| 8 | 0.031 | 0.036 | -0.076 | 0.131 |

Table C1:  Statistics for Fan beam pointed along equator, beam aligned with longitude lines, for uniform spaced grids (worst case)