3GPP TSG-RAN WG4 Meeting #87	draft R4-1808171

Busan, Korea, 21 - 25 May 2018

Agenda item:	7.5.10

Source: 		NTT DOCOMO, INC.

Title: 	WF on TDD configuration for NR UE REFSENS

Document for:	Approval

1. Background

At the RAN4 #87, the WF on TDD configuration for NR REFSENS was agreed [1]:

- The following parameters need to be decided by RAN4 #87 to finalize reference measurement channel for NR REFSENS For FR1 and FR2 - TDD configuration (UL/DL pattern) - DL HARQ timing related parameters (K0, K1) - Max number of HARQ process - For FR1, the following TDD/HARQ configurations are applied for NR REFSENS requirements - Semi-static TDD configuration - UL/DL pattern - FFS: TDD configuration for LTE side in EN-DC combinations (whether to apply the same configuration as NR side) - FFS: Different TDD pattern between EN-DC and other scenario SCS; Alt.1; Alt.2; Alt3; Alt4. 15kHz; Aligned with LTE config #2 (5ms periodicity). 30kHz; {0, 33, 0, 41} (Option 3 in R4-1803445); Aligned with LTE config #2 (5ms periodicity); {0 0 0 32 1} with 2.5ms periodicity; {0 0 0 12 1} with 2.5ms periodicity. 60kHz; {0, 33, 0, 41} (Option 3 in R4-1803445); Aligned with LTE config #2 (5ms periodicity); LTE config #2 with 1.25ms periodicity; {0 0 13 1} with 1ms periodicity. - For FR2 - semi-static TDD configuration - Postpone the discussion for FR2 to RAN4 #87 - K0 = [0] - FFS: K1 and max # of HARQ process

Based on above WF, at the RAN4 #87, TDD configuration for NR REFSENS was discussed in the contributions [2-6]. This document summarizes the proposals and provides agreements to fix RMC for NR REFSENS requirements.

2. Common for FR1 and FR2

Possible agreement:

- K0 = 0 for all SCSs

- For K1 values:

  - Fixed K1 values in the specification only for testing purpose

  - Exact K1 values will be discussed and decided in the next July ad-hoc meeting

3. FR1

3.1. 15kHz SCS

Aligned LTE config #2 was already agreed at the last meeting, and the open issues are K0, K1 and # of HARQ process.

Proposals at RAN4 #87:

| Intel [3] | Align LTE special subframe #4 (D12/G1/U1); HARQ timing is as follows; HARQ process = 6. |
| --- | --- |

Possible agreements:

- Use the following TDD configuration for UE REFSENS requirements with FR1 15kHz SCS:

| RRC configurations |  | Agreements |
| --- | --- | --- |
| UL-DL-configuration-common | referenceSubcarrierSpacing | 15 kHz |
|  | dl-UL-TransmissionPeriodicity | ms5 |
|  | nrofDownlinkSlots | 3 |
|  | nrofDownlinkSymbols | 10 |
|  | nrofUplinkSlots | 1 |
|  | nrofUplinkSymbols | 2 |
| PDSCH-ServingCellConfig | nrofHARQ-ProcessesForPDSCH | 8 |
| PUCCH-Config | dl-DataToUL-ACK | K1 config #1 4, 3, 7, 6 |

- K1 configuration #1:

3.2. 30kHz SCS

Proposals at RAN4 #87:

| Intel [3] | Align LTE config #2 with D10/G2/U2; HARQ timing is as follows; HARQ process = 8. |
| --- | --- |
| DOCOMO[2] | Align LTE config #2 with D6/G4/U4; HARQ timing is as follows; HARQ process = 16; At least for EN-DC/NR CA band combinations that do not support simultaneous Tx/Rx |
| Huawei [4] | {DDDSU} as follows (S=D8/G4/U2); K1 belongs {2, 3, 4, 5, 6, 7, 8}; HARQ process = 16. A/N timing (K1 values) is unclear |
| Qualcomm [5][7] | For scenarios where NR does not need to be aligned with LTE slot pattern, use DSDU slot pattern {0,32,0,41} as follows: |
| Ericsson [6] | Align LTE config #2 {DDDSU}; HARQ process can be reduced e.g. 4 than max number; Other configuration can be introduced into UE demodulation requirements with phase approach; Special subframe configuration and A/N timing (K1 values) are unclear |

Summary of proposals:

| RRC configurations |  | Intel [3] | DOCOMO [2] | Huawei [4] | Qualcomm [5] [7] |  | Ericsson [6] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| UL-DL-configuration-common | referenceSubcarrierSpacing | 15 kHz | 30kHz | 30kHz | 30kHz |  | 30kHz |
|  | dl-UL-TransmissionPeriodicity | ms5 | ms5 | ms2p5 | ms1 | ms1 | ms5 |
|  | nrofDownlinkSlots | 3 | 7 | 3 | 1 | 0 | 7 |
|  | nrofDownlinkSymbols | 12 | 6 | 8-10 | 10 | 12 |  |
|  | nrofUplinkSlots | 1 | 2 | 1 | 0 | 1 | 2 |
|  | nrofUplinkSymbols | 1 | 4 | 2 | 2 | 0 |  |
| PDSCH-ServingCellConfig | nrofHARQ-ProcessesForPDSCH | 8 | 16 | 16 | 4 |  | e.g. 4 |
| PUCCH-Config | dl-DataToUL-ACK | K1 config #2 (see below) | K1 config #3 (see below) |  | K1 config #4 (see below) |  |  |
| Note |  |  | At least for EN-DC/NR CA band combinations that do not support simultaneous Tx/Rx |  | For UL-DL config common | For UL-DL config common2 | Other configuration can be introduced into UE demodulation requirements with phase approach |

- K1 configurations are as follows:

  - K1 configuration #2:

  - K1 configuration #3:

  - K1 configuration #4:

K1 = {8,7,6,5,4,3,2}

Possible agreements:

- Slot format = {DDDDDDDSUU}

  - Alt. 1. S = {D8, G4, U2}

  - Alt. 2. S = {D6, G4, U4}

- HARQ process:

  - Alt.1. 16 process

  - Alt.2. 8 process for REFSENS and 16 process will be discussed in UE demod test

3.3. 60kHz SCS (FR1)

Proposals at RAN4 #87:

| Intel [3] | Option 1: Aligned LTE config #0 with D6/G4/U4; HARQ timing is as follows; HARQ process = 8. Option 2: Same config with 30kHz SCS with half periodicity (2.5ms) |
| --- | --- |
| Ericsson [6] | Align LTE config #2 {DDDSU}; HARQ process can be reduced e.g. 4 than max number; Other configuration can be introduced into UE demodulation requirements with phase approach; Special subframe configuration and A/N timing (K1 values) are unclear |

Summary of proposals:

| RRC configurations |  | Intel [3] |  | Ericsson [6] |
| --- | --- | --- | --- | --- |
|  |  | Option 1 | Option2 |  |
| UL-DL-configuration-common | referenceSubcarrierSpacing | 15 kHz | 30 kHz | 60kHz? |
|  | dl-UL-TransmissionPeriodicity | ms5 | ms2p5 | ms5 |
|  | nrofDownlinkSlots | 1 | 3 | 15 |
|  | nrofDownlinkSymbols | 12 | 12 |  |
|  | nrofUplinkSlots | 1 | 1 | 4 |
|  | nrofUplinkSymbols | 3 | 1 |  |
| PDSCH-ServingCellConfig | nrofHARQ-ProcessesForPDSCH | 8 | 8 | e.g. 4 |
| PUCCH-Config | dl-DataToUL-ACK | K1 config #5 (see below) | Same as K1 config #2 for 30kHz SCS |  |
| Note |  |  |  | Other configuration can be introduced into UE demodulation requirements with phase approach |

- K1 configurations are as follows:

  - K1 configuration #5:

Possible agreements:

- Slot pattern

  - Alt.1 {DDDSU} with 1.25 ms if RAN2 spec is allowed.

  - Alt.2 {DDSU} with 1 ms if RAN2 spec does not allow Alt.1.

- {DDDSU} with 1.25 ms is agreed as working assumption

  - {DDSU} with 1 ms if Rel.15 RAN2 spec does not allow this configuration before July RAN4 ad-hoc meeting

- S = {D6, G4, U4}

- HARQ process is 8

4. FR2

4.1. 60kHz SCS (FR2)

Proposals at RAN4 #87:

| Intel [3] | {DDDSU} for 60kHz SCS to allow to configure up to 64 SSB. |
| --- | --- |
| Huawei [4] | {DDDSU} with S slot format: nrofDownlinkSymbols = 8~10, nrofUplinkSymbols = 2, 1.25ms periodicity |
| Ericsson [6] | Use DDDSU to align with LTE TDD UL/DL config 2; HARQ process can be reduced e.g. 4 than max number. Special subframe configuration and A/N timing (K1 values) are unclear |
| Qualcomm | DDSU |

Summary of proposals:

| RRC configurations |  | Intel [3] | Huawei [4] | Ericsson [6] |
| --- | --- | --- | --- | --- |
| UL-DL-configuration-common | referenceSubcarrierSpacing | 60 kHz | 60kHz | 60kHz |
|  | dl-UL-TransmissionPeriodicity | ms1p25 | ms1p25 | ms1p25? |
|  | nrofDownlinkSlots | 3 | 3 | 3 |
|  | nrofDownlinkSymbols | 12 | 8-10 |  |
|  | nrofUplinkSlots | 1 | 1 | 1 |
|  | nrofUplinkSymbols | 1 | 2 |  |
| PDSCH-ServingCellConfig | nrofHARQ-ProcessesForPDSCH | 6 | 16 | e.g. 4 |
| PUCCH-Config | dl-DataToUL-ACK | Same as K1 config #1 for 15kHz SCS |  |  |

Possible agreements:

- Slot format

  - Alt. 1{DDDSU} for FR2 60kHz SCS

    - Opt.1: S ={D4, G6, U4}

    - Opt.2: S ={D13, G1}

  - Alt. 2 {DDSU}

- Number of HARQ

  - Opt.1: 8

  - Opt.2: 6

4.2. 120kHz SCS

Proposals at RAN4 #87:

| Intel [3] | {DDDDDDDSUU} for 120kHz SCS to allow to configure up to 64 SSB. |
| --- | --- |
| DOCOMO [2] | {DSDSU} as follows; HARQ process = 16. 0.625ms periodicity, Mini-slot based scheduling (DL: 4 symbol slot, UL: 2 symbol slot) K1=6 should be tested on top of proposal 5 in order to maximize spectrum efficiency during SSB burst. |
| Huawei [4] | {DDDSU} with S slot format: nrofDownlinkSymbols = 8~10, nrofUplinkSymbols = 2, 0.625ms periodicity A/N timing (K1 values) is unclear |
| Qualcomm [5] [7] | {DDSU} as follows. A/N timing (K1 values) and # of HARQ process are unclear |
| Ericsson [6] | Use DDDSU to align with LTE TDD UL/DL config 2; HARQ process can be reduced e.g. 4 than max number. Special subframe configuration and A/N timing (K1 values) are unclear |

Summary of proposals:

| RRC configurations |  | Intel [3] | DOCOMO [2] | Huawei [4] | Qualcomm [5] | Ericsson [6] |
| --- | --- | --- | --- | --- | --- | --- |
| UL-DL-configuration-common | referenceSubcarrierSpacing | 60 kHz | 120 kHz | 120kHz | 120KHz | 120kHz |
|  | dl-UL-TransmissionPeriodicity | ms1p25 | ms0p625 | ms0p625 | ms0p5 | ms0p625? |
|  | nrofDownlinkSlots | 3 | 1 | 3 | 2 | 3? |
|  | nrofDownlinkSymbols | 12 | 0 | 8-10 | 12 |  |
|  | nrofUplinkSlots | 1 | 1 | 1 | 1 | 1? |
|  | nrofUplinkSymbols | 1 | 0 | 2 | 0 |  |
| PDSCH-ServingCellConfig | nrofHARQ-ProcessesForPDSCH | 8 | 16 | 16 |  | e.g. 4 |
| PUCCH-Config | dl-DataToUL-ACK | Same as K1 config #2 for 30kHz SCS | K1 config #6 and #7 (see below) |  |  |  |
| Note |  |  | Need to configure UL DL configuration dedicated |  |  | For UE performance tests to use TDD config based on operators input |

- K1 configurations are as follows:

  - K1 configuration #6:

  - K1 configuration #7:

Possible agreements:

Slot pattern:

  - {DDDSU}

    - Opt.1: S ={D4, G6, U4}

    - Opt.2: S ={D12, G2}

    - Opt.3: S ={D10, G2, U2}

    - Opt.4: S ={D11, G2, U1}

  - Other configuration will be discussed in UE demodulation requirements

- Number of HARQ process

  - Opt.1: 8

  - Opt.2: 16

5. Agreements (summary):

Note: Agreed / not agreed

Common for FR1 and FR2

- K0 = 0 for all SCSs

- For K1 values:

  - Fixed K1 values in the specification only for testing purpose

  - Exact K1 values will be discussed and decided in the next July ad-hoc meeting

For 15kHz SCS in FR1:

| RRC configurations |  | Agreements |
| --- | --- | --- |
| UL-DL-configuration-common | referenceSubcarrierSpacing | 15 kHz |
|  | dl-UL-TransmissionPeriodicity | ms5 |
|  | nrofDownlinkSlots | 3 |
|  | nrofDownlinkSymbols | 10 |
|  | nrofUplinkSlots | 1 |
|  | nrofUplinkSymbols | 2 |
| PDSCH-ServingCellConfig | nrofHARQ-ProcessesForPDSCH | 8 |

For 30kHz SCS in FR1

- Slot format = {DDDDDDDSUU}

  - S = {D6, G4, U4}

- HARQ process:

  - Alt.1. 16 process

  - Alt.2. 8 process for REFSENS and 16 process will be discussed in UE demod test

For 60kHz SCS in FR1

- {DDDSU} with 1.25 ms is agreed as working assumption

  - {DDSU} with 1 ms if Rel.15 RAN2 spec does not allow this configuration before July RAN4 ad-hoc meeting

- S = {D6, G4, U4}

- HARQ process is 8

For 60kHz SCS in FR2

- Slot format = {DDDSU}

  - S ={D4, G6, U4}

- Number of HARQ

  - 8

For 120kHz SCS in FR2

- Slot pattern = {DDDSU}

  - S ={D10, G2, U2}

- Other configuration will be discussed in UE demodulation requirements

- Number of HARQ process

  - Opt.1: 8

  - Opt.2: 16

References

- R4-1805717　WF on TDD configuration for NR UE REFSENS　NTT DOCOMO

- R4-1806200	TDD configuration for REFSENS requirements 	DOCOMO Communications Lab.

- R4-1806281	NR UE REFSENS SNR	Intel Corporation

- R4-1807915	Discussion on TDD configuration for NR UE REFSENS	Huawei, HiSilicon

- R4-1806631	Slot pattern for TDD REFSENS tests	Qualcomm Incorporated

- R4-1806119	TDD UL/DL configuration for UE REFSENS and performance tests	Ericsson

- R4-1806365	TDD Slot Patterns for Demodulation Performance Tests	Qualcomm Incorporated