3GPP TSG-RAN WG4 Meeting #87	R4-1808175

Busan, Korea, May 21 – 25, 2018

Source: 	Intel

Title: 	Ad-Hoc Minutes for FR2 EIRP Spherical Coverage

Agenda Item:	7.5.8.2

Document for:	Discussion

# Spherical Coverage

## Contributions list

| # | Tdoc number | Source | Title | Type | For |
| --- | --- | --- | --- | --- | --- |
| 1 | R4-1806352 | Apple Inc., Intel Corporation | Network Performance Analysis for Spherical Coverage | discussion | Discussion |
| 2 | R4-1806435 | Samsung | Network performance analysis for spherical coverage | discussion | Discussion |
| 3 | R4-1807658 | Qualcomm Incorporated | Why spherical coverage matters | other | Approval |
| 4 | R4-1806727 | Ericsson | Spherical coverage requirement of different types for EN-DC | other | Approval |
| 5 | R4-1806728 | Ericsson | Introduction of additional UE spherical coverage requirements for EN-DC | draftCR | Endorsement |
| 6 | R4-1806183 | Apple Inc., Intel Corporation, vivo, Xiaomi, OPPO, SGS Wireless, TCL, Spreadtrum, Asus, Kyocera | Proposals for concluding the spherical coverage requirement for FR2 handheld UEs | other | Approval |
| 7 | R4-1806184 | Apple Inc., Intel Corporation, vivo, Xiaomi, OPPO, SGS Wireless, TCL, Spreadtrum, Asus, Kyocera | Draft CR to TS38.101-2 on the power class requirement for FR2 handheld UEs | draftCR | Endorsement |
| 8 | R4-1806185 | Apple Inc. | EIRP CDF results for FR2 handheld UEs | discussion | Discussion |
| 9 | R4-1806419 | Samsung | EIRP values for spherical coverage in FR2 | discussion | Approval |
| 10 | R4-1806676 | LG Electronics France | Measurement results of spherical EIRP CDF curves for NR UE at FR2 | other | Approval |
| 11 | R4-1806814 | MediaTek Beijing Inc. | Spherical Coverage Analysis | discussion | Approval |
| 12 | R4-1807490 | Sony | UE Spherical coverage measurements at mmWave 28GHz | discussion | Discussion |
| 13 | R4-1807807 | Huawei Device Co., Ltd | On spherical coverage requirement of EIRP | other | Approval |
| 14 | R4-1806663 | Qualcomm Incorporated | FR2 Type 1 UE EIS and EIRP Spherical Coverage | other | Approval |
| 15 | R4-1807931 | Motorola Mobility Germany GmbH | Spherical Coverage for FR2 | discussion | Discussion |

## Proposals summary

| Source | Proposals |
| --- | --- |
| Apple, Intel | R4-1806352 Observation 1: Due to handgrip and body blockage, it is reasonable to assume that only one of the antenna panels of the UE is active at a time. Observation 2: In the UMa scenario when ISD is 200 m and all users are outdoor, the outage performance of one and two panel UEs is comparable. Observation 3: In the UMa scenario when ISD is 200 m, the difference in the mean UL throughput loss when UEs have realistic single and two panels is about 7%. Furthermore, the mean DL throughput performance is comparable. Observation 4: In the dense urban scenario, the throughput performance when UEs have realistic single and two panels is quite comparable. Observation 5: Different “continuous” CDF degradation methods provide varied results but even the linear method (arguably the median CDF between the 20%-tile and 50%-tile degradation points) increases the mean and 5%-tile throughput loss by less than 5% compared to the discontinuous approach. Observation 6: The linear CDF degradation at 20%-tile increases the average UE throughput loss by less than 8% compared to the 20%-tile discontinuous degradation while the discontinuous degradation of CDF at 50%-tile results in largest throughput loss. Observation 7: The network performance is less sensitive to the variations of EIRP CDF at 20%-tile value. Proposal 1: A single panel UE is a viable option and can be considered as the baseline for the spherical coverage requirement. Proposal 2: The spherical coverage requirement should be specified at not smaller than 50%-tile value. |
| Samsung | R4-1806435 Results for urban macro with ISD of 200m and 200MHz bandwidth Deviation from a ref 13.8dBm (-8.6 dBm from peak); UL throughput change wrt that for 0dB deviation. Mean; 5%-tile. 50%-tile; -3dB; -6.17%; -30.94%. -2dB; -4.42%; -18.77%. 0dB; 0%; 0%. +2dB; 4.79%; 20.66%. +3dB; 7.23%; 27.88%. Observation: With deviation of -3dB to +3dB at the 50%-tile of EIRP CDF, the mean UL throughput change ranges from -6.17% to 7.23%, whereas the 5%-tile UL throughput change ranges from -30.94% to 27.88%. The change in the mean throughput is significantly less compared to that in the 5%-tile throughput. |
| Qualcomm | R4-1807658 - Since the network needs to be designed based on a target cell edge coverage, the low percentile points are more important than the 50%-tile - CDF shape has large impact on the reliability achieved at a given distance from BS - Low percentile points determine the size of cell edge - Defining a spherical coverage percentile point below 50% is important for reliability of RRM testing and performance Proposal 1: define a spherical coverage requirement corresponding to at least two antenna modules Proposal 2: define a CDF percentile point below 50% (e.g. 20%) |
| Ericsson | R4-1806727 To facilitate development of early mmW UEs, we propose that two sets of spherical coverage requirements be specified in the Rel-15 version of 38.101.3: - A mandatory type 1 requirement that can be met using one module - An optional enhanced type 2 requirement for designs with e.g. improved antenna/module performance. The type 2 requirement would be the same as that applicable to SA operation. R4-1806728 Accompanying draft CR to include table for spherical coverage requirements in TS 38.101-3. |
| Apple, Intel | R4-1806185 Observation 1: Because Model 1 implements a partial display panel and represents a result for comparative purposes only, Model 1 results cannot be used to determine the spherical coverage requirement for FR2 handheld UEs. Observation 2: The result for Model 2 (A) corresponds to Assumption 1 with full display panel. Observation 3: The result for Model 2 (A+B) corresponds to Assumption 3 with full display panel and Back/Back antenna panel locations. Observation 4: The result for Model 3 corresponds to Assumption 1 with full display panel. Observation 5: The gap between simulation and measurement, as reported in [12], is between 1.5 and 1.7 dB. Observation 6: The first step toward improving the confidence in our simulated results’ prediction of the EIRP CDF performance has been taken with results showing alignment within 0.5 dB. Observation 7: The measurement results raise a concern with the efficacy of setting a potential requirement on any percentile below the 50th, since it becomes difficult to separate actual UE performance at these percentiles from any test system related effects, such as DUT fixturing. It is not recommended to consider defining an EIRP CDF requirement for any percentiles below the 50th. Observation 8: The intra-band variation of peak EIRP is 2.0 dB for 28 GHz and 2.5 dB for 39 GHz. Observation 9: Over the 28 GHz frequency range, performance at the 50%-tile CDF has a spread of 2.8 dB compared to the 2.0 dB assumption for peak EIRP. Observation 10: Over the 39 GHz frequency range, performance at the 50%-tile CDF has a spread of 4.6 dB compared to the 2.5 dB assumption for EIRP. Observation 11: Based on the above observations, intra-band variation of the CDF exceeds the variation of the peak by a margin of 0.8 dB for 28 GHz and by a margin of 2.1 dB for 39 Ghz. Observation 12: Because no market-ready devices are available to derive the spherical coverage CDF requirements based on the joint band passing rate (JBPR) framework [10], the derivation of band-specific requirements based on simulation results should consider the most common set of baseline assumptions (Assumption 1 with full display panel) together with additional factors accounting for intra-band variation of performance and the simulation vs. measurement margin. Observation 13: We suggest to define the spherical coverage requirement at not smaller than 50%-tile value. Proposal 1: Companies are encouraged to follow the data-driven approach described in [14] to derive the spherical coverage requirement during the RAN4 #87 meeting. From the perspective of a company who has completed the simulation and measurement aspects of the work plan, it is our understanding that the requirement can be defined according to the proposals prepared in [14]. In the event of strong disagreement with these proposals, it is also acceptable to completely postpone the effort on spherical coverage until commercial devices are available for a measurement campaign. |
| Apple, Intel, vivo, Xiaomi, OPPO, SGS Wireless, TCL, Spreadtrum, Asus, Kyocera | R4-1806183 Proposal 1: Based on the applicability of EIRP CDF simulation assumptions, the spherical coverage requirement is defined by assuming the following baseline architecture for the UE: - Front/Back/Side cover materials are Glass/Glass/Metal - No antenna elements in the outer edge of the UE - Full display panel - Single array Proposal 2: Based on the analysis in [8], intra-band variation of the CDF exceeds the variation of the peak by a margin of 0.8 dB for 28 GHz and by a margin of 2.1 dB for 39 GHz. Proposal 3: The gap between simulation and measurement, as reported in [13], is between 1.5 and 1.7 dB and is included in the spherical coverage requirement derivation as the simulation vs. measurement margin. Proposal 4: With a preference to Option 2, the proposal for the power class requirements for 28 GHz is - Peak EIRP = 22.5 dBm - EIRP 50%-tile CDF = 8.0 dBm Proposal 5: With a preference to Option 2, the proposal for the power class requirements for 39 GHz is - Peak EIRP = 20.5 dBm - EIRP 50%-tile CDF = 2.5 dBm Proposal 6: Based on the related observations in [6], a single panel UE is a viable option and can be considered as the baseline for the spherical coverage requirement. Proposal 7: The spherical coverage requirement is defined at not smaller than 50%-tile value. R4-1806184 Accompanying draft CR updating min peak EIRP values and adding spherical coverage table. |
| Samsung | R4-1806419 Observation 1: Observations about the impact of full display and metal side cover should be taken into account in defining the requirements for 50%-tile. Observation 2: For 28GHz, the simulation results submitted until RAN4#86bis are in the range of 5.5 to 14 dB down from the peak at 50%-tile. The mean value is 10.9 dB down from the peak. Observation 3: For 39GHz, our EIRP value at 50%-tile point is 10.6 dB down from the peak. Observation 4: The 50%-tile requirement can be defined with a margin of 3 dB on top of simulation data, which is 11.6 dB down from the peak, or 10.8 dBm, for our simulation data of 28 GHz. Proposal 1: For 28 GHz, RAN4 is recommended to define the EIRP for spherical coverage to be 10.8 dBm at 50%-tile, 11.6 dB down from the peak. Proposal 2: For 39 GHz, RAN4 is recommended to define the EIRP for spherical coverage to be 7 dBm at 50%-tile, 13.6 dB down from the peak. |
| LGE | R4-1806676 Simulated vs measured results Simulation basis; Measurement basis. Peak EIRP; 22.4 dBm; 22.4dBm. Spherical EIRP (50%); 22.4 -12.5 = 9.9dBm; 22.4 -13.9 = 8.5 dBm. Spherical EIRP requirements Required EIRP (at n257/n258); Required EIRP (at n260). Peak EIRP; 22.4 dBm; 20.6dBm. Spherical EIRP (50%); 22.4-12.5-1.5 =8.4 dBm; 20.6-13.9-1.5 = 5.2 dBm. Proposal 1: For the spherical coverage of power class at mmWave, RAN4 should specified the 50%-tile spherical coverage as shown in Table 1 with considering 1.5dB measurement vs. simulation power offset. |
| MediaTek | R4-1806814 50%-tile EIRP specification calculation Operating Band; 28 GHz; 39 GHz. Peak EIRP (dBm); 22.4; 20.6. Normalized 50%-tile EIRP (dB); -11.6; [-11.6]. Calculated 50%-tile EIRP (dBm); 10.8; [9]. Proposed 50%-tile EIRP (dBm); 11; [9]. Proposal 1: 50%-tile requirement for EIRP CDF in 28 GHz band is 11 dBm. Proposal 2: 50%-tile requirement for EIRP CDF in 39 GHz band is [9] dBm. |
| Sony | R4-1807490 Summary of measured CDF spherical performance Assumption; -; 2-modules; Back-side only; Front-side only. Display; Partial / Full; Partial. No antenna arrays; Back side; 1; 1; N/A. Front side; 1; N/A; 1. Side cover (cover material near antennas); Metal / Plastic; Plastic; Plastic; Plastic. Back cover; Glass / Plastic; Plastic; Plastic; N/A. Front cover; Glass / Plastic; Glass; N/A; Glass. EIRP @ 100%%-tile point; dBm; 22.4dBm (agreement in #86bis [4]). EIRP @ 50%; dB; 9.1; 12.5; 13.1. EIRP @ 40%; dB; 10.1; 14.2; 14.7. EIRP @ 30%; dB; 11.1; 16.2; 17.9. EIRP @ 20%; dB; 12; 19.1; 21. Observation 1: The gap between a single and two modules is bigger at the lower CDF points. Observation 2: Implementation loss due to high-loss glass (Front-side) is 1 – 2 dB higher compared to plastic (Back-side) cover material which is consistent with simulations. |
| Huawei | R4-1807807 Observation: Due to the difference in test methodology between peak EIRP and spherical coverage, it is reasonable for UE vendors to consider different MU and TT when providing feasibility analysis on 50%-tile EIRP as compared to peak EIRP. Proposal 1: 50%-tile EIRP requirement for band n257, n258 and n261 should be 10.7 dBm (-11.7 dBm) in Rel. 15. Proposal 2: 50%-tile EIRP requirement for band n260 should be further discussed and specified in Rel.16. |
| Qualcomm | R4-1806663 Proposal 1: FR2 Type 1 UEs shall have independent EIS and EIRP spherical coverage requirements. Proposal 2: For a UE with 2 modules, spherical coverage CDF droop at the 50%ile point, relative to peak value, shall be less than 8dB. Proposal 3: Spherical coverage specification additionally at [20]%ile CDF is FFS |
| Motorola | R4-1807931 - Summarizing our choice of assumptions: - Two 1x4 arrays - Full display panel - Front/Back/Side housing materials are primarily glass/glass/metal with some compromise - Arrays partially intrude into the outer edge - Additional de-rating factors to get a maximum rather than nominal delta-dB value - Intra-band variation (variation in the delta-dB, aside from variation in the absolute peak EIRP value): not fully characterized, estimated as 1.5 dB. - Measurement vs. simulation margin: our view is that this is a first-order factor for absolute values like peak EIRP, but not a significant factor for the delta-dB value here in question, and estimate this as 0 dB. - Finite number of beams penalty: the above data are based on 5 degree incrementing of beam angles. We can estimate the impact of limiting to a smaller number of beams on 30 degree increments by inspecting the pattern taper (in the relevant plane) of the formed beams at +/-15 degrees from the nominal pointing direction. We estimate this to contribute an additional 1.5 dB to the raw delta-dB value. Proposal: The following delta-dB values be subtracted from the minimum peak EIRP values previously agreed, to establish the 50th percentile EIRP values: - 7.4 dB in 28 GHz - 8.9 dB in 39 GHz |

From Monday’s AH session:

Agree to discuss

- A mandatory Type 1 requirement that can be met using one module

- An optional enhanced Type 2 requirement for designs with e.g. improved antenna/module performance.

## Discussion Topics:

[Qualcomm] we believe the lower percentile will be beneficial

[Qualcomm] mandatory should be based on 2 panels

Focus will be the 50%-tile point

## Topic 1: Mandatory Type 1

- Defining mandatory Type 1

  - Can we define mandatory as one panel?

No consensus was reached on whether it should be one panel, therefore two options are considered:

- Option 1: derive with 1 panel data (Table 1)

- Option 2: derive with 2 panel data (Table 2)

## Topic 2: How do we define each option to finalize the 50%-tile requirement?

- There is a variety of assumptions for the reported data and some is based of simulations, while a few are based on measurements. To help analyze the data, two tables are provided below summarizing the 50%-tile values for 1 panel and 2 panels.

No consensus was reached on whether mandatory should be one panel, therefore two options are considered. Each table will be treated separately.

Table 1 - Summary of 50%-tile values – 1 panel

| Source | Data type | 28GHz Drop ¦ EIRP |  | 39GHz Drop ¦ EIRP |  | Simulation assumptions | Additional margins (raw data adjusted) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Apple | Simulated | -14.50 | 8.00 | -18.0 | 2.5 | Co-located, dual band, full display, metal case | Yes |
| LGE | Measured | -14.00 | 8.40 | -15.4 | 5.2 | Co-located, dual-band, full display, metal case | Yes |
| Samsung | Simulated | -11.60 | 10.8 | -13.6 | 7.0 | Co-located, dual-band, full display, metal case | Yes |
| Sony | Measured/Simulated | -13.10 | 9.30 | -12.8 | 7.8 | Co-located, dual-band, partial display, metal case |  |
| Huawei | Simulated | -11.70 | 10.7 | - | - | Co-located, single band, full display, metal case | Yes |
| Average |  | -12.98 | 9.44 | [-14.95] | [5.63] |  |  |

- Some margins taken by companies include simulation vs measurement variation and intra-band variation

- Simulation assumptions and updates need to be reviewed (highlighted)

- Averages for 28GHz data are stable

[Chair] Since there are only 3 data points, how do we proceed with 39GHz?

[Apple] There is a spread in the 39GHz data, we need to align on what was simulated and considered. Colocation? Intra-band variation?

[Samsung] We are aligned in our assumptions

[LGE] We did not consider all

[Chair] LGE, would you adjust your data for this?

[LGE] We would like to further discuss with Apple offline. From the development part feedback, we consider dual band and colocation. We aligned these assumptions with other companies, so prefer one level not to range.

[Chair] Average values for 39GHz data will be kept in brackets with current range until the outcome of that discussion. The

Table 2 - Summary of 50%-tile values – 2 panels

| Source | Data type | 28GHz Drop ¦ EIRP |  | 39GHz Drop ¦ EIRP |  | Simulation assumptions | Additional margins (raw data adjusted) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Apple | Simulated | -13.0 | 9.5 | -16.5 | 4.0 | Co-located, dual-band, full display, metal case | Yes |
| Qualcomm | Simulated | -8.00 | 14.4 | -10.0 | 10.6 | Co-located, single-band, partial display, glass case | Yes |
| MediaTek | Measured | -11.6 | 10.8 | -11.6 | 9.0 | Co-located, dual-band, full display, metal case | Yes |
| Sony | Measured/Simulated | -9.10 | 13.3 | -9.20 | 11.4 | Co-located, dual-band, partial display, metal case |  |
| Samsung | Simulated | -10.0 | 12.4 | -12.2 | 8.4 | Co-located, dual-band, full display, metal case | Yes |
| Motorola | Simulated/Estimated | -7.40 | 15.0 | [-8.90] | [11.7] | Co-located, dual-band, full display, mostly metal case | Yes |
| Huawei | Simulated | -10.3 | 12.1 | - | - | Co-located, single band, full display, metal case | Yes |
| LGE | Simulated | -12.2 | 10.2 | - | - | Co-located, dual-band, full display, metal case | Yes |
| DOCOMO | Simulated | -9.0 | 13.4 | - | - | Single-band, full display, metal case |  |
| Average |  | -10.1 | 12.3 | [-11.4] | [9.2] |  |  |

Averages for 28GHz data are stable once new inputs are review. How do we proceed with 39GHz data?

Motorola agreed to have 39GHz data in brackets, since these are not based on simulations (based on estimations).

Companies to provide data TBD by tomorrow afternoon. Technical justification of what was simulated and how it was simulated needs to be included with the data and shared on RAN4 reflector, otherwise the numbers will not be included in the final table.

Agreements:

- No consensus was reached on whether mandatory should be one or two panels, therefore discussion will continue with two options considered.

  - Option 1: derive with 1 panel data (Table 1)

    - 28GHz: 12.98 dB drop, peak EIRP = 9.44 dBm

    - 39GHz: [-18 to -13.6] drop, [2.5 to 7.0] peak EIRP  pending confirmation

  - Option 2: derive with 2 panel data (Table 2)

    - 28GHz: 10.1 dB drop, peak EIRP = 12.3 dBm

    - 39GHz: [X]  pending updates

Evaluation of data updates

- Majority of available data is simulated

- Data for 28GHz is more stable than 39GHz

- The spread for Table 1 (for 1 panel):

  - 28GHz = 2.9 dB

  - 39GHz = 5.2 dB

- The spread for Table 2 (for 2 panel):

  - 28GHz = 5.6 dB

  - 39GHz = 7.6 dB

Questions to finalize data and options

[Q1] Table 1 - 39GHz: are we OK to take the average number? Taking the average means:

- 15 dB drop and min peak EIRP of 5.6 dB

[Q2] Table 2 - 39GHz: highlighted data needs confirmation

- Updates provided by Qualcomm, MediaTek, Samsung, DOCOMO, and Sony  review assumptions

- Motorola data  based on estimation, not simulation

[Q3] Can we confirm the data in Table 1 as stable? If so, can we agree that Option 1 will be based on the data average?

- Option 1 defines 50%-tile as:

  - 28 GHz: 13 dB drop from peak, minimum peak EIRP = 9.4 dBm

  - 39 GHz: 15 dB drop from peak, minimum peak EIRP = 5.6 dBm

[Q4] Can we confirm the data in Table 2 as stable? If so, can we agree that Option 2 will be based on the data average?

- Option 2 defines 50%-tile as:

  - 28 GHz: 10.1 dB drop from peak, minimum peak EIRP = 12.3 dBm

  - 39 GHz: 11.4 dB drop from peak, minimum peak EIRP =   9.2 dBm

| Source | # panels | 28GHz 50%-tile EIRP | 39GHz 50%-tile EIRP | Assumptions |  |  |  | 28 GHz Freq. Range* | 39 GHz Freq. Range* |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Co-located | Multi-band | Display | Case |  |  |
| Apple | 1 | 8 | 2.5 | Y | Y | Full | Metal | 26 - 30 | 37 - 43 |
| LGE |  | 8.4 | 5.2 | Y | Y | Full | Metal |  |  |
| Samsung |  | 10.8 | 7 | Y | Y | Full | Metal |  |  |
| Sony |  | 9.3 | 7.8 | Y | Y | Partial | Metal | 26.5 - 29.5 |  |
| Huawei |  | 10.7 | - | Y | Single | Full | Metal |  |  |
| Apple | 2 | 9.5 | 4 | Y | Y | Full | Metal | 26 – 30 | 37 - 43 |
| Qualcomm |  | 14.4 | 10.6 | Y | Single | Partial | Glass | target band | target band |
| MediaTek |  | 10.8 | 9 | Y | Y | Full | Metal | 24.5 – 29.5 | 37 - 40 |
| Sony |  | 13.3 | 11.4 | Y | Y | Partial | Metal |  |  |
| Samsung |  | 12.4 | 8.4 | Y | Y | Full | Metal |  |  |
| Motorola |  | 15 | [11.7] | Y | Y | Full | Mostly metal |  |  |
| Huawei |  | 12.2 | - | Y | Single | Partial | Metal |  |  |
| LGE |  | 10.2 | - | Y | Y | Full | Metal |  |  |
| DOCOMO |  | 13.4 | - | N | Single | Full | Metal |  |  |

*Note: Bandwidth (frequency range) of simulations needs to be verified