3GPP TSG-RAN WG4 Meeting #87	R4-1808333

Busan, Korea, 21 – 25 May 2018

Agenda Item:	7.8.4.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	TP to TR 38.141-2: Clarifications on OTA sensitivity requirement (7.2.1, 7.3.1)

Document for:	Approval

# 1.	Introduction

For BS capable of redirecting the receiver target in FR1, it is stated in TS 38.104 [1] and TS 38.141-2 [2] as a normative requirement that the OSDD shall include ‘The set(s) of RAT, Channel bandwidth and declared minimum EIS level applicable to all sensitivity RoAoA in the OSDD’. Hence, it may be interpreted that the declared minimum EIS level is also applicable to the non-active sensitivity RoAoA inside the receiver target redirection range, i.e. the declared minimum EIS level shall also be met even when the receiver target is set towards another sensitivity RoAoA. Although there is an informative note stating: ‘The declared OTA sensitivity shall apply only to the active sensitivity RoAoA inside the receiver target redirection range.’, it cannot be used to override a normative requirement. Moreover, ‘polarization match’ is defined in the Definitions clause, but ‘polarization matching’ is used in the requirement text.

This contribution provides a text proposal to clarify in TS 38.141-2 that for BS capable of redirecting the receiver target in FR1, the declared minimum EIS level is applicable to any active sensitivity RoAoA inside the receiver target redirection range in the OSDD, and use ‘polarization match’ in the requirement text.

# 2.	Text proposal

<Start of change>

### 7.2.1	Definition and applicability

The OTA sensitivity requirement is based upon the declaration of one or more OTA sensitivity direction declarations (OSDD), related to a BS type 1-H and BS type 1-O receiver.

The BS type 1-H and BS type 1-O receiver may optionally be capable of redirecting/changing the receiver target by means of adjusting BS settings resulting in multiple sensitivity RoAoA. The sensitivity RoAoA resulting from the current AAS BS settings is the active sensitivity RoAoA.

If the AAS BS is capable of redirecting the receiver target related to the OSDD then the OSDD shall include:

-	BS channel bandwidth and declared minimum EIS level applicable to any active sensitivity RoAoA inside the receiver target redirection range in the OSDD.

-	A declared receiver target redirection range, describing all the angles of arrival that can be addressed for the OSDD through alternative settings in the BS.

-	Five declared sensitivity RoAoA comprising the conformance testing directions as detailed in [x].

-	The receiver target reference direction.

NOTE 1:	Some of the declared sensitivity RoAoA may coincide depending on the redirection capability.

NOTE 2:	In addition to the declared sensitivity RoAoA, several sensitivity RoAoA may be implicitly defined by the receiver target redirection range without being explicitly declared in the OSDD.

If the BS is not capable of redirecting the receiver target related to the OSDD, then the OSDD includes only:

-	BS channel bandwidth and declared minimum EIS level applicable to the sensitivity RoAoA in the OSDD.

-	One declared active sensitivity RoAoA.

-	The receiver target reference direction.

NOTE 3:	For BS without target redirection capability, the declared (fixed) sensitivity RoAoA is always the active sensitivity RoAoA.

The OTA sensitivity EIS level declaration shall apply to each supported polarization, under the assumption of polarization match.

<Next change>

### 7.3.1	Definition and applicability

The OTA REFSENS requirement is a directional requirement and is intended to ensure the minimum OTA reference sensitivity level for a declared OTA REFSENS RoAoA. The OTA reference sensitivity power level EISREFSENS is the minimum mean power received at the RIB at which a reference performance requirement shall be met for a specified reference measurement channel. The OTA REFSENS requirement shall apply to each supported polarization, under the assumption of polarization match.

<End of change>

# References

[1]	3GPP TR 38.104 v15.1.0, “NR; Base Station (BS) radio transmission and reception”.

[2]	R4-180xxxx, “TS 38.141-2 v0.1.0”, Huawei.