3GPP TSG-RAN WG4 Meeting #87															R4-1808121

Busan, Korea (Republic of), 21 - 25 May 2018

Source: 	Nokia, Nokia Shanghai Bell

Title: 					Intraband EN-DC AH Agenda

Agenda Item: 			7.2.3.10

Document for:	Information

# Agenda

## n41 A-MPR

Proposal: A-MPR = max (MPR, AMPR’)

Note that AMPR’ = 0dB means only MPR is applied
Given the parameters defined in Table 1 and symbol definitions in Table 2,

if RBstart ≤ fstart,max,IMD3 / (12SCS)
and LCRB ≤ AWmax,IMD3 / (12SCS)
and FC - BWChannel/2 < FUL_low + offsetIMD3,
then

the AMPR’ is defined according to Table 3,

else, if RBstart ≤ LCRB/2 + start / (12SCS)
and LCRB ≤  AWmax,regrowth / (12SCS)
and FC - BWChannel/2 < FUL_low + offsetregrowth,
then

the AMPR’ is defined according to Table 3,

else,

only AMPR’ = 0dB and apply MPR.

Table 2. Parameters for region edges and frequency offsets [1] with some exceptions

| Parameter | Symbol | Value |  |  | Related condition |
| --- | --- | --- | --- | --- | --- |
|  |  | OFDM | DFT-S-OFDM |  |  |
| Max allocation start in IMD3 region | fstart,max,IMD3 | 0.33 BWChannel |  |  | RBstart ≤ fstart,max,IMD3 / (12SCS) |
| Max allocation BW in IMD3 region | AWmax,IMD3 | 8 MHz |  |  | LCRB ≤ AWmax,IMD3 / (12SCS) |
| Max freq. offset for IMD3 region | offsetmax,IMD3 | [BWChannel – 6 MHz] |  |  |  |
| Freq. offset required to avoid A-MPR in IMD3 region | offsetIMD3 | offsetmax,IMD3 |  |  | FC - BWChannel/2 ≥ FUL_low + offsetIMD3 |
| Right edge of regrowth region | start | [0.08 BWChannel] |  |  | RBstart ≤ LCRB/2 + start / (12SCS) |
| Max allocation BW in regrowth region | AWmax,regrowth | 100 MHz |  |  | LCRB ≤ AWmax,regrowth / (12SCS) |
| Freq. offset required to avoid A-MPR in regrowth region | offsetregrowth | Max (10MHz, 0.25* BWChannel MHz] |  | Max (10MHz, 0.45* BWChannel MHz] | FC - BWChannel/2 ≥ FUL_low + offsetregrowth |

Table 2. Symbols adopted from 38.101 and 36.101

| Symbol | Definition |
| --- | --- |
| BWChannel | Channel bandwidth |
| FC | Centre frequency of channel |
| FUL_low | The lower edge frequency of the uplink operating band |
| SCS | Subcarrier spacing |
| RBstart | Index of lowest allocated resource block; RBstart=0 is the lowest RB in the channel. |
| LCRB | Number of contiguously allocated resource blocks |

Table 3. A-MPR’ values

| Access | Modulation | A-MPR' [dB] |
| --- | --- | --- |
| DFT-S-OFDM | pi/2-BPSK | 3.5 |
|  | QPSK | 4 |
|  | 16-QAM | 4 |
|  |  | 64-QAM |  | 4 |  |
|  |  | 256-QAM |  | 4.5 |  |
| CP-OFDM | QPSK | 5.5 |
|  | 16-QAM | 5.5 |
|  | 64-QAM | 5.5 |
|  | 256-QAM | 6.5 |

Table 4. Example Frequency Offset Values in Table Format

| BW, MHz | 10 | 15 | 20 |  | 40 | 50 | 60 | 80 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offsetIMD3, MHz | [4.5] | [9.5] | [14.5] |  | [34.5] | [44.5] | [54.5] | [74.5] | [94.5] |
| CP-OFDM offsetregrowth, MHz | [10] | [10] | [10] |  | [10] | [12.5] | [15] | [20] | [25] |
| DFT-s--OFDM offsetregrowth, MHz | [10] | [10] | [10] |  | [18] | [22.5] | [27] | [36] | [45] |

Discussion:

Chair: Can we agree that n41 A-MPR is defined as presented in this agenda?

No objection, A- MPR scheme is agreed.

## A-MPR for DC_(n)71B

Following contributions have been submitted to this meeting.

- R4-1806809	A-MPR for DC_(n)71B	Intel Corporation

- R4-1806979	A-MPR for DC_(n)71B	Nokia

- R4-1806668	Required A-MPR analysis for EN-DC_(n)71B UE	LG Electronics France

- R4-1808058	[NR] Band 71 Intra-Band EN-DC and B12 and B29 Protection	Skyworks Solutions Inc.

- R4-1808059	[NR] Band 71 Intra-Band EN-DC PA Back-Off Required for ACLR and SEM	Skyworks Solutions Inc.

Discussion: This agenda item was skipped

## A-MPR for DC_(n)41AA

- R4-1807830	A-MPR Allowance for Band 41/n41 EN-DC R-IMD	Sprint Corporation

- R4-1807831	Effect of Channel Arrangements on RIM3 Products and A-MPR	Sprint Corporation

- R4-1807832	Combined A-MPR for RIM3 and Spectral Regrowth	Sprint Corporation

- R4-1807833	Measurements for Band 41/n41 EN-DC A-MPR	Sprint Corporation

- R4-1807834	Draft CR for 38.101-3 DC_(n)41AA A-MPR	Sprint Corporation

- R4-1808061	[NR] Band 41 contiguous UL Intra-Band EN-DC required backoff	Skyworks Solutions Inc.

Discussion:

Skyworks: measurements were done equal power + equal BO. We have used equal PSD in our measurements.

Qualcomm: did you also looked inband emissions and EVM?

Sprint: Additional Spur, ACLR and SEM were studied.

Next Revision of R4-1807830 was presented and discussed.

Skyworks: Single carrier A-MPR uses scs so should we aling

Sprint: From our perspective harmonization is not necessary

R4-1807831 and R4-1807832 and R4-1807834 was discussed.

Qualcomm: This does not take into account EVM or in band emissions.

Skyworks: There are square brackets or shall we keep those so that we can veryfy, especially the non-contiguous.

Sprint: We are not sure if brackets are allowed or not but if brackets are allowed some of the values could be in brackets.

## A-MPR for DC_41A_n41A

- R4-1807835	Draft CR for 38.101-3 DC_41A_n41A A-MPR	Sprint Corporation

Discussion:

Skyworks: Do we have same understanding for A,B,C and D for all channel arrengenemts.

Sprint: Formulas are general but measurement data is based on 20+60

Qualcomm: Based on experience channel bandwidth can affect. WE are not saying that we need to measure all possible combinations

Sprint is open to other companies how to enhance this proposal

Ericsson: In general the worst case BO as for ET designs. Should not you fallback to ATP in such case?

Skyworks: it is not APT it is fully linear PA

Ericsson: A-MPR is a minimum requirements and UE vendors can do better. This is a complicated issue.

Apple: For the time beeing we do not know what are the most critical cases thus we need to be conservative. WE can use A-MPR versioning in REL-16

Sprint: WE have heard that often UE vendors take the easy route and use all A-MPR. In this case we can have conservative numbers now and have A-MPR versioning later.