3GPP TSG-RAN WG4 Meeting #87	R4-1808306

## Busan, Korea, 21 - 25 May, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.113 | CR |  | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.113: CISPR32 requirement tables |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-05-08 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During previous RAN4#86bis meeting, correction to the radiated emission limits for the ancillary equipement was agreed, based on the CISPR32 specification alignment in R4-1804602. This has triggered discussion on the copy-pasting the CISPR requirement tables into the RAN4 EMC specifications. In this DraftCR, removal of the CISPR32 requirement tables is proposed, by replacing them by detailed references to CISPR content. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - 8.2.2.3, 8.3.3, 8.4.3, 8.5.3: removal of tables, introduction of CISPR 32 references to the same technical content (as in the deleted tables). - 8.4.2: editorials. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Additional RAN4 maintenance work in case of CISPR32 modification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.3, 8.3.3, 8.4.2, 8.4.3, 8.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

### 8.2.2	Radiated emission, Ancillary equipment

This test is only applicable to ancillary equipment not incorporated in the radio equipment and intended to be measured on a stand-alone basis, as declared by the manufacturer. This test shall be performed on a representative configuration of the ancillary equipment.

This test is not applicable for ancillary equipment incorporated in the radio equipment, or for ancillary equipment intended to be measured in combination with the radio equipment. In these cases the requirements of the relevant product standard for the effective use of the radio spectrum shall apply.

#### 8.2.2.1	Definition

This test assesses the ability of ancillary equipment to limit unwanted emission from the enclosure port.

#### 8.2.2.2	Test method

The test method shall be in accordance with CISPR 32 11.

#### 8.2.2.3	Limits

The ancillary equipment shall meet the limits according to CISPR 32 11 table A.4 and table A.5.

For the referred limit values, the following shall apply:

- Where the limits value varies over a given frequency range, it changes linearly with respect to the logarithm of the frequency.

- Where there is a step in the relevant limit, the lower value shall be applied at the transition frequency.

------------------------------ Next modified section ------------------------------

## 8.3	Conducted emission DC power input/output port

This test is applicable to equipment which may have DC cables longer than 3 m.

If the DC power cable of the radio equipment is intended to be less than 3 m in length, and intended only for direct connection to a dedicated AC to DC power supply, then the measurement shall be performed only on the AC power input of that power supply as specified in subclause 8.4.

This test shall be performed on a representative configuration of the radio equipment, the associated ancillary equipment, or representative configuration of the combination of radio and ancillary equipment.

### 8.3.1	Definition

This test assesses the ability of radio equipment and ancillary equipment to limit internal noise from the DC power input/output ports.

### 8.3.2	Test method

The test method shall be in accordance with CISPR 32 [11] and the Artificial Mains Network (AMN) shall be connected to a DC power source.

In the case of DC output ports, the ports shall be connected via an AMN to a load drawing the rated current of the source.

A measuring receiver shall be connected to each AMN measurement port in turn and the conducted emission recorded.

The equipment shall be installed with a ground plane as defined in CISPR 32 [11]. The reference earth point of the AMNs shall be connected to the reference ground plane with a conductor as short as possible.

The measurement receiver shall be in accordance with the requirements of section one of CISPR 16-1-1 [12].

### 8.3.3	Limits

The equipment shall meet the limits according to CISPR 32 11 table A.9, which are defined for the average detector receiver and for quasi-peak detector receiver--. If the average limit is met when using a quasi-peak detector, the equipment shall be deemed to meet both limits and measurement with the average detector receiver is not necessary.

Where there is a step in the referred limit values, the lower value shall be applied at the transition frequency.

| - |  |  |
| --- | --- | --- |
| - |  |  |

------------------------------ Next modified section ------------------------------

## 8.4	Conducted emissions, AC mains power input/output port

This test is applicable to equipment powered by the AC mains.

This test is not applicable to AC output ports which are connected directly (or via a circuit breaker) to the AC power port of the EUT.

This test shall be performed on a representative configuration of the radio equipment, the associated ancillary equipment, or representative configuration of the combination of radio and ancillary equipment.

### 8.4.1	Definition

This test assesses the ability of radio equipment and ancillary equipment to limit internal noise from the AC mains power input/output ports.

### 8.4.2	Test method

The test method shall be in accordance with CISPR 32 [11].

Mains connected ancillary equipment which is not part of the EUT shall be connected to the mains via a separate AMN. According to CISPR 16-1-1 [12], the Protective Earth (PE) conductor shall also be terminated by a 50 Ω/50 μH common mode RF impedance.

### 8.4.3	Limits

The equipment shall meet the limits according to CISPR 32 11 table A.10, which are defined for the average detector receiver and for quasi-peak detector receiver--. If the average limit is met when using a quasi-peak detector, the equipment shall be deemed to meet both limits and measurement with the average detector receiver is not necessary.

For the referred limit values, following shall apply:

- Where the limits value varies over a given frequency range, it changes linearly with respect to the logarithm of the frequency.

- Where there is a step in the relevant limit, the lower value shall be applied at the transition frequency.

| - |  |  |
| --- | --- | --- |
| - |  |  |

Alternatively, for equipment intended to be used in telecommunication centres the limits given in CISPR 32 11 table A.9 shall be used.

| - |  |  |
| --- | --- | --- |
| - |  |  |

------------------------------ Next modified section ------------------------------

## 8.5	Conducted emissions, telecommunication port

This test is applicable for radio equipment and/or ancillary equipment for fixed use which have telecommunication ports.

This test shall be performed on a representative configuration of radio equipment, the associated ancillary equipment, or a representative configuration of the combination of radio and ancillary equipment.

### 8.5.1	Definition

This test assesses the EUT unwanted emission present at the telecommunication ports.

### 8.5.2	Test method

<texts will be added>

### 8.5.3	Limits

The telecommunication ports shall meet the limits according to CISPR 32 [11] table A.12, clause A12.2.

For the referred limit values, following shall apply:

- Where the limits value varies over a given frequency range, it changes linearly with respect to the logarithm of the frequency.

- Where there is a step in the relevant limit, the lower value shall be applied at the transition frequency.

Alternatively, for equipment intended to be used in telecommunication centres only, the limits given in CISPR 32 11 table A.11 clause A11.2 may be used.

------------------------------ End of modified sections ------------------------------