3GPP TSG RAN WG4 Meeting #88	R4-1809632

Gothenburg, SE, 20th – 24th Aug, 2018

3GPP TSG-RAN WG2 Meeting NR Adhoc 1807		R2-1810963

Montreal, Canada, 2nd - 6th July 2018

Title:	LS on additional TDD configuration periodicities

Response to:	–

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	TSG RAN WG2

To:	TSG RAN WG1

Cc:	TSG RAN WG4

Contact Person:

#### Name:	Tero Henttonen

E-mail Address:	tero.henttonen @nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 has discussed the TDD configuration periodicities and noticed that in order to avoid a time offset between LTE and NR, RAN2 discussed the addition of 3ms and 4ms periodicities to NR and confirmed it is feasible to do that.

From RAN2 viewpoint, aligning the LTE and NR TDD configurations can be done by either using subframe shifting or by adding 4ms and 3ms periodicities to the signalling to match the TD-LTE config 1 and 2 respectively by concatenating two periodicities matching the TD-LTE pattern as shown below:

| TD-LTE config 1: | D | S | U | U | D |
| --- | --- | --- | --- | --- | --- |
| NR 4 ms + 1 ms periodicities concatenated | D | S | U | U | D |
| TD-LTE config 2: | D | S | U | D | D |
| NR 3 ms + 2 ms periodicities concatenated | D | S | U | D | D |

Since using subframe shifting may require additional considerations in the future if frame-alignment is required for some features (like is done e.g. in LTE for some features), RAN2 has considered adding the 3ms and 4ms periodicities as a possible alternative. However, RAN2 also considers that the existing rules in RAN1 about 20ms being divisible with the concatenated TDD configuration periodicity should still be followed (i.e. RAN2 does not intend to change the existing RAN1 specification rules on the concatenated TDD configuration periodicity and/or periodicity combinations).

Since this may affect RAN1 specifications, RAN2 would request RAN1 to indicate whether they have concerns with adding these periodicities for the common TDD configuration.

2. Actions:

To RAN1 group.

ACTION: 	RAN2 respectfully asks RAN1 to indicate whether they have concerns to support the 3ms and 4ms TDD DL-UL pattern periodicities in common TDD configuration.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP TSG RAN WG2 #103	20 – 24 Aug 2018		Gothenburg

3GPP TSG RAN WG2#103bis	8 - 12 Oct 2018   	 	Chengdu

3GPPRAN2#104 		12 - 16 Nov 2018    		Spokane