3GPP TSG- RAN WG4 Meeting #88 	R4-1809710

# Gothenburg, Sweden, 20 – 24 August 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.817-02 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR 37.812-02: Finalizing the measurement step size for BS type 2-O out-of-band blocking conformance testing (10.6.2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2018-06-22 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | While the measurement step size for each BS channel bandwidth is included in the endorsed draft CR (R4-1809502) in RAN4 AH-1807, the measurement step size for interfering signal at frequency above 6 GHz is yet to be finalized. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Finalizing the measurement step size for BS type 2-O out-of-band blocking conformance testing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The measurement step size for interfering signal at frequency above 6 GHz remains to be finalized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

### 10.6.2	FR2 OTA out-of-band blocking

For base station operating at the upper frequency region of FR2, it is relevant to discussion the possibility to limit the upper frequency for conformance testing in a similar manner as for FR2 spurious emission. For OTA spurious emission the upper frequency limit for FR2 is set to 60 GHz. The reason for why limiting the test to 60 GHz, is that essential test equipment such as cables and connectors gets very expensive and unreliable. Hence the upper conformance test frequency for OTA out-of-band blocking should also be set to 60 GHz.

The base station out-of-band blocking requirement is tested with a CW interferer and the wanted uplink signal. A spurious response occurs when the m-th harmonic of the interferer frequency mixes with the n-th harmonic of an internal signal, like a clock or a local oscillator signal, and this mixing product falls within the receive bandwidth of the wanted signal (or near the receive bandwidth, if the receiver selectivity is insufficient to suppress the spurious mixing product). The frequency of the unwanted mixing product equals ±m·fCW±n·finternal, where fCW is the interferer frequency and finternal is the frequency of the internal signal involved in the mixing product. m can assume the values 1, 2, 3, … and n can assume the values 0, 1, 2, …. Spurious responses for which the harmonic number m of the interferer signal equals 4 or more are usually not significant. Hence, the frequency step size should be chosen such that any spurious response due to a maximum third harmonic of the interferer will be captured within the receive bandwidth of the wanted signal.

To reduce the amount of time used for the time-consuming OTA conformance testing, while continue to ensure that any spurious response within the receive bandwidth will still be verified with sufficient granularity, the measurement step size for interfering signal at frequency above 6 GHz can be set according to the minimum supported BS channel bandwidth as shown in Table 10.6.2-1 in the NR BS type 2-O OTA out-of-band blocking conformance testing. For interfering signal at frequency below 6 GHz, the traditionally measurement step size of 1 MHz can be kept.

Table 10.6.2-1: Measurement step size for out-of-band blocking conformance testing

| Minimum supported BS Channel bandwidth of wanted signal [MHz] | 50 | 100 | 200 | 400 |
| --- | --- | --- | --- | --- |
| Measurement step size for interfering signal [MHz] | 15 | 30 | 60 | 120 |

<End of change>