3GPP TSG-RAN WG4#88 Meeting	R4-1809798

Gothenburg, Sweden, 20 – 24 August 2018

Agenda item:	7.13.1.5

Title: 	Initial simulation results and discussion on NR CQI tests

Source: 	Intel Corporation

Document for:	Discussion

# Introduction

In RAN4#86 Meeting (Feb. 2018), we have provided our preliminary view on the scope of the NR UE performance requirements, and raised a number of questions to trigger the discussion on the topic of NR UE CSI feedback performance requirements [1]. In RAN4#87 Meeting (May 2018), the work plan [2] was approved for NR UE demodulation and CSI feedback performance requirements. And, the agreements related to the NR UE CSI performance requirements were reached in the WF [3] as listed below:

- First focused on the mandatory features

- Overall Test Configuration & Scope

  - First priority focused on:

    - NZP CSI-RS for channel measurement
CSI-IM for interference measurement

    - CSI report types: periodic CSI report and aperiodic CSI report

    - CSI-RS resources types: periodic and aperiodic

- PMI reporting

  - In Rel-15 focused on Type-I single panel codebook

In RAN4-AH-1807 Meeting (July 2018), a WF [4] on NR UE CSI requirements was approved regarding the initial simulation assumptions for FR1 and FR2 CQI/PMI/RI tests. Companies were encouraged to provide initial simulation results and views on the assumptions and scenarios for NR UE CSI feedback tests. In this contribution, we first provide our initial results for FR1 and FR2 CQI reporting tests. And then, we provide our views on the NR CQI reporting test for further simulation assumptions alignment.

# Initial CQI Reporting Results and Discussion

Table 1 and 2 below list our initial results of Median CQI reporting vs. SNR for FR1 with 256QAM CQI table and FR2 with 64QAM CQI table. It should be noted that the figures listed in Table 1 and 2 were obtained with rather preliminary simulation assumptions and for information only, and modifications to these tables could be needed based on further discussion on NR UE CSI feedback performance requirements.

Table 1. Median CQI vs. SNR (256QAM CQI table for FR1)

| SNR | Median CQI |
| --- | --- |
| -4 | 1 |
| -2 | 1 |
| 0 | 2 |
| 2 | 3 |
| 4 | 3 |
| 6 | 4 |
| 8 | 5 |
| 10 | 6 |
| 12 | 7 |
| 14 | 8 |
| 16 | 9 |
| 18 | 10 |
| 20 | 11 |
| 22 | 12 |
| 24 | 13 |
| 26 | 14 |
| 28 | 15 |

Table 2. Median CQI vs. SNR (64QAM CQI table for FR2)

| SNR | Median CQI |
| --- | --- |
| -4 | 1 |
| -2 | 2 |
| 0 | 3 |
| 2 | 4 |
| 4 | 5 |
| 6 | 6 |
| 8 | 7 |
| 10 | 8 |
| 12 | 9 |
| 14 | 10 |
| 16 | 11 |
| 18 | 12 |
| 20 | 13 |
| 22 | 14 |
| 25 | 15 |

During the course of conducting the mapping between SNR vs. CQI, it was found that the configuration of DMRS could have impact on the mapping relation. It is because that, for NR, there are two types of DMRS configurations, namely DMRS Type 1 and Type 2. Both types of DMRS can be configured with single or multiple additional DMRS symbols. Obviously, additional DMRS symbols would increase the density of REs carrying DMRS symbols. One benefit of configuring additional DMRS is to facilitate better channel estimation performance and, in turn, improve demodulation performance under the same channel conditions. From this perspective, the reported CQI might be different under the same channel conditions. This additional DMRS issue was also identified in [5]. In order to maintain good channel estimation performance and limit the transmission overhead, we propose that:

Proposal 1: when computing the reporting CQI vs. SNR, configuration of one additional DMRS should be assumed at the UE side.

Moreover, in order to further align the simulation results, we make the following proposals on NR CQI reporting performance tests.

Proposal 2: Simulation assumptions for NR CQI tests in Table 3.

Table 3 Test parameters for static CQI reporting performance requirements

| Parameters | FR1 FDD | FR1 TDD | FR2 TDD |
| --- | --- | --- | --- |
| CBW[MHz]/SCS[KHz] | 10MHz/15KHz | 40MHz/30KHz | 100MHz/120KHz |
| DL UL configuration | N/A | DDDSU “S” slot is not scheduled for PDSCH | DDDSU “S” slot is not scheduled for PDSCH |
| Modulation | 256QAM table | 256QAM table | 64QAM table |
| CSI-RS/CSI-IM periodicity and slot offset | [5/0] | [5/0] | [5/0] |
| Propagation channel | AWGN TS_36.101_Static | AWGN TS_36.101_Static | AWGN TS_36.101_Static |
| Correlation and antenna configuration | 2×2 | 2×2 | 2×2 |
| Number of Tx CSI-RS ports | 2 | 2 | 2 |
| NZP CSI-RS configuration CDM type/CSIRS resource mapping/CSI-RS-Density | [FD-CDM2/density 1] | [FD-CDM2/density 1] | [FD-CDM2/density 1] |
| reportConfigType | periodic | periodic | periodic |
| Reporting periodicity and slot offset | [5/1] | [5/1] | [5/1] |
| Rank | 2 | 2 | 2 |
| Codebook Restriction | 010000 (fixed rank2) | 010000 (fixed rank2) | 010000 (fixed rank2) |
| PTRS presence | OFF | OFF | ON |
| CSI-RS Type | - NZP CSI-RS for channel measurement - CSI-IM for interference measurement | - NZP CSI-RS for channel measurement - CSI-IM for interference measurement | - NZP CSI-RS for channel measurement - CSI-IM for interference measurement |
| CSI type | Type I single-panel | Type I single-panel | Type I single-panel |
| PMI frequency granularity | Wideband | Wideband | Wideband |
| CQI frequency granularity | Wideband | Wideband | Wideband |

# Conclusion

In this contribution, we first provide our initial results for FR1 and FR2 CQI reporting tests. And then, we provide our views on the NR CQI reporting test for further simulation assumptions alignment. We make the following proposals:

Proposal 1: when computing the reporting CQI vs. SNR, configuration of one additional DMRS should be assumed at the UE side.

Proposal 2: Simulation assumptions for NR CQI tests in Table 3.

# Reference

[1] R4-1801758, “Views on NR UE performance requirements”, Intel Corporation, Feb. 2018

[2] R4-1808029, “Work plan for UE performance requirements”, Samsung, May 2018

[3] R4-1808027, “Way forward on NR UE demodulation and CSI requirements”, Ericsson, May 2018

[4] R4-1809358, “Way forward on NR UE CSI requirements”, Samsung, July 2018

[5] R4- 1809251, “DMRS assumptions for CSI reporting”, AT&T, July 2018