3GPP TSG-RAN WG4#88 Meeting	R4-1809799

Gothenburg, Sweden, 20 – 24 August 2018

Agenda item:	7.13.1.5

Title: 	Initial simulation results and discussion on NR PMI tests

Source: 	Intel Corporation

Document for:	Discussion

# Introduction

In RAN4#86 Meeting (Feb. 2018), we have provided our preliminary view on the scope of the NR UE performance requirements, and raised a number of questions to trigger the discussion on the topic of NR UE CSI feedback performance requirements [1]. In RAN4#87 Meeting (May 2018), the work plan [2] was approved for NR UE demodulation and CSI feedback performance requirements. And, the agreements related to the NR UE CSI performance requirements were reached in the WF [3] as listed below:

- First focused on the mandatory features

- Overall Test Configuration & Scope

  - First priority focused on:

    - NZP CSI-RS for channel measurement
CSI-IM for interference measurement

    - CSI report types: periodic CSI report and aperiodic CSI report

    - CSI-RS resources types: periodic and aperiodic

- PMI reporting

  - In Rel-15 focused on Type-I single panel codebook

In RAN4-AH-1807 Meeting (July 2018), a WF [4] on NR UE CSI requirements was approved regarding the initial simulation assumptions for FR1 and FR2 CQI/PMI/RI tests. Companies were encouraged to provide initial simulation results and views on the assumptions and scenarios for NR UE CSI feedback tests. In this contribution, we first provide our initial results for FR1 and FR2 PMI reporting tests. And then, we provide our views on the NR PMI reporting test for further simulation assumptions alignment.

# Initial PMI Reporting Results and Discussion

Figure 1 to 4 below show our initial results of (“follow” and “random”) PMI vs. SNR for FR1 with 256QAM CQI table and FR2 with 64QAM CQI table. It should be noted that Figure 1 and 4 were obtained with rather preliminary simulation assumptions and for information only, and modifications to these figures could be needed based on further discussion on NR UE CSI feedback performance requirements.

Figure 1. Simulation results for throughput vs. SNR

(FR1 “Follow” and “Random” PMI, TDL-A, 8×2 XPL high, Rank=2)

Figure 2. Simulation results for throughput ratio vs. SNR

(FR1 “Follow” and “Random” PMI, TDL-A, 8×2 XPL high, Rank=2)

Figure 3. Simulation results for throughput vs. SNR

(FR2 “Follow” and “Random” PMI, TDL-A, 2×2 ULA medium, Rank=1)

Figure 4. Simulation results for throughput ratio vs. SNR

(FR2 “Follow” and “Random” PMI, TDL-A, 2×2 ULA medium, Rank=1)

Moreover, in order to further align the simulation results, we make the following proposals on NR PMI reporting performance tests.

Proposal 1: Simulation assumptions for NR PMI tests in Table 1.

Table 1 Test parameters for PMI reporting performance requirements

| Parameters | FR1 FDD | FR1 TDD | FR2 TDD |
| --- | --- | --- | --- |
| CBW[MHz]/SCS[KHz] | 10MHz/15KHz | 40MHz/30KHz | 100MHz/120KHz |
| DL UL configuration | N/A | DDDSU “S” slot is not scheduled for PDSCH | DDDSU “S” slot is not scheduled for PDSCH |
| Modulation | 256QAM table | 256QAM table | 64QAM table |
| MCS | MCS13 | MCS13 | MCS4 |
| CSI-RS/CSI-IM periodicity and slot offset | [5/0] | [5/0] | [5/0] |
| Propagation channel | TDL 30ns, 5Hz | TDL 30ns, 5Hz | TDL 30ns, 5Hz |
| Correlation and antenna configuration | 8×2 XPL high | 8×2 XPL high | 2×2 FFS |
| Number of Tx CSI-RS ports | 8 | 8 | 2 |
| NZP CSI-RS configuration CDM type/CSIRS resource mapping/CSI-RS-Density | [FD-CDM4/density 1] | [FD-CDM4/density 1] | [FD-CDM2/density 1] |
| reportConfigType | aperiodic | aperiodic | aperiodic |
| Reporting periodicity and slot offset | [5/1] | [5/1] | [5/1] |
| Rank | 2 | 2 | 1 |
| Codebook Mode | 1 | 1 | N/A |
| Codebook Restriction | N/A | N/A | N/A |
| (N1, N2) | (2, 2) | (2, 2) | N/A |
| (O1, O2) | (4, 4) | (4, 4) | N/A |
| Reporting granularity | Wideband | Wideband | Wideband |
| PTRS presence | OFF | OFF | ON |
| CSI-RS Type | - NZP CSI-RS for channel measurement - CSI-IM for interference measurement | - NZP CSI-RS for channel measurement - CSI-IM for interference measurement | - NZP CSI-RS for channel measurement - CSI-IM for interference measurement |
| CSI type | Type I single-panel | Type I single-panel | Type I single-panel |
| PMI frequency granularity | Wideband | Wideband | Wideband |
| CQI frequency granularity | Wideband | Wideband | Wideband |

# Conclusion

In this contribution, we first provide our initial results for FR1 and FR2 PMI reporting tests. And then, we provide our views on the NR PMI reporting test for further simulation assumptions alignment. We make the following proposals:

Proposal 1: Simulation assumptions for NR PMI tests in Table 1.

# Reference

[1] R4-1801758, “Views on NR UE performance requirements”, Intel Corporation, Feb. 2018

[2] R4-1808029, “Work plan for UE performance requirements”, Samsung, May 2018

[3] R4-1808027, “Way forward on NR UE demodulation and CSI requirements”, Ericsson, May 2018

[4] R4-1809358, “Way forward on NR UE CSI requirements”, Samsung, July 2018