3GPP TSG-RAN WG4 Meeting #88	                                         R4-1809807

Gothenburg, SE, 20th – 24th Aug, 2018

Agenda Item:	7.11.8.3

Source:	Intel Corporation

Title:	On remaining issues PSCell addition requirements

Document for:		Discussion

# Introduction

In the latest RAN4 meeting a CR for NR SCell addition requirements [1] has been endorsed. But some of requirements/side conditions were still opened as TBD.

### 7.31.2 NR PSCell Addition Delay Requirement The requirements in this section shall apply for the UE which is configured with PCell, and may also be configured with one or more SCells. Upon receiving NR PSCell addition in subframe n, the UE shall be capable to transmit PRACH preamble towards NR PSCell no later than in subframe n + Tconfig PSCell: Where: Tconfig_PSCell = TRRC_delay + Tprocessing + Tsearch + T∆ + TPSCell_ DU + [2]ms TRRC_delay is the RRC procedure delay as specified in [2]. Tprocessing is the SW processing time needed by UE, including RF warm up period. Tprocessing = 20ms if PSCell is in FR1 or in FR2 and UE provides the measurement report within the last [TBD] ms for the PSCell before the PSCell is added. Otherwise Tprocessing = 40ms. Tsearch is the time for AGC settling and PSS/SSS detection. - For PSCell in FR1: if the target cell is known, then Tsearch = 0 ms. If the target cell is an unknown intra-frequency cell and signal quality is sufficient for successful cell detection on the first attempt, then Tsearch = SMTC periodicity. If the target cell is an unknown inter-frequency cell and signal quality is sufficient for successful cell detection on the first attempt, then Tsearch = [3*SMTC periodicity] ms; - For PSCell in FR2: if the target cell is an intra-frequency cell and signal quality is sufficient for successful cell detection on the first attempt, then Tsearch = [N1*SMTC periodicity] ms. If the target cell is an inter-frequency cell and signal quality is sufficient for successful cell detection on the first attempt, then Tsearch = [N1*3* SMTC periodicity] ms. Editor’s note: N1 is the scaling factor due to Rx beam sweeping, and the value is [TBD]. T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = SMTC periodicity if the PSCell is in FR1. Otherwise T∆ = [TBD]* SMTC periodicity. TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the NR PSCell. TPSCell_ DU is up to x*10 +10 ms. x is defined in the table 6.3.3.2-2 of [6]. In FR1, the NR PSCell is known if it has been meeting the following conditions: During the last 5 seconds before the reception of the NR PSCell configuration command: - the UE has sent a valid measurement report for the NR PSCell being configured and - the NR PSCell being configured remains detectable according to the cell identification conditions specified in section 9.3 of TS 38.133 [50], - NR PSCell being configured also remains detectable during the NR PSCell configuration delay according to the cell identification conditions specified in section 9.3 of TS 38.133 [50]. otherwise it is unknown.

Therefore in this contribution we provided more considerations on these remaining issues (e.g. Tprocessing).

# Discussion

  - Tprocessing

In [1] NR PSCell configuration delay requirements are specified as:

### 7.31.2 NR PSCell Addition Delay Requirement The requirements in this section shall apply for the UE which is configured with PCell, and may also be configured with one or more SCells. Upon receiving NR PSCell addition in subframe n, the UE shall be capable to transmit PRACH preamble towards NR PSCell no later than in subframe n + Tconfig PSCell: Where: Tconfig_PSCell = TRRC_delay + Tprocessing + Tsearch + T∆ + TPSCell_ DU + [2]ms TRRC_delay is the RRC procedure delay as specified in [2]. Tprocessing is the SW processing time needed by UE, including RF warm up period. Tprocessing = 20ms if PSCell is in FR1 or in FR2 and UE provides the measurement report within the last [TBD] ms for the PSCell before the PSCell is added. Otherwise Tprocessing = 40ms.

Thus one of open issue is to define the condition to justify which Tprocessing requirements (20ms or 40ms) shall be applicable.

However, in the previous discussions on Tprocessing, it is assumed that in most scenarios the processing time for PSCell add should be same as the processing time the UE takes during handover. However, in the last RAN4 meeting, it was agreed that it is unnecessary to differentiate Tprocessing for HO [2][3] indeed.

Therefore, the same value can be used for Tprocessing under PScell addition scenarios also.

Proposal 1: Tprocessing  in NR PSCell addition requirements [1]can be 40ms for all cases.

  - Tsearch in FR2

Consistently it is better that align the number of RX beams for NR FR2 HO requirements with other similar requirements for NR FR2(e.g. beam failure detection for FR2).

Thus we can propose that:

Proposal 2: The time for fine time tracking and acquiring full timing information of the target cell in FR2 HO, can be defined as:

T∆ = [8]* SMTC periodicity

# Conclusion

In this contribution, further considerations on the remaining issues of NR PSCell addition requirements. In conclusion, the following observations and proposals can be drawn:

Proposal 1: Tprocessing  in NR PSCell addition requirements [1]can be 40ms for all cases.

Proposal 2: The time for fine time tracking and acquiring full timing information of the target cell in FR2 HO, can be defined as:

T∆ = [8]* SMTC periodicity

# References

- R4-1809379	, “CR to 36.133 on NR PSCell addition and release delay”

- R4-1808749 “On remaining issues Handover requirements for NR”

- R4-1808751, “Clarification on inter-RAT handover requirement for NR for TS38.133”

- 3GPP TS 36.133 v15.2.0