3GPP TSG-RAN WG4 Meeting #88	                                         R4-1809809

Gothenburg, SE, 20th – 24th Aug, 2018

Agenda Item:	7.11.5

Source:	Intel Corporation

Title:	Further discussion on requirements in NR RRC_Idle

Document for:		Discussion

# Introduction

In the latest RAN4 meeting a CR and WF for NR Idle/Inactive states mobility requirements have been approved [1][2]. But some of remaining issues for NR idle/inactive state were still open, e.g. the impacts on measurement requirement relax due to PO collision. Therefore in this contribution we provided more considerations to resolve these remaining issues for NR Idle requirements in [1].

# Impacts of PO collision

In WF[2],the following aspects of PO in NR shall be addressed:

- RAN4 to study the impact of restricting UE to not drop any paging due to SSB-based measurements regardless of whether paging occasion (PO) is TDM or FDM wrt the SSB location. - The RRM measurement requirements in RRC idle/inactive states may be impacted due to paging reception depending on at least the following parameters: - DRX cycle length - SMTC period - The RRM measurement requirements in RRC idle/inactive states may be impacted under the following scenario: - When PO is FDM wrt SSB in FR2 - When PO is TDM wrt SSB in FR1 - When PO is TDM wrt SSB in FR2 - Investigate until the next meeting (RAN4#88) how to address collisions, e.g.: - Maximum DRX cycle length up to which the measurement times in RRC idle/inactive states need to be extended by certain scaling factor (X1, X2, X3) - The value of the scaling factor, X1 for serving cell measurements and Nserv, X2 for intra-frequency measurements, X3 for inter-frequency measurements. - Note: It is FFS whether values of X1, X2 and X3 are different or the same. - SMTC period threshold above which the measurement times in RRC idle/inactive states need to be extended.

Therefore in this section more analysis on these aspects were provided below.

- Impacts on UE power consumption for the paging

Firstly considering the variant PO configurations which were agreed in RAN2 [4](e.g. both TDM and FDM), PO collision impacts on UE power consumption for the paging were given below.

In Figure 1 below, a scenario in which PO is TDMed with SSB is given. Generally gNB cannot always guarantee to schedule a PO immediately after a QCLed SSB to every UE, especially when there are massive number of UEs served by a same gNB and they are receiving different paging messages (PO is UE specific and therefore frequency/time multiplexed for UEs). As a result, the time-gap between a warm-up SSB and a follow-up PO (denoted as “w”) can be long as a SSB burst period as shown in the figure below.

Figure 1. UE power-on duration in case of TDM PO and SSB

Particularly it shall be noted that the AGC and pre-sync before the paging is absolutely necessary in a realistic implementation. Otherwise the paging reception without correct AGC setting is not reliable. For example, in LTE power on time for PO in a paging cycle can be longer than the time of PO subframe(e.g. the extension of PO about 1TTI at least). Same as LTE implementation in NR when paging receiving, UE needs to power on its RF chain earlier than PO for the fine timing adjustment and AGC. This is very critical to guarantee the robust paging receiving.

Observation 1: In order to acquire the fine timing and proper AGC for successful paging receiving, in NR idle state UE needs power on RX in a SMTC window before a PO.

However, as discussed in the previous meeting [5], major concerns on the extended UE power-on duration is the power consumption since UE need more than one wake-up during a paging cycle. And as RAN2 agreed in the last meeting[4],one PO comprises of ‘N’ useful paging PDCCH monitoring occasion where ‘N’ is equal to number of actual transmitted SSBs. Thus UE power-on window for pre-sync and AGC estimation can include “N * 0.5slot” (each of SSB can be less than 0.5slot) and uncertainty. Unlike LTE idle mode, UE activation time for both SSB and PO within a paging cycle can be not contiguous as shown in Figure 1, which can reduce the power consumption greatly especially when the time gap(“w”) is too long . Thus UE power-on duration for the robust paging receiving in idle mode needs about “1*SMTC_period + 4* Ttransition_onAndoff” besides the PO (“N*slot”) itself. Ttransition_onAndoff is the UE transition time between RF chain power on and off. Given these assumptions, as an example, two numeric evaluation results for the power consumption impact due to the extended UE power-on time for both PO and SSB TDMed are given below.

| DRX cycle | 320ms | 1280ms |
| --- | --- | --- |
| Ttransition_onAndoff | 0.2ms | 0.2ms |
| Carrier Frequency | FR2 | FR1 |
| Slot | 0.125ms | 1ms |
| SMTC period | 5ms | 5ms |
| N | 16 | 4 |
| Percentage of total power on time included PO and SSB over a paging cycle (%) | 2.24% | 0.77% |

On the other hand, it shall be noted that if the interval between two consecutive SSB and PO is large enough, the AGC gain from the previous SSB can be expired. Regarding to the similar AGC approach for the inter-frequency measurement in LTE, AGC gain could be expired within a measurement gap (e.g. 160ms). At same time the maximum time gap between PO and the closest SSB ahead is less than a SSB burst period which is configurable as one of {5,10,20,40,80,160}ms. It is clear that maximum possible time gap between TDM PO and SSB will be not larger than a SSB burst period. That is the AGC gain based on the adjacent SSB can be always work well for the PO within a DRX cycle.

Observation 2: The AGC gain for PO based on TDM SSB can be valid in a same paging cycle.

According to RAN2 agreements [4], it is possible that FDM SSB and PO is possible. Similarly as illustrated in Figure 2, if FDMed SSB and PO are aligned with the subframe(slot) boundary, it is not feasible to obtain the AGC and timing from the SSB FDMed in a same SSB burst set. That is regardless its capability of simultaneous SSB and PO reception in case of FDMed SSB and PO, UE needs to wake up earlier than PO for SSB AGC.

Figure 2. UE power-on duration in case of FDM PO and SSB

Observation 3: In case of FDM SSB and PO, within a paging cycle additional UE wakes up beside PO is desired also.

And if there is other UE wake up beside the PO, UE power consumption issue was addressed for TDM case already. Similarly from the realistic implementation, the power consumption increase brought by the more UE wakes up for AGC and measurement during a paging cycle can be acceptable.

At same time the other constriction on the gap between the previous SSB and PO is expected to guarantee the valid AGC. Regarding to this interval in case of FDMed SSB and PO depends on SSB burst period and the maximum SSB burst period is 160ms, we can conclude that:

Observation 4: For FDM PO and SSB, the AGC gain based on the closest SSB before PO can be valid in a paging cycle.

From these evaluation results and observations above, we can conclude

Proposal 1:  It is necessary and feasible to make UE wake up more than one time during a DRX cycle for successful paging reception, which will impact little on the total power consumption.

- Impacts on UE measurement requirements for cell reelection in RRC_Idle

Secondly, the impacts on the measurement due to potential PO collision are addressed below.

For UE measurements on both serving cell and intra/inter-frequency cell in NR idle, since UE needs to wake up in a SMTC window for AGC and correct PO, SSBs in this SMTC window can be available for serving cell and intra-frequency measurement every a DRX cycle without any other extension of UE power-on duration for paging.

Figure 3. Measurements in NR idle

Observation 5: UE can perform measurements on the serving cell and intra-frequency cells every a DRX cycle.

But for the inter-frequency measurement, since UE needs retune to other frequency carrier, it seems that UE need extra power-on duration, in which SSBs of the inter-frequency neighbor cells can be received. For example, in the figure above, UE will perform inter-frequency measurement within the duration marked by green shadow which occurred every two DRX cycles.

In comparison with the intra-frequency measurement, UE needs to wake up one more time for inter-frequency measurements. And the transition time for RF chain power on/off and retuning can be a little bit longer than the that for power on/off only (e.g. 0.5ms vs 0.2ms).

Observation 6:  Inter-frequency measurement will increase UE power consumption further.

Thus in order to better trade-off between the power consumption and mobility performance, it need not to measure the inter frequency cells every a DRX cycle when DRX cycle is less than 320ms. As a result, the inter-frequency measurement requirements in NR idle cell reselection which specified by DRX cycle shall be relaxed for saving power.

Therefore, we can propose:

Proposal 2: The inter-frequency measurement requirements in NR idle [7] shall be relaxed by the scaling factor of [1.5] when DRX_cycle<=[320]ms.

# Tdect requirements in FR2

Moreover, for FR2 since the RX beam sweeping is applicable, Tdect time shall be scaled. In addition, in order to limited the total delay of cell reselection we would like to use the same number for RX beams sweeping as that of FR2 cell identification requirement [8].

In summary, we can proposed that:

Proposal 3: For FR2 cell reselection requirements, the scaling factor because of RX beams can be up to [8].

# Conclusion

In this contribution, further considerations on the remaining issues in NR Idle mode were provided. In conclusion, the following observations and proposals can be drawn:

Observation 1: In order to acquire the fine timing and proper AGC for successful paging receiving, in NR idle state UE needs power on in a SMTC window before a PO.

Observation 2: The AGC gain for PO based on TDM SSB can be valid in a same paging cycle.

Observation 3: In case of FDM SSB and PO, within a paging cycle additional UE wakes up beside PO is desired also.

Observation 4: For FDM PO and SSB, the AGC gain based on the closest SSB before PO can be valid in a paging cycle.

Proposal 1:  It is necessary and feasible to make UE wake up more than one time during a DRX cycle for successful paging reception, which will impact little on the total power consumption.

Observation 5: UE can perform measurements on the serving cell and intra-frequency cells every a DRX cycle.

Observation 6:  Inter-frequency measurement will increase UE power consumption further.

Proposal 2: The inter-frequency measurement requirements in NR idle [7] shall be relaxed by the scaling factor of [1.5] when DRX_cycle<=[320]ms.

Proposal 3: For FR2 cell reselection requirements, the scaling factor because of RX beams can be up to [8].

# References

- R4-1809329, “CR on cell reselection requirement in idle state”

- R4-1809328, “Way forward on collision between paging occasion and SSB”

- R4-18xxxxx, “RAN4#87 Chairman report”

- R2-1809174, “Reply LS on Agreements on Paging”

- R4-1808830, “Remaining Issue Discussion on Idle State for SA NR”

- 3GPP TS 36.133 v15.2.0

- 3GPP TS 38.133 v15.2.1

- R4-1808755, “On Cell Identification Requirements without Gap”