3GPP TSG-RAN WG4 Meeting #88	R4-1809857

Gothenburg, SE, 20th – 24th Aug, 2018

Agenda Item:	7.13.1.4

Source: 	Intel Corporation

Title: 	NR UE PBCH Demodulation requirements

Document for:	Discussion

# Introduction

In RAN4 AH1807 PBCH demodulation requirements were discussed but no agreement made. In this contribution we present our views on PBCH performance requirements in NR and propose test cases.

# Discussion

The PBCH is part of SS/PBCH Block. It has been agreed to introduce performance requirements for PBCH in Rel-15. No new conformance and special test loop mode shall be introduced for NR PBCH. For the purpose of verifying performance of PBCH, we recommend introducing tests with 2Rx. PBCH is transmitted in along with PSS/SSS. In NR the RRM requirements based on SS/PBCH block are limited to 2Rx. For UE demodulation requirements we can limit the test cases to 2Rx and need not mandate 4Rx to be used for PBCH decoding.

Proposal #1: Introduce PBCH performance requirements for 2Rx only

## Test Metrics

The performance tests for PBCH shall evaluate performance of PBCH in fading conditions. The requirements shall be based on probability of miss-detection of PBCH (Pm-bch). The requirement shall be set as SNR @ 1% Pm-bch.

Proposal #2: Requirements for PBCH shall be based on probability of miss-detection of PBCH (Pm-bch); SNR @ 1% Pm-bch

## Reference UE receiver assumptions

For PBCH performance tests, the baseline receiver shall be LMMSE. The requirements shall be based on soft combining of PBCH symbols of the same SSB-index within the PBCH TTI

Proposal #3: The PBCH performance tests shall be based on LMMSE receiver

Proposal #4: The PBCH requirements shall be based on soft combining of PBCH symbols within the PBCH TTI

## Test cases for PBCH

The following test cases in Table 1 are proposed to test PBCH in different conditions.

Table 1: Parameters for PBCH Tests

| Test# | Test1 | Test2 | Test3 | Test4 |
| --- | --- | --- | --- | --- |
| FR | 1 | 1 | 2 | 2 |
| SCS (KHz) | 15 | 30 | 120 | 240 |
| CBW (MHz) | 10 | 40 | 100 | 100 |
| SS Burst Periodicity (ms) | 20 | 20 | 20 | 20 |
| SS Block Pattern | Case A | Case B/ Case C | Case D | Case E |
| Propagation condition | TDL-A; 30ns; 10Hz | TDL-C; 300ns; 100Hz | TDL-A; 30ns; 10Hz | TDL-A; 30ns; 10Hz |
| Antenna configuration and correlation | 1x2 Low | 1x2 Low | 1x2 Low | 1x2 Low |

Proposal #5: Define PBCH demodulation tests cases as above.

# Conclusion

In this paper we present our views in PBCH performance requirements and have the following proposals:

Proposal #1: Introduce PBCH performance requirements for 2Rx only

Proposal #2: Requirements for PBCH shall be based on probability of miss-detection of PBCH (Pm-bch); SNR @ 1% Pm-bch

Proposal #3: The PBCH performance tests shall be based on LMMSE receiver

Proposal #4: The PBCH requirements shall be based on soft combining of PBCH symbols within the PBCH TTI

Proposal #5: Define PBCH demodulation tests cases as :

| Test# | Test1 | Test2 | Test3 | Test4 |
| --- | --- | --- | --- | --- |
| FR | 1 | 1 | 2 | 2 |
| SCS (KHz) | 15 | 30 | 120 | 240 |
| CBW (MHz) | 10 | 40 | 100 | 100 |
| SS Burst Periodicity (ms) | 20 | 20 | 20 | 20 |
| SS Block Pattern | Case A | Case B/ Case C | Case D | Case E |
| Propagation condition | TDL-A; 30ns; 10Hz | TDL-C; 300ns; 100Hz | TDL-A; 30ns; 10Hz | TDL-A; 30ns; 10Hz |
| Antenna configuration and correlation | 1x2 Low | 1x2 Low | 1x2 Low | 1x2 Low |