3GPP TSG-RAN4 Meeting #88 	R4-1809863

# Gothenburg, Sweden, 20 – 24 Aug, 2018

Agenda item:	7.11.3

Source:	Intel Corporation

Title:	On gap-assisted serving cell measurement

Document for:	Discussion

# Introduction

RAN2 has sent an LS[1] to RAN4 for confirming gap assisted serving cell measurement, which is duplicated as below,

RAN2 would like to inform RAN4 about the following RAN2 agreement made in RAN2#99 (October 2017). Agreements for BWP operation in CONNECTED mode: […] 6 The cell defining SS block is considered as the time reference of the serving cell, and for RRM serving cell measurements based on SSB (irrespective of which BWP is activated).. It is RAN2 understanding that the concerned serving cell measurements can be performed using gaps when UE’s active BWP does not contain the cell defining SSB (e.g. according to what RAN4 define as "intra-frequency measurements with measurement gaps"). RAN2 would like to ask RAN4 to confirm that this is in line with RAN4’s understanding.

In this contribution we discuss the serving cell measurement when cell defining SSB is outside active BWP, and corresponding LS is provided in [2].

# Discussion

In RAN4 we had some related agreements in the previous meetings:

- Agreements on definition of SSB based intra-frequency and inter-frequency measurement (RAN4 #AH NR3 in Nagoya)

- SSB based Intra-frequency Measurement: A measurement is defined as a SSB based intra-frequency measurement provided the center frequency of the SSB of the serving cell indicated for measurement and the center frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs are also the same. - SSB based Inter-frequency Measurement: A measurement is defined as a SSB based inter-frequency measurement provided the center frequency of the SSB of the serving cell indicated for measurement and the center frequency of the SSB of the neighbour cell are different, or the subcarrier spacing of the two SSBs are different.

- Agreement on intra-frequency with gap (RAN4 #84bis in Dubrovnik)

Usage of measurement gaps - RAN4 has discussed the following use cases for measurement gaps - Measurement of interfrequency NR cells, - Measurement of intrafrequency NR cells when the intrafrequency SS burst is not within the UEs active bandwidth part and hence the UE needs to perform RF retuning.

- Measurement definition (RAN4 #86 meeting in Athens)

- Measurement type C: intra-frequency measurement with MG - E.g. Intra-frequency measurement outside active BWP

Based on the above agreement, it’s apparent that, if the cell defining SSB is outside of UE active BWP then MG is used for intra-frequency measurement on that cell; and in SSB based intra-frequency measurement case, serving cell SSB center frequency is same as neighbor cell SSB center frequency. In addition, UE is not allowed to tune the RF outside the MG in order to keep data reception/transmission in the current active BWP. Thus, if cell defining SSB of serving cell is outside current UE active BWP, UE need to use the same methodology to conduct the measurement as intra-frequency measurement with MG, that is, in this case, serving cell measurement is conducted with MG.

Proposal: Reply the LS(R2-1810932) to indicate that RAN2’s understanding is in line with RAN4’s.

# Conclusion

In this contribution we discuss the serving cell measurement when cell defining SSB is outside active BWP, and corresponding LS is provided in [2].

Proposal: Reply the LS(R2-1810932) to indicate that RAN2’s understanding is in line with RAN4’s.

# References

[1] R2-1810932, “LS on gap-assisted serving cell measurement”, Qualcomm, RAN2 #AH1807

[2] R4-1809864, “Reply LS on gap-assisted serving cell measurement”, RAN4 #88