3GPP TSG-RAN WG4 Meeting #88 	R4-1809875

Gothenburg, Sweden, August 20 – 24, 2018

Agenda item:	7.11.8.4

Source:	Intel Corporation

Title:	Further discussion on BWP switching Delay

Document for:	Discussion

# Introduction

In RAN4 #AH1807 meeting one WF was approved by RAN4 [1] for BMP switching. The text in the WF related to the delay duration of BWP switching was duplicated as below,

- Depending on the proposals from interested companies, options to introduce new BWP switching delay are to be further discussed - Option 1: Keep type 1 and type 2 unchanged. - Option 2: Revise Type 1 delay and keep Type 2 unchanged. - Option 3: Keep Type 1 and 2 unchanged and introduce Type 3 delay more than 2ms. - Option 4: Revise type 1 delay less than 2ms, introduce type 3 delay more than 2ms and keep Type 2 unchanged. - If option 3 or 4 is agreed, an LS will be send to RAN2 to introduce Type 3 BWP switching delay. - RAN4 is to define two types of requirements for BWP switching delay involving only Baseband parameter changes, based on what parameters is changing. It is FFS if the delay for BWP switching involving only baseband parameter changes is the same as scenario 1/2/3 or scenario 4.

There are some agreements on BWP switching delay from RAN4 #87 and RAN4 #AH1807 meetings, which is stated as below

Agreements: - BWP switch delay requirement is specified in the unit of slot of the serving cell, if the requirement is defined. - The reference SCS when SCS changes from SCS1 to SCS2 is the lager SCS. - There is no extra delay for channel estimation after BWP switching. - Same delay requirements apply for DCI-based and timer-based switching.

The exact BWP switching requirements are still TBD. In this contribution we provide further discussion on the requirements for BWP switching delay.

# BWP switching delay requirement

BWP switch is triggered by two mechanisms: DCI based command or timer based reversion to default BWP. Since it has been agreed that DCI-based and timer-based switching share the same delay requirement, this contribution focuses on the DCI-based BWP switching. For DCI-based active BWP switch, the transition time of active DL or UL BWP switch is the time duration from the end of last OFDM symbol of the PDCCH carrying the active BWP switch DCI till the beginning of a slot indicated by K0 in the active DL BWP switch DCI or K2 in the active UL BWP switch DCI. BWP switching delay, which is same as the transition time for BWP switching, is the duration of BWP switching delay is the maximum time it takes UE to physically reconfigure RF and baseband from existing BWP to new BWP. During BWP switching delay, UE is not expected to transmit and receive data. The BWP switching delay includes the following three components:

- Time for DCI decoding

- Time for preparing for RF/baseband reconfiguration (software overhead)

- Time for RF tuning (applying new RF/baseband parameters)

Furthermore, it has been approved that the BWP switching delay requirement is in the unit of slot of the serving cell. Therefore, the total delay time in terms of µs will be rounded to the next slot boundary for calculating the BWP switching delay. In Figure 1, the timeline for BWP switching delay (in the unit of slot) is depicted

Figure 1: Illustration of the definitions of BWP switching delay in the unit of slot

The BWP switch discussed in RAN4 group focuses on the case of DCI based BWP switching. The delay time includes the time used to decode DCI (in case of DCI based BWP switch), recalculate RF/baseband configuration (e.g. spur estimation, synthesizers reprogramming estimation and LOWIF calculation etc.) and the time UE physically reconfigure RF and baseband. No UE DL and UL data transmission/reception is expected during the BWP switching delay duration.

For DCI decoding, it takes about 350µs, which includes PDCCH decoding and AGC adjustment. The time for RF tuning is 500µs for FR1 (or 250µs for FR2). The main delay duration for BWP switching comes from the procedure of preparing for the RF/baseband reconfiguration, which is the software overhead to program baseband/RF [3].

The software overhead delay does not only depend on what aspect of BWP are changing, e.g., SCS, bandwidth, LO position etc., but also depends on what software architecture is adopted in UE implementation. In [2], the BWP switching delay parameters are specified, i.e., Type 1 delay is 600µs for scenario 1/2/3 and 400µs for scenario 4; Type 2 delay is 2000µs for scenario 1/2/3 and 950µs for scenario 4. As it is explained in [3], the existing delay requirement is based on the assumption that software overheads are set to be 650µs for scenario 4 and 1.3ms for scenario 1/2/3. Such the delay requirements are aggressive for UE implementation. Even when BWP switching with SCS change only (in scenario 4 occurs, all related estimators and adaptive filters need to be reconfigured for the new BWP and the measurement resources regarding on the old BWP will be aborted. Such internal resource mapping and reconfiguration may be run in semi-dynamic way depending on the software architecture in use. Thus, we propose that the software overhead delay is 1.5ms for BWP switching in scenario 4 and 2ms for BWP switching in scenario 1/2/3. The resulting BWP switching delay is

TBWP Switch Delay=TDCI decoding+Tprepare RF/Baseband parameter+TRF tuning+margin

=350us+1500usor 2000us+500us+margin≈ 2 ms,  & for scenario 4,3 ms,  & for scenario 1/2/3

According to RAN1 conclusion, the delay duration starts from the last OFDM symbol of PDCCH carrying the active BWP switch DCI until the beginning of a slot indicated by K0 in the active DL BWP switch DCI or K2 in the active UL BWP switch DCI. The timeline for BWP switching with 3ms/2ms delay is depicted in Figure 2.

Another issue is on BWP switching delay requirement is the AGC adjustment time after the BWP switching. Before starting the operation at the new BWP, UE may need to adjust the AGC accordingly for data communication. If network configures SSB in the slot immediately after BWP switching delay duration, UE can adjust the AGC within one SMTC duration; otherwise, UE can only use the AGC value associated with the old BWP for the slot with the new BWP. The BWP switching delay may be further extended if it is required that AGC settlement is completed within the delay duration. It is FFS for RAN4 if AGC adjustment time is included in the BWP switching delay.

Figure 2: Timeline for BWP switching with 2ms/3ms delay

Proposal 1: Depending on UE capability, UE shall finish BWP switch within the time duration defined in Table 1 below,

Table 1: BWP switching delay

| SCS | Type 1 delay in µs | Type 1 delay in Slot | Type 2 delay in µs | Type 2 delay in Slot | Type 3 delay in µs | Type 3 delay in Slot | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15kHz | 600 | 1 | 2000 | 2 | 3000 | 3 | Scenario 1/2/3 |
|  | 400 | 1 | 950 | 1 | 2000 | 2 | Scenario 4 |
| 30kHz | 600 | 2 | 2000 | 4 | 3000 | 6 | Scenario 1/2/3 |
|  | 400 | 1 | 950 | 2 | 2000 | 4 | Scenario 4 |
| 60kHz | 600 | 3 | 2000 | 8 | 3000 | 12 | Scenario 1/2/3 |
|  | 400 | 2 | 950 | 4 | 1500 | 8 | Scenario 4 |
| 120kHz | 600 | 6 | 2000 | 16 | 3000 | 24 | Scenario 1/2/3 |
|  | 400 | 4 | 950 | 8 | 1500 | 16 | Scenario 4 |

# Conclusion

In this contribution, BWP switching delay is further discussed and Type 3 delay requirement is introduced. The proposals are listed as follows,

Proposal 1: Depending on UE capability, UE shall finish BWP switch within the time duration defined in Table 1 below,

Table 1: BWP switching delay

| SCS | Type 1 delay in µs | Type 1 delay in Slot | Type 2 delay in µs | Type 2 delay in Slot | Type 3 delay in µs | Type 3 delay in Slot | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15kHz | 600 | 1 | 2000 | 2 | 3000 | 3 | Scenario 1/2/3 |
|  | 400 | 1 | 950 | 1 | 2000 | 2 | Scenario 4 |
| 30kHz | 600 | 2 | 2000 | 4 | 3000 | 6 | Scenario 1/2/3 |
|  | 400 | 1 | 950 | 2 | 2000 | 4 | Scenario 4 |
| 60kHz | 600 | 3 | 2000 | 8 | 3000 | 12 | Scenario 1/2/3 |
|  | 400 | 2 | 950 | 4 | 1500 | 8 | Scenario 4 |
| 120kHz | 600 | 6 | 2000 | 16 | 3000 | 24 | Scenario 1/2/3 |
|  | 400 | 4 | 950 | 8 | 1500 | 16 | Scenario 4 |

# References

[1] R4-1808001, “Way forward on BWP Switching”, Mediatek, RAN4#87

[2] R4-1803283, “LS on BWP switching delay”, Intel, RAN4#86

[3] R4-1802270, “BWP Switch Timeline”, Qualcomm, RAN4#86