3GPP TSG-RAN4 Meeting AH#88 					R4-1809898

Gothenburg, Sweden, 20nd July – 6th July, 2018

Title:	Discussion on interruptions test cases for EN-DC

Source:	CATT

Agenda Item:	7.12.3.7

Document for:	Discussion

- Introduction

In last RAN4 AH#1807 meeting, RAN4 had some initial discussion on RRM testing methodology and a test case list was approved in [1]. In this contribution, we have some analysis and discussion on interruptions at transitions test cases for EN-DC in phase I.

- Discussion

The core requirements of interruptions at transition for EN-DC in TS38.133 are captured as follows:

Interruptions at transitions between active and non-active during DRX Up to 1% probability of missed ACK/NACK when the configured PSCell DRX cycle is less than 640ms, and 0.625% probability of missed ACK/NACK is allowed when the configured PSCell DRX cycle is 640ms or longer. Each interruption shall not exceed X slot as defined in table 8.2.1.2.1-1. Table 8.2.1.2.1-1 Interruption length X at transition between active and non-active during DRX NR Slot length (ms); Interruption length X. Sync; Async. 0; 1; 1; 2. 1; 0.5; 1; 2. 2; 0.25; 3. 3; 0.125; 5. Interruptions at transitions from non-DRX to DRX Each interruption shall not exceed X slot as defined in table 8.2.1.2.1-1.

For EN-DC, since individual DRX state and cycle are configured on LTE PCell and NR PSCell, and the interruptions occur when DRX transition procedure is performed. And the test cases are designed by considering the following aspects:

- FDD/TDD

- Synchronous/Asynchronous

- FR1/FR2

In order to reduce the number of test case and save the overall testing time, we should to follow the principle that the tests for coverage for TDD and FDD should be across requirements rather than testing for each requirement for both FDD and TDD. Thus, we propose to define the test cases in table 1 for interruption at transitions between active and non-active during DRX:

Table 1 Interruptions at transition with EN-DC

| Interruptions at transition with EN-DC |  |
| --- | --- |
| 1 | E-UTRAN FDD – NR FR1 FDD interruptions at transitions between active and non-active during DRX in synchronous EN-DC |
| 2 | E-UTRAN TDD – NR FR1 TDD interruptions at transitions between active and non-active during DRX in asynchronous EN-DC |
| 3 | E-UTRAN TDD – NR FR2 TDD interruptions at transitions between active and non-active during DRX in asynchronous EN-DC |

In TS38.133 section 8. 2.1.2, interruptions at transitions between active and non-active during DRX are specified for both LTE PCell and NR PSCell. As a result, both interruptions to LTE PCell and NR PSCell should be tested. In addition, interruptions requirements for LTE PCell and NR PSCell are specified according to different DRX cycle configuration (1% probability of missed ACK/NACK when DRX cycle < 640ms and 0.625% probability when DRX≥ 640ms). Thus different DRX cycle for both LTE PCell and NR PSCell should be configured during the test.

In the rest part of this section, we would like to discuss the principle when designing test case for interruptions at transitions. The purpose of this test is to verify that when LTE PCell is in DRX and NR PSCell is in non-DRX, NR PSCell interruptions due to transitions from active to non-active and from non-active to active during LTE PCell DRX the UE missed ACK/NACK does not exceed the limits, this test will verify the missed ACK/NACK rate for NR PSCell in EN-DC specified in TS38.133 section 8. 2.1.2. In the test there are two cells: Cell1 and Cell2. Cell1 is LTE PCell and Cell2 is NR PSCell. The test consists of one time period, with duration of T1. During T1, NR PSCell is continuously scheduled in DL while LTE PCell is not scheduled and has DRX configured.

- Conclusion

In this contribution, discussion on test case of interruptions at transitions between active and non-active during DRX for EN-DC is provided. Furthermore, an example of test case is also presented.

- Reference

[1]	R4-1809388,	Test case list for NR,	 Ericsson

[2]	R4-1809537,	AdHoc minutes for NR RRM test cases,	 Huawei

[3]	TS38.133 V15.2.0

[4]	TS36.133 V15.3.0

- Annex

A.8.2.1	E-UTRAN FDD – NR FR1 FDD interruptions at transitions between active and non-active during DRX in synchronous EN-DC

A.8.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that when LTE PCell is in DRX and NR PSCell is in non-DRX, NR PSCell interruptions due to transitions from active to non-active and from non-active to active during LTE PCell DRX the UE missed ACK/NACK does not exceed the limits. This test will verify the missed ACK/NACK rate for NR PSCell in EN-DC specified in TS38.133 section 8. 2.1.2.

The test parameters are given in Table A.8.2.1.1-1, A. 8.2.1.1-2 and A. 8.2.1.1-3 below. In the test there are two cells: Cell1 and Cell2. Cell1 is LTE PCell on the FDD primary component (RF channel 1) and Cell2 is NR PSCell on the FR1 FDD secondary component (RF channel 2).. The test consists of one time period, with duration of T1. During T1, NR PSCell is continuously scheduled in DL while LTE PCell is not scheduled and has DRX configured. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell1 and Cell2. Cell1 shall be configured as LTE PCell and Cell2 shall be configured as NR PSCell. Prior to start of T1 the DRX inactivity timer for the LTE PCell have already expired. During T1 the UE shall be continuously scheduled on NR PSCell while not scheduled on LTE PCell. PDCCH indicating a new transmission on PSCell shall be sent continuously during the whole time duration to ensure UE would not enter DRX state on PSCell.

Table A.8.2.1.1-1: General test parameters for E-UTRAN FDD – NR FR1 FDD interruptions at transitions between active and non-active during DRX in synchronous EN-DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | One is E-UTRAN RF channel and the other is NR RF channel |
| Active PCell |  | Cell1 | PCell on E-UTRAN RF channel number 1. |
| Configured PSCell |  | Cell2 | PSCell on NR RF channel number 2. |
| SCS for NR PSCell | KHz | 15 |  |
| SSB periodicity | ms | 20 |  |
| CP length |  | Normal | Applicable to cell1 and cell 2 |
| DRX |  | ON | DRX related parameters are defined in Table A. 8.2.1.1-3 |
| Measurement gap pattern Id |  | OFF |  |
| T1 | s | 10 |  |
| Note 1: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.x.x.x. Note 2: A UE capable of both synchronous and asynchronous EN-DC operations is only required to pass this test case in accordance with the principle defined in section A.x.x. |  |  |  |

Table A.8.2.1.1-2: Cell specific test parameters for E-UTRAN FDD – NR FR1 FDD interruptions at transitions between active and non-active during DRX in synchronous EN-DC

| Parameter | Unit | Cell1 | Cell2 |
| --- | --- | --- | --- |
|  |  | T1 | T1 |
| RF Channel Number |  | 1 | 2 |
| BWchannel | MHz | 10MHz: NRB,c = 50 | 10MHz: NRB,c = 50 |
| PSCell PDSCH parameters: DL Reference Measurement Channel |  | 10MHz: R.3 FDD | 10MHz: X.X FDD |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 10MHz: R.6 FDD | 10MHz: X.X FDD |
| OCNG Patterns |  | 10MHz: OP.10 FDD | 10MHz: X.X FDD |
| PBCH_RA | dB | 0 | 0 |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_PB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_PB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Note 2 | dBm/15 KHz | -101 | -101 |
|  | dB | 19 | 19 |
|  | dB | 19 | 19 |
| RSRP Note 3 | dBm/15 KHz | -82 | -82 |
| SCH_RP Note 3 | dBm/15 KHz | -82 | -82 |
| Io Note 3 | dBm/Ch BW | -54.16 +10log (NRB,c /50) | -54.16 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN | AWGN |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low | 1x2 Low |
| Time offset to cell1 Note 4 | s | - | 33 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |

Table A.8.2.1.1-3: DRX-Configuration for E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC

| Field | Cell2 | Comment |
| --- | --- | --- |
|  | Value |  |
| onDurationTimer | ms1 | Specified in clause 6.3.2 in TS 38.331 |
| drx-InactivityTimerNote 1 | ms1 |  |
| drx-RetransmissionTimerDL | sl1 |  |
| drx-RetransmissionTimerUL | sl1 |  |
| longDRX-CycleStartOffset | ms160 |  |
| shortDRX | disable |  |

A.8.2.1.2	Test Requirements

The UE shall be continuously scheduled in NR PSCell during the entire length of T1. UE shall not be scheduled in LTE PCell during T1. During the time duration T1 the UE shall transmit at least 99% of ACK/NACK on NR PSCell.

Interruption on NR  PSCell shall not exceed X slot as defined in table 8.2.1.2-1.

Table 8.2.1.2-1 Interruption length X at transition between active and non-active during DRX

|  | NR Slot length (ms) | Interruption length X |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 1 | 2 |
| 2 | 0.25 | 3 |  |
| 3 | 0.125 | 5 |  |

The rate of correct events observed during repeated tests shall be at least 90%.