3GPP TSG-RAN WG4 Meeting #88	R4-1809942

# Gothenburg, Sweden, 20th – 24th Aug 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for Test Case of Non-Contention-based Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-08-09 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Test cases for non-contention-based random access is missing in TS38.133. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add test cases for non-contention-based random access in TS38.133 for FR1 and FR2 respectively. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirement for non-contention-based random access can’t be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### >>>>>Start of Change 1<<<<<

#### A.6.2.2.x	NR FR1 – Non-Contention Based Random Access Test

##### A.6.2.2.x.1	Test Purpose and Environment

The purpose of this test is to verify that the behavior of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits. This test will verify the requirements in Clause 6.2.2.2 and Clause 7.1.2 in an AWGN model.

For this test a single cell is used. The test parameters are given in tables A.6.2.2.x.1-1 and A.6.2.2.x.1-2.

Table A.6.2.2.x.1-1: General test parameters for FR1 non-contention based random access test

| Parameter | Unit | Value | Comments |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| SSB SCS | KHz | 15 |  |
| OCNG Pattern Note 1 |  | TBD | As defined in TBD. |
| PDSCH parameters Note 4 |  | TBD | As defined in TBD. |
| PDCCH parameters |  | TBD | As defined in TBD. |
| EPRE ratio of PSS to SSS | dB | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS | dB |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS | dB |  |  |
| EPRE ratio of PSS to SSS | dB |  |  |
|  | dB | 3 |  |
|  | dBm/15 KHz | -98 |  |
|  | dB | 3 |  |
| Io Note 2 | dBm/9.36 MHz | -65.3 |  |
| SS-RSRP Note 3 | dBm/15 KHz | -95 |  |
| ss-PBCH-BlockPower | dBm/15 KHz | -5 | As defined in clause 6.3.2 in TS 38.331. |
| Configured UE transmitted power () | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1. |
| PRACH Configuration Index | - | 87 | Preamble Format A1, As defined in table 6.3.3.2-2 in TS 38.211. |
| Backoff Parameter Index | - | 2 | As defined in table 7.2-1 in TS 38.321. |
| Propagation Condition | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: Io level has been derived from other parameters for information purpose. It is not a settable parameter. Note 3: RSRP level has been derived from other parameters for information purposes. It is not a settable parameter. Note 4: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. |  |  |  |

Table A.6.2.2.x.1-2: RACH-Configuration parameters for FR1 non-contention based random access test

| Field | Value | Comment |
| --- | --- | --- |
| msg1-FDM | one |  |
| msg1-FrequencyStart | 10 |  |
| zeroCorrelationZoneConfig | 0 |  |
| preambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n6 |  |
| powerRampingStep | dB2 |  |
| ra-ResponseWindow | sl10 | 10 slots |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB | one, n4 |  |
| rsrp-ThresholdSSB | RSRP_0 | -140dBm≤ SS-RSRP<-139dBm |
| rsrp-ThresholdCSI-RS | RSRP_0 | -140dBm≤ SS-RSRP<-139dBm |
| ra-ssb-OccasionMaskIndex | 1 | Allowed PRACH occasion(s) of SSB: PRACH occasion index 1 |
| ra-OccasionList | 0 | RA occasions corresponding to CSI-RS that UE shall use |
| Note: For further information see Clause 6.3.2 in TS 38.331. |  |  |

##### A.6.2.2.x.2	Test Requirements

Non-Contention based random access is triggered by explicitly assigning a random access preamble via dedicated signalling in the downlink. In the test, the non-contention based random access procedure is not initialized for Other SI requested from UE.

A.6.2.2.x.2.1	SSB-based Random Access Preamble Transmission

To test the UE behavior specified in Subclause 6.2.2.2.2.1 for SSB-based Random Access Preamble tranmsision, with the contention-free Random Access Resources and the contention-free PRACH occasions associated with SSBs configured, the System Simulator shall receive the Random Access Preamble which belongs to one of the Random Access Preambles associated with the SSB with SS-RSRP above the configured rsrp-ThresholdSSB.

In addition, the System Simulator shall receive the Random Access Preamble on the PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with SS-RSRP above the configured rsrp-ThresholdSSB, and the selected PRACH occasion shall belongs to the PRACH occassions permitted by the restrictions given by the ra-ssb-OccasionMaskIndex.

In addition, the power applied to all preambles shall be in accordance with what is specified in Subclause 6.2.2.2. The power of the first preamble shall be -30 dBm. The power of the first preamble shall be -30 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Subclause 7.1.2.

A.6.2.2.x.2.2	CSI-RS-based Random Access Preamble Transmission

To test the UE behavior specified in Subclause 6.2.2.2.2.1 for CSI-RS-based Random Access Preamble tranmsision, with the contention-free Random Access Resources and the contention-free PRACH occasions associated with CSI-RSs configured, the System Simulator shall receive the Random Access Preamble which belongs to one of the Random Access Preambles associated with the CSI-RS with CSI-RSRP above the configured cfra-csirs-DedicatedRACH-Threshold.

In addition, the System Simulator shall receive the Random Access Preamble on the PRACH occasion which belongs to the PRACH occasions corresponding to the CSI-RS with CSI-RSRP above the configured cfra-csirs-DedicatedRACH-Threshold, and the selected PRACH occasion shall belongs to the PRACH occassions permitted by the restrictions given by the ra-OccasionList corresponding to the selected CSI-RS.

In addition, the power applied to all preambles shall be in accordance with what is specified in Subclause 6.2.2.2. The power of the first preamble shall be -30 dBm. The power of the first preamble shall be -30 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Subclause 7.1.2.

A.6.2.2.x.2.3	Random Access Preamble Transmission for Beam Failure Recovery

To test the UE behavior specified in Subclause 6.2.2.2.2.1 for beam failure recovery triggerred Random Access Preamble tranmsision, with the contention-free Random Access Resources and the contention-free PRACH occasions for beam failure recovery request associated with any of the SSBs and/or CSI-RSs is configured, the System Simulator shall receive the Random Access Preamble which belongs to one of the Random Access Preambles associated with SSB with SS-RSRP above the configured rsrp-ThresholdSSB, or one of the Random Access Preambles associated with CSI-RS with CSI-RSRP above the configured cfra-csirs-DedicatedRACH-Threshold.

In addition, the System Simulator shall receive the Random Access Preamble on the PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with SS-RSRP above the configured rsrp-ThresholdSSB or belongs to the PRACH occasions corresponding to the CSI-RS with CSI-RSRP above the configured cfra-csirs-DedicatedRACH-Threshold, and the selected PRACH occasion shall belongs to the PRACH occassions permitted by the restrictions given by the ra-ssb-OccasionMaskIndex or by the restrictions given by the ra-OccasionList corresponding to the selected CSI-RS.

In addition, the power applied to all preambles shall be in accordance with what is specified in Subclause 6.2.2.2. The power of the first preamble shall be -30 dBm. The power of the first preamble shall be -30 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Subclause 7.1.2.

A.6.2.2.x.2.4	Random Access Response Reception

To test the UE behavior specified in Subclause 6.2.2.2.2.2 the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. In response to the first 4 preambles, the System Simulator shall transmit a Random Access Response not corresponding to the transmitted Random Access Preamble.

The UE may stop monitoring for Random Access Response(s) if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS38.321 [7], and transmit with the calculated PRACH transmission power if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

In addition, the power applied to all preambles shall be in accordance with what is specified in Subclause 6.2.2.2. The power of the first preamble shall be -30 dBm. The power of the first preamble shall be -30 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Subclause 7.1.2.

A.6.2.2.x.2.4	Random Access Response Reception for Beam Failure Recovery

To test the UE behavior after beam failure recovery triggered random access specified in Subclause 6.2.2.2.2.2, the System Simulator shall transmit PDCCH addressed to UE’s C-RNTI is received after 5 preambles have been received by the System Simulator. In response to the first 4 preambles, the System Simulator shall transmit PDCCH not addressed to UE’s C-RNTI.

The UE may stop monitoring for Random Access Response(s) if the UE received PDCCH addressed to UE’s C-RNTI.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS38.321 [7], and transmit with the calculated PRACH transmission power if all UE received PDCCH not addressed to UE’s C-RNTI.

In addition, the power applied to all preambles shall be in accordance with what is specified in Subclause 6.2.2.2. The power of the first preamble shall be -30 dBm. The power of the first preamble shall be -30 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Subclause 7.1.2.

A.6.2.2.x.2.5	No Random Access Response Reception

To test the UE behavior specified in subclause 6.2.2.2.2.3 the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. The System Simulator shall not respond to the first 4 preambles.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window configured in RACH-ConfigCommon.

In addition, the power applied to all preambles shall be in accordance with what is specified in Subclause 6.2.2.2. The power of the first preamble shall be -30 dBm. The power of the first preamble shall be -30 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Subclause 7.1.2.

A.6.2.2.x.2.6	No Random Access Response Reception for Beam Failure Recovery

To test the UE behavior specified in subclause 6.2.2.2.2.3 the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. The System Simulator shall not respond to the first 4 preambles.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if no PDCCH addressed to UE’s C-RNTI is received within the RA Response window configured in BeamFailureRecoveryConfig, as defined in clause 5.1.4 in TS 38.321 [7].

In addition, the power applied to all preambles shall be in accordance with what is specified in Subclause 6.2.2.2. The power of the first preamble shall be -30 dBm. The power of the first preamble shall be -30 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Subclause 7.1.2.

### >>>>>End of Change 1<<<<<

### >>>>>Start of Change 2<<<<<

#### A.6.2.2.x	NR FR2 – Non-Contention Based Random Access Test

##### A.6.2.2.x.1	Test Purpose and Environment

The purpose of this test is to verify that the behavior of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits. This test will verify the requirements in Clause 6.2.2.2 and Clause 7.1.2 in an AWGN model.

For this test a single cell is used. The test parameters are given in tables A.6.2.2.x.1-1 and A.6.2.2.x.1-2.

Table A.6.2.2.x.1-1: General test parameters for FR2 contention based random access test

| Parameter | Unit | Value | Comments |
| --- | --- | --- | --- |
| NR RF Channel Number |  | TBD |  |
| BWchannel | MHz | 100 |  |
| SSB SCS | KHz | 120 |  |
| OCNG Pattern Note 1 |  | TBD | As defined in TBD. |
| PDSCH parameters Note 4 |  | TBD | As defined in TBD. |
| PDCCH parameters |  | TBD | As defined in TBD. |
| EPRE ratio of PSS to SSS | dB | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS | dB |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS | dB |  |  |
| EPRE ratio of PT-RS to PDSCH | dB |  |  |
| EPRE ratio of PSS to SSS | dB |  |  |
|  | dB | 3 |  |
|  | dBm/120 KHz | -95 |  |
|  | dB | 3 |  |
| Io Note 2 | dBm/95.04 MHz | -61.2 |  |
| SS-RSRP Note 3 | dBm/120 KHz | TBD |  |
| ss-PBCH-BlockPower | dBm/120 KHz | -2 | As defined in clause 6.3.2 in TS 38.331. |
| Configured UE transmitted power () | dBm | TBD | As defined in clause 6.2.4 in TS 38.101-2. |
| PRACH Configuration Index | - | 236 | Preamble Format C2, As defined in table 6.3.3.2-2 in TS 38.211. |
| Backoff Parameter Index | - | 0 | As defined in table 7.2-1 in TS 38.321. |
| Propagation Condition | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: Io level has been derived from other parameters for information purpose. It is not a settable parameter. Note 3: RSRP level has been derived from other parameters for information purposes. It is not a settable parameter. Note 4: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. |  |  |  |

Table A.6.2.2.x.1-2: RACH-Configuration parameters for FR2 contention based random access test

| Field | Value | Comment |
| --- | --- | --- |
| msg1-FDM | one |  |
| msg1-FrequencyStart | 10 |  |
| zeroCorrelationZoneConfig | 0 |  |
| preambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n6 |  |
| powerRampingStep | dB2 |  |
| ra-ResponseWindow | sl10 | 10 slots |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB | one, n4 |  |
| rsrp-ThresholdSSB | RSRP_0 | -140dBm≤ SS-RSRP<-139dBm |
| rsrp-ThresholdCSI-RS | RSRP_0 | -140dBm≤ SS-RSRP<-139dBm |
| ra-ssb-OccasionMaskIndex | 1 | Allowed PRACH occasion(s) of SSB: PRACH occasion index 1 |
| ra-OccasionList | 0 | RA occasions corresponding to CSI-RS that UE shall use |
| Note: For further information see Clause 6.3.2 in TS 38.331. |  |  |

##### A.6.2.2.x.2	Test Requirements

Non-Contention based random access is triggered by explicitly assigning a random access preamble via dedicated signalling in the downlink. In the test, the non-contention based random access procedure is not initialized for Other SI requested from UE.

A.6.2.2.x.2.1	SSB-based Random Access Preamble Transmission

To test the UE behavior specified in Subclause 6.2.2.2.2.1 for SSB-based Random Access Preamble tranmsision, with the contention-free Random Access Resources and the contention-free PRACH occasions associated with SSBs configured, the System Simulator shall receive the Random Access Preamble which belongs to one of the Random Access Preambles associated with the SSB with SS-RSRP above the configured rsrp-ThresholdSSB.

In addition, the System Simulator shall receive the Random Access Preamble on the PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with SS-RSRP above the configured rsrp-ThresholdSSB, and the selected PRACH occasion shall belongs to the PRACH occassions permitted by the restrictions given by the ra-ssb-OccasionMaskIndex.

In addition, the power applied to all preambles shall be in accordance with what is specified in Subclause 6.2.2.2. The power of the first preamble shall be -30 dBm. The power of the first preamble shall be -30 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-2 [19]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-2 [19].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Subclause 7.1.2.

A.6.2.2.x.2.2	CSI-RS-based Random Access Preamble Transmission

To test the UE behavior specified in Subclause 6.2.2.2.2.1 for CSI-RS-based Random Access Preamble tranmsision, with the contention-free Random Access Resources and the contention-free PRACH occasions associated with CSI-RSs configured, the System Simulator shall receive the Random Access Preamble which belongs to one of the Random Access Preambles associated with the CSI-RS with CSI-RSRP above the configured cfra-csirs-DedicatedRACH-Threshold.

In addition, the System Simulator shall receive the Random Access Preamble on the PRACH occasion which belongs to the PRACH occasions corresponding to the CSI-RS with CSI-RSRP above the configured cfra-csirs-DedicatedRACH-Threshold, and the selected PRACH occasion shall belongs to the PRACH occassions permitted by the restrictions given by the ra-OccasionList corresponding to the selected CSI-RS.

In addition, the power applied to all preambles shall be in accordance with what is specified in Subclause 6.2.2.2. The power of the first preamble shall be -30 dBm. The power of the first preamble shall be -30 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-2 [19]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-2 [19].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Subclause 7.1.2.

A.6.2.2.x.2.3	Random Access Preamble Transmission for Beam Failure Recovery

To test the UE behavior specified in Subclause 6.2.2.2.2.1 for beam failure recovery triggerred Random Access Preamble tranmsision, with the contention-free Random Access Resources and the contention-free PRACH occasions for beam failure recovery request associated with any of the SSBs and/or CSI-RSs is configured, the System Simulator shall receive the Random Access Preamble which belongs to one of the Random Access Preambles associated with SSB with SS-RSRP above the configured rsrp-ThresholdSSB, or one of the Random Access Preambles associated with CSI-RS with CSI-RSRP above the configured cfra-csirs-DedicatedRACH-Threshold.

In addition, the System Simulator shall receive the Random Access Preamble on the PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with SS-RSRP above the configured rsrp-ThresholdSSB or belongs to the PRACH occasions corresponding to the CSI-RS with CSI-RSRP above the configured cfra-csirs-DedicatedRACH-Threshold, and the selected PRACH occasion shall belongs to the PRACH occassions permitted by the restrictions given by the ra-ssb-OccasionMaskIndex or by the restrictions given by the ra-OccasionList corresponding to the selected CSI-RS.

In addition, the power applied to all preambles shall be in accordance with what is specified in Subclause 6.2.2.2. The power of the first preamble shall be -30 dBm. The power of the first preamble shall be -30 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-2 [19]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-2 [19].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Subclause 7.1.2.

A.6.2.2.x.2.4	Random Access Response Reception

To test the UE behavior specified in Subclause 6.2.2.2.2.2 the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. In response to the first 4 preambles, the System Simulator shall transmit a Random Access Response not corresponding to the transmitted Random Access Preamble.

The UE may stop monitoring for Random Access Response(s) if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS38.321 [7], and transmit with the calculated PRACH transmission power if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

In addition, the power applied to all preambles shall be in accordance with what is specified in Subclause 6.2.2.2. The power of the first preamble shall be -30 dBm. The power of the first preamble shall be -30 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-2 [19]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-2 [19].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Subclause 7.1.2.

A.6.2.2.x.2.4	Random Access Response Reception for Beam Failure Recovery

To test the UE behavior after beam failure recovery triggered random access specified in Subclause 6.2.2.2.2.2, the System Simulator shall transmit PDCCH addressed to UE’s C-RNTI is received after 5 preambles have been received by the System Simulator. In response to the first 4 preambles, the System Simulator shall transmit PDCCH not addressed to UE’s C-RNTI.

The UE may stop monitoring for Random Access Response(s) if the UE received PDCCH addressed to UE’s C-RNTI.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS38.321 [7], and transmit with the calculated PRACH transmission power if all UE received PDCCH not addressed to UE’s C-RNTI.

In addition, the power applied to all preambles shall be in accordance with what is specified in Subclause 6.2.2.2. The power of the first preamble shall be -30 dBm. The power of the first preamble shall be -30 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-2 [19]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-2 [19].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Subclause 7.1.2.

A.6.2.2.x.2.5	No Random Access Response Reception

To test the UE behavior specified in subclause 6.2.2.2.2.3 the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. The System Simulator shall not respond to the first 4 preambles.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window configured in RACH-ConfigCommon.

In addition, the power applied to all preambles shall be in accordance with what is specified in Subclause 6.2.2.2. The power of the first preamble shall be -30 dBm. The power of the first preamble shall be -30 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-2 [19]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-2 [19].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Subclause 7.1.2.

A.6.2.2.x.2.6	No Random Access Response Reception for Beam Failure Recovery

To test the UE behavior specified in subclause 6.2.2.2.2.3 the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. The System Simulator shall not respond to the first 4 preambles.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if no PDCCH addressed to UE’s C-RNTI is received within the RA Response window configured in BeamFailureRecoveryConfig, as defined in clause 5.1.4 in TS 38.321 [7].

In addition, the power applied to all preambles shall be in accordance with what is specified in Subclause 6.2.2.2. The power of the first preamble shall be -30 dBm. The power of the first preamble shall be -30 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-2 [19]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-2 [19].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Subclause 7.1.2.

### >>>>>End of Change 2<<<<<